3GPP TSG-RAN WG2 Meeting #109bis-e	R2-20xxxxx

# Online, April 20 – 30, 2020

Agenda item:	6.8.2.4

Source: 	Ericsson

Title: 	Email discussion report: [AT109bis-e][602][POS] LPP ASN.1 structural issues (Ericsson)

Document for:		Discussion and Decision

# 1. 	Introduction

This document summarizes the following email discussion:

- [[AT109bis-e][602][POS] LPP ASN.1 structural issues (Ericsson)

Status: Started

Scope: Initial discussion on the issues raised in R2-2003144

Intended outcome: Report of potential easy agreements and remaining open issues (R2-2003983) suitable for capture in the LPP ASN.1 issue list

Deadline:  Comments by Tuesday 2020-04-28 1000 UTC; output document Wednesday 2020-04-29 1000 UTC

  - 2	Discussion about LPP ASN.1 structural issues

Different structural issues and aspects are addressed in the following subsections. As dfiscussed in RAN2-109-e, the guiding metric when comparing different representation is ASN.1 PER-encoded examples. Suggested proposals are adopted from [1].

  - 2.1	Association of DL-PRS AD and UE-Based AD

DL-PRS is fundamental is the NR positioning assistance data, and in the case of UE-based positioning, there is a need to provide additional assistance data of TRP location, beam information and relative time differences of DL-PRS transmissions – the UE-based assistance data.

The DL-PRS AD is represented in a hierarchy:

- 1-4 Positioning Frequency Layers (PFL)

  - 1-64 TRPs

    - 1-2 DL-PRS Resource Sets

      - 1-64 DL-PRS Resources

The following figure illustrate the highest two levels of the hierarchy in an example with two PFLs.

In case of UE-based positioning, the UE is also configured with UE-based assistance data, and each TRP of the DL-PRS AD needs to be associated to elements of the UEB AD. Two options are discussed in [1]:

Option 1.1: Matching UEB AD hierarchy (37.355 V16.0.0.)

The UEB AD is represented by an hierarchy of exactly the same size as DL-PRS AD

Option 1.2: Linear UEB AD hierarchy

By listing only the unique elements in TRP lists, and introducing references from elements in the DL-PRS AD hierarchy to the TRP list elements, the representation can be very compact as illustrated by the following example based on an example with three sectors per TRP and two frequency layers. Groups of six TRPs from the DL-PRS AD only need to refer to one TRP location:

The cost is the pointer (list index) from each TRP in the DL-PRS AD hierarchy to the UEB AD TRP list.

In [1] these two representations are compared in terms of PER-encoded bytes in two examples, a minimalistic example which gives the same size, and the 3GPP Indoor Open Office scenario for FR2, which gives Option 1.1: 6.4 kBytes, and Option 1.2: 5.5 kBytes – a reduction in size of 13% or in absolute terms a reduction of more than three SI messages if using Option 1.2 instead of Option 1.1.

Companies are asked to Provide their opinion and preference on the two representations of UE-basedAD

| 1.1 Representation of UE-based assistance data |  |
| --- | --- |
| Company | Comments |
| Ericsson | We are concerned about the combined size of the AD and find it necessary to take any opportunity to ensure that the AD is as compact and efficiently represented as possible. Therefore, we prefer Option 1.2 with linear UE-based assistance data. |

From [1], the following related proposals are adopted:

- Change the UE-based assistance data structure to linear lists and agree to the text proposals in Annex 1.

- There is no need to include the TRP-ID in the UEB AD hierarchy. It is obtained by association via the references to the DL-PRS AD hierarchy.

  - 2.2	Representation of beam directions

UE-based AD may include information about the antenna beam directions in the network. At RAN2-109-e, two different representations of beam directions were discussed:

- Option 2.1: 0.1 degrees resolution

- Option 2.2: 1 degrees resolution with an optional refinement to 0.1 degrees.

This is not a measurement, but a configuration and is related to how accurate an operator can determine the main direction of the installed antenna beams in the network. To determine antenna beam directions as precisely as 0.1 degrees means specific operator efforts, and if antenna beam directions are known at a 1 degree resolution, then Option 2.1 means signing overhead, while Option 2.2 is more appropriate.

The two representations were used in [1] to PER-encode two examples, one where the operator has determined the antenna beam directions with a 0.1 degree resolution, and one with a 1 degree resolution for the 3GPP IOO scenario. In the former case, both Option 2.1 and Option 2.2 gives the same number of PER-encoded bytes, while in the latter case, Option 2.2 gives an 18% reduction in PER-encoded bytes compared to Option 2.1 – or a difference in absolute terms of more than one SI message.

In light of the new information about the impact on PER-encoded size reduction from Option 2.2 compared to Option 2.1, companies are asked to comment of the suitable ASN.1 representation for beam directions

| 2.1 Beam direction representation of either 0.1 degrees or 1 degrees with an optional 0.1 degree refinement |  |
| --- | --- |
| Company | Comments |
| Ericsson | Again, we are concerned about the combined size of the AD and our understanding and experience is that in many cases the operator only is able to determine the antenna beam direction configuration of a 1 degree resolution. RAN1 has discussed the angle measurement accuracy, and disclosed that a 0.1 degree resolution is appropriate for angle measurements. However, RAN1 is not discussing the resolution of configuration parameters such as the beam direction. Option 2.2 provides a much more efficient coding in this case which has a significant impact on the size of the AD. We therefore prefer the more efficient coding of Option 2.2. |

- Introduce a 1 degree resolution and an optional 0.1 degree refinement for beam direction representation and agree to the text proposal in Annex 2.

  - 2.3	Cartesian relative coordinates

The baseline representation of TRP coordinates is based on a reference point and relative coordinates in relation to the reference point in delta latitutde-longitude-height. This is most suitable to represent macro and micro deployments, but not very suitable not very efficient to represent indoor deployments such as the indoor open office. In such an environment, it is more appropriate to define a reference point such as a lower left coordinate, and then define relative XYZ coordinates in relation to the reference point.

The relative location of a TRP etc can then instead be a choice between the IE in baseline and a new IE with a cartesian XYZ coordinate as in the following:

-- ASN1START

RelativeLocationLoLaAlt-r16 ::= SEQUENCE {

milli-arc-second-units-r16 	ENUMERATED { mas0-03, mas0-3, mas3, mas30, ...},

height-units-r16			ENUMERATED {mm, cm, m, ...},

delta-latitude-r16			Delta-Latitude-r16,

delta-longitude-r16			Delta-Longitude-r16,

delta-height-r16			Delta-Height-r16,

locationUNC-r16				LocationUncertainty-r16				OPTIONAL,		-- Need OP

...

}

RelativeLocation-r16 ::= CHOICE {

relativeLocationLoLaAlt-r16		RelativeLocationLoLaAlt-r16,

relativeLocationxyz-r16			RelativeLocationXYZ-r16

}

RelativeLocationXYZ-r16 ::= SEQUENCE {

xyz-units-r16				ENUMERATED {cm, dm, ...},

delta-x-r16					INTEGER (0..4095),

delta-y-r16					INTEGER (0..4095),

delta-z-r16					INTEGER (0..4095),

locationUNC-r16				LocationUncertainty-r16				OPTIONAL,		-- Need OP

...

}

-- ASN1STOP

In 3GPP IOO example in [1], PER-encoded 187 bytes is needed for the current baseline representation of relative coordinates, and 152 bytes for the proposed alternative cartesian representation of relative coordinates. The main driver for an alternative representation of relative locations is convenience for the operator.

Companies are asked to provide comments about possible and relevant representations of relative locations for DL-PRS transmission entities. Basically, which of the below options company prefer.

- Option 3.1. Current structure with a relative location only in delta latitutde-longitude-height

- Option 3.2. A choice of two relative location alternatives, either delta latitutde-longitude-height or delta cartesian XYZ.

| 3.1 Relative location representation also in local Cartesian coordinates |  |
| --- | --- |
| Company | Comments |
| Ericsson | NR positioning is expected to provide benefits in a wide range of scenarios and use cases. Some of these are supported by outdoor deployment where relative TRP locations naturally are represented by delta latitude-longitude-height. Other situations such as indoor deployments are much more conveniently represented by relative cartesian coordinates. Therefore, we find it natural to ensure that both representations are possible so we support Option 3.2 |

- RAN2 to discuss and agree to an alternative cartesian relative position representation.

- RAN2 to agree to the text proposal in Annex 3.

  - 2.4 UEB high level AD structuring

As discussed in the email discussion, there is now one grouping of UE-based assistance data information in the IE NR-PositionCalculationAssistanceData-r16  as part of the AD provided to the target device point to point and a different grouping of the UE-based assistance data information in the IEs NR-UEB-TRP-LocationData-r16 and NR-UEB-TRP-RTD-Info-r16  provided via broadcast.

The coding standard in LPP to date is to compile the pos SIBs based on existing IEs. Therefore it seems more natural and clear to group the UE-based assistance data in the IEs NR-UEB-TRP-LocationData-r16 and NR-UEB-TRP-RTD-Info-r16  for both peer to peer and broadcast.

Companies are asked to provide comments about the UE-based assistance data grouping and if it is relevant to have one and the same grouping of the information no matter how the assistance data is distributed. In particular, consider the following two options:

- Option 4.1. One grouping based on IE NR-PositionCalculationAssistanceData-r16  for peer to peer unicast AD distribution and a different grouping based on the IEs NR-UEB-TRP-LocationData-r16 and NR-UEB-TRP-RTD-Info-r16  broadcast distribution

- Option 4.2. The same grouping based on the IEs NR-UEB-TRP-LocationData-r16 and NR-UEB-TRP-RTD-Info-r16  for both peer to peer unicast and broadcast distribution

| 4.1 UE-based assistance data grouping |  |
| --- | --- |
| Company | Comments |
| Ericsson | We prefer to follow the practice of one representation and grouping of information that is used both in per to peer unicast and broadcast – the practice we have followed this far In LPP. Already when the UE-based positioning was discussed via email, different groupings were discussed, and QC stressed that “The field/IE names are typically selected to describe its content”. Clearly, the IE names NR-UEB-TRP-LocationData-r16 and NR-UEB-TRP-RTD-Info-r16 are examples of that, while the IE name NR-PositionCalculationAssistanceData-r16 is not, instead something that the receiver is supposed to use the data for. We are in favor of one and the same grouping of information and support Option 4.2. The combined text proposal for Option 4.2 and 5.1 can be as follows, with a new IE introduced in 6.4.3: |
| #### – NR-Pos-ProvideAssistanceData The IE NR-Pos-ProvideAssistanceData is used by the location server to provide assistance data to enable UE-assisted and UE-based NR downlink positioning. It may also be used to provide NR positioning specific error reason. -- ASN1START NR-Pos-ProvideAssistanceData-r16 ::= SEQUENCE { nr-DL-PRS-AssistanceData-r16 NR-DL-PRS-AssistanceData-r16 OPTIONAL, -- Need ON nr-UEB-TRP-LocationData-r16 NR-UEB-TRP-LocationData-r16 OPTIONAL, -- Cond UEB nr- UEB-TRP-RTD-Info-r16 NR- UEB-TRP-RTD-Info-r16 OPTIONAL, -- Cond UEB nr-Pos-Error-r16 NR-Pos-Error-r16 OPTIONAL, -- Need ON ... } -- ASN1STOP Conditional presence; Explanation. UEB; The field is mandatory present for UE based NR positioning; otherwise it is not present.. |
| Ericsson | In this way we can maintain a clear specification that follows the coding standards that we have adopted to date |

- Represent UEB AD grouped into NR-UEB-TRP-LocationData and NR-UEB-TRP-RTD-Info for both unicast and broadcast of assistance data

  - 2.5	NR DL-PRS Assistance Data

With the agreement to separate the NR positioning support in LPP over several positioning methods comes an issue about how to provide the DL-PRS AD. If only one positioning method is configured, then it is not so important, but if there are several methods configured, then it makes sense to place the DL-PRS AD more central. As before, there exists an index IE to point at what parts of the DL-PRS assistance data that is relevant per positioning method if it is relevant.

Three options were discussed during the email discussion concerning the final check of the running CR:

- Option 5.1. Lift up DL-PRS AD one level to ProvideAssistanceData as nr-DL-PRS-ProvideAssistanceData:

[[  nr-DL-PRS-ProvideAssistanceData-r16     NR-DL-PRS-ProvideAssistanceData-r16             OPTIONAL,   -- Need ON

nr-Multi-RTT-ProvideAssistanceData-r16  NR-Multi-RTT-ProvideAssistanceData-r16   OPTIONAL,   -- Need ON

nr-DL-AoD-ProvideAssistanceData-r16     NR-DL-AoD-ProvideAssistanceData-r16       OPTIONAL,   -- Need ON

nr-DL-TDOA-ProvideAssistanceData-r16    NR-DL-TDOA-ProvideAssistanceData-r16   OPTIONAL    -- Need ON

]]

- Option 5.2. Include nr-DL-PRS-AssistanceData in the common provide assistance data

CommonIEsProvideAssistanceData ::= SEQUENCE {

...,

[[

segmentationInfo-r14                        SegmentationInfo-r14                            OPTIONAL        -- Need ON

]],

[[

periodicAssistanceData-r15            PeriodicAssistanceDataControlParameters-r15

OPTIONAL        -- Cond PerAD

]],

[[

nr-DL-PRS-AssistanceData-r16               NR-DL-PRS-AssistanceData-r16             OPTIONAL,   -- Need ON

]]

}

- Option 5.3. Include nr-DL-PRS-AssistanceData per positioning method with conditional presence

NR-DL-TDOA-ProvideAssistanceData-r16 ::= SEQUENCE {

nr-DL-PRS-AssistanceData-r16                    NR-DL-PRS-AssistanceData-r16                          OPTIONAL,       -- Cond NotProvidedInOtherMethod

nr-SelectedDL-PRS-IndexList-r16 SEQUENCE (SIZE (1..nrMaxFreqLayers)) OF NR-SelectedDL-PRS-PerFreq-r16 OPTIONAL,-- Need ON

nr-PositionCalculationAssistanceData-r16

NR-PositionCalculationAssistanceData-r16      OPTIONAL,       -- Cond UEB

nr-DL-TDOA-Error-r16                            NR-DL-TDOA-Error-r16                         OPTIONAL,       -- Need ON

...

}

The nice things with Option 5.1 is that it becomes more clear and readable, that it allows a separate DL PRS error, and that it also will include the UE-based assistance data in a clearly common place. Also, Option 5.2 separates the DL PRS data from the positioning methods, and could have DL-PRS specific errors as part of some common structure, but it makes the common structure less clean. It is also possible as in Option 5.3 to place the DL-PRS in the positioning methods with a conditional presence so that it only needs to be provide once, but then without the possibility of a separate DL PRS error message, and a more messy structure.

Companies are asked to comment on the suitable placmement of the DL-PRS AD in the LPP structure to make it a clean and clear representation, and indicate their preferred option among the three options presented

| 5.1 DL-PRS AD placement in the LPP message structure for provisioning of location information |  |
| --- | --- |
| Company | Comments |
| Ericsson | We consider Option 5.1 as the most clear and readable placement and it also allow a specific error message for NR DL-PRS – this central component in NR positioning. For text proposal, see previous issue, where a new IE generic for NR positioning is added to 6.4.3. |

- Introduce DL-PRS AD as an IE NR-DL-PRS-ProvideAssistanceData as part of the ProvideAssistanceData IE

  - 2.6	Reference TRP indication

The discussion about TRP reference indication in [1] is already part of the email discussion #601 and is therefore omitted here.

# 3. 	Other Issues

Any other LPP ASN.1 structural issues which do not fit into the section 2 above?

| Company | Issue |
| --- | --- |

# 4. 	Summary

# 5. 	References

[1]		 R2-2003144, “Important LPP structural aspects”, Ericsson

# Annex 1: Text Proposal for 37.355 Subsection 6.4.1

### 6.4.1	Common Lower-Level IEs

[…]

#### -	NR-TRP-LocationInfo

The IE NR-TRP-LocationInfo is used by the location server to provide the coordinates of the antenna reference points for a set of TRPs. For each TRP, the ARP location can be provided for each associated PRS Resource ID per PRS Resource Set.

-- ASN1START

NR-TRP-LocationInfo-r16 ::= SEQUENCE {

referencePoint-r16			ReferencePoint-r16				OPTIONAL,	-- Cond NotSameAsPrev

trp-LocationInfoList-r16	SEQUENCE (SIZE (0..255)) OF TRP-LocationInfoElement-r16,

...

}

NR-TRP-LocationInfoPerFreqLayer-r16 ::= SEQUENCE {

referencePoint-r16			ReferencePoint-r16				OPTIONAL,	-- Cond NotSameAsPrev

trp-LocationInfoList-r16	SEQUENCE (SIZE (1..64)) OF TRP-LocationInfoElement-r16,

...

}

TRP-LocationInfoElement-r16 ::= SEQUENCE {

trp-Location-r16				RelativeLocation-r16					OPTIONAL,	-- Need OP

trp-DL-PRS-ResourceSets-r16		SEQUENCE (SIZE(1..2)) OF

DL-PRS-ResourceSets-TRP-Element-r16	OPTIONAL,	-- Need OP

...

}

DL-PRS-ResourceSets-TRP-Element-r16 ::= SEQUENCE {

dl-PRS-ResourceSetARP-r16			RelativeLocation-r16				OPTIONAL,	-- Need OP

dl-PRS-Resource-ARP-List-r16		SEQUENCE (SIZE(1..64)) OF

DL-PRS-Resource-ARP-Element-r16	OPTIONAL,	-- Need OP

...

}

DL-PRS-Resource-ARP-Element-r16 ::= SEQUENCE {

dl-PRS-Resource-ARP-location-r16	RelativeLocation-r16				OPTIONAL,	-- Need OP

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| NotSameAsPrev | The field is mandatory present in the first entry of the NR-TRP-LocationInfoPerFreqLayer list; otherwise it is optionally present, need OP. |

NR-TRP-LocationInfo field descriptions

referencePoint This field specifies the reference point used to define the TRP location in the trp-LocationInfoList. If this field is absent, the reference point is the same as in the previous entry of the NR-TRP-LocationInfoPerFreqLayer list.

trp-LocationInfoList This field provides the antenna reference point locations of the DL-PRS Resources for the TRPs. The list index is the reference used from the DL-PRS assistance data to associate a TRP of the DL-PRS to an element in this list. This field comprises the following sub-fields: - trp-Location: This field provides the location of the TRP relative to the referencePoint location. If this field is absent the TRP location coincides with the referencePoint location. - trp-DL-PRS-ResourceSets: This field provides the antenna reference point location(s) of the DL-PRS Resource Set(s) associated with this TRP. If this field is absent, the antenna reference point location(s) of the DL-PRS Resource Set(s) coincides with the trp-Location location. This field comprises the following sub-fields: - dl-PRS-ResourceSetARP: This field provides the antenna reference point location of the DL-PRS Resource Set relative to the trp-Location location. If this field is absent, the antenna reference point location of this DL-PRS Resource Set coincides with the trp-Location location. - dl-PRS-Resource-ARP-List: This field provides the antenna reference point location(s) of the DL-PRS Resource(s) associated with this resource set of the TRP. If this field is absent, the antenna reference point location(s) of the DL-PRS Resources coincides with the dl-PRS-ResourceSetARP location. This field comprises the following sub-fields: - dl-PRS-Resource-ARP-location: This field provides the antenna reference point location of the DL-PRS Resource associated with the DL-PRS Resource Set of the TRP relative to the dl-PRS-ResourceSetARP location. If this field is absent, the antenna reference point location of this DL-PRS Resource coincides with the dl-PRS-ResourceSetARP location.

[…]

#### –	NR-DL-PRS-BeamInfo

The IE NR-DL-PRS-BeamInfo is used by the location server to provide spatial direction information of the DL-PRS Resources. The list index is the reference used from the DL-PRS assistance data to associate a TRP of the DL-PRS to an element in this list.

-- ASN1START

NR-DL-PRS-BeamInfo-r16 ::= SEQUENCE (SIZE (0..255)) OF NR-DL-PRS-BeamInfo-r16

NR-DL-PRS-BeamInfo-r16 ::= SEQUENCE {

lcs-gcs-translation-parameter-r16	LCS-GCS-Translation-Parameter-r16	OPTIONAL,	-- Need OP

dl-prs-BeamInfoSet-r16				DL-PRS-BeamInfoSet-r16,

...

}

DL-PRS-BeamInfoSet-r16 ::= SEQUENCE (SIZE(1..2)) OF DL-PRS-BeamInfoResourceSet-r16

DL-PRS-BeamInfoResourceSet-r16 ::= SEQUENCE (SIZE(1..64)) OF DL-PRS-BeamInfoElement-r16

DL-PRS-BeamInfoElement-r16 ::= SEQUENCE {

dl-PRS-Azimuth-r16				INTEGER (0..3599),

dl-PRS-Elevation-r16			INTEGER (0..1800)				OPTIONAL,	-- Need ON

...

}

LCS-GCS-Translation-Parameter-r16 ::= SEQUENCE {

alpha-r16						INTEGER (0..3599),

beta-r16						INTEGER (0..3599),

gamma-r16						INTEGER (0..3599),

...

}

-- ASN1STOP

NR-DL-PRS-Beam-Info field descriptions

lcs-gcs-translation-parameter This field provides the angles α (bearing angle), β (downtilt angle) and γ (slant angle) for the translation of a Local Coordinate System (LCS) to a Global Coordinate System (GCS) as defined in TR 38.901 [x]. If this field is absent, the dl-PRS-Azimuth and dl-PRS-Elevation are provided in a GCS.

dl-prs-BeamInfoSet This field provides the DL-PRS beam information for each DL-PRS Resource of the DL-PRS Resource Set associated with this TRP.

dl-PRS-Azimuth This field specifies the azimuth angle of the boresight direction in which the DL-PRS Resources associated with this DL-PRS Resource ID in the DL-PRS Resource Set are transmitted. For a Global Coordinate System (GCS), the azimuth angle is measured counter-clockwise from geographical North. For a Local Coordinate System (LCS), the azimuth angle is measured measured counter-clockwise from the x-axis of the LCS. Scale factor 0.1 degrees; range 0 to 359.9 degrees.

dl-PRS-Elevation This field specifies the elevation angle of the boresight direction in which the DL-PRS Resources associated with this DL-PRS Resource ID in the DL-PRS Resource Set are transmitted. For a Global Coordinate System (GCS), the elevation angle is measured relative to zenith and positive to the horizontal direction (elevation 0 deg. points to zenith, 90 deg to the horizon). For a Local Coordinate System (LCS), the elevation angle is measured relative to the z-axis of the LCS (elevation 0 deg. points to the z-axis, 90 deg to the x-y plane). Scale factor 0.1 degrees; range 0 to 180 degrees.

alpha This field specifies the bearing angle α for the translation of the LCS to a GCS as defined in TR 38.901 [x]. Scale factor 0.1 degrees; range 0 to 359.9 degrees.

beta This field specifies the downtilts angle β for the translation of the LCS to a GCS as defined in TR 38.901 [x]. Scale factor 0.1 degrees; range 0 to 359.9 degrees.

gamma This field specifies the slant angle γ for the translation of the LCS to a GCS as defined in TR 38.901 [x]. Scale factor 0.1 degrees; range 0 to 359.9 degrees.

#### –	NR-RTD-Info

The IE NR-RTD-Info is used by the location server to provide time synchronization information between a reference TRP and a list of neighbour TRPs. The list index is the reference used from the DL-PRS assistance data to associate a TRP of the DL-PRS to an element in this list.

-- ASN1START

# NR-RTD-Info-r16 ::= SEQUENCE {

referenceTRP-RTD-Info-r16		ReferenceTRP-RTD-Info-r16,

rtd-InfoList-r16				RTD-InfoList-r16,

...

}

ReferenceTRP-RTD-Info-r16 ::= SEQUENCE {

ref-trp-id-r16					TRP-ID-r16,

refTime-r16						CHOICE {

systemFrameNumber-r16		BIT STRING (SIZE (10)),

utc-r16						UTCTime,

...

},

rtd-RefQuality-r16				NR-TimingMeasQuality-r16				OPTIONAL,	-- Need ON

...

}

RTD-InfoList-r16 ::= SEQUENCE (SIZE (0..254)) OF RTD-InfoElement-r16

RTD-InfoElement-r16 ::= SEQUENCE {

subframeOffset-r16			INTEGER (0..1966079),

rtd-Quality-r16				NR-TimingMeasQuality-r16,

...

}

-- ASN1STOP

NR-RTD-Info field descriptions

referenceTRP-RTD-Info This field defines the reference TRP for the RTD and comprises the following sub-fields: - ref-trp-id: This field specifies the identity of the reference TRP. - refTime: This field specifies the reference time at which the rtd-InfoList is valid. The systemFrameNumber choice refers to the SFN of the reference TRP. - rtd-RefQuality: This field specifies the quality of the timing of reference TRP, used to determine the RTD values provided in rtd-InfoList.

subframeOffset This field specifies the subframe boundary offset at the TRP antenna location between the reference TRP and this neighbour TRP in time units where Δfmax=480∙103 Hz and (TS 38.211 [x]). The offset is counted from the beginning of a subframe #0 of the reference TRP to the beginning of the closest subsequent subframe of this neighbour TRP. Scale factor 1 Tc.

rtd-Quality This field specifies the quality of the RTD.

#### –	NR-DL-PRS-AssistanceData

The IE NR-DL-PRS-AssistanceData is used by the location server to provide DL-PRS assistance data.

-- ASN1START

# NR-DL-PRS-AssistanceData-r16 ::= SEQUENCE {

nr-DL-PRS-ReferenceInfo-r16 DL-PRS-IdInfo-r16	OPTIONAL,		-- Need ON

# nr-DL-PRS-AssistanceDataList-r16	SEQUENCE (SIZE (1..nrMaxFreqLayers)) OF NR-DL-PRS-AssistanceDataPerFreq-r16,

nr-SSB-Config-r16			SEQUENCE (SIZE (0..255)) OF NR-SSB-Config-r16,	...

# }

# NR-DL-PRS-AssistanceDataPerFreq-r16 ::= SEQUENCE {

# nr-DL-PRS-AssistanceDataPerFreq SEQUENCE (SIZE (1..nrMaxTRPsPerFreq)) OF NR-DL-PRS-AssistanceDataPerTRP-r16,

nr-DL–PRS-PositioningFrequencyLayer-r16	NR-DL–PRS-PositioningFrequencyLayer-r16	OPTIONAL,	--Need ON

...

# }

# NR-DL-PRS-AssistanceDataPerTRP-r16 ::= SEQUENCE {

nr-DL-PRS-expectedRSTD-r16		INTEGER (-3841..3841),

nr-DL-PRS-expectedRSTD-uncerainty-r16	INTEGER (-246..246),

# trp-ID-r16					TRP-ID-r16		OPTIONAL,

nr-TRP-UEB-refIndices-r16		NR-TRP-UEB-refIndices-r16	OPTIONAL,	-- Cond UEB

nr-DL-PRS-Config-r16			NR-DL-PRS-Config-r16,

...

# }

NR-TRP-UEB-refIndices-r16 ::= SEQUENCE {

trp-locInfo-index-r16		INTEGER (0..255),

trp-beamInfo-index-r16		INTEGER (0..255)				OPTIONAL,	-- Cond BeamInfo

trp-rtdInfo-index-r16		INTEGER (0..254)				OPTIONAL, 	-- Cond RTDInfo

}

NR-DL–PRS-PositioningFrequencyLayer-r16 ::= SEQUENCE {

dl-PRS-SubcarrierSpacing-r16		ENUMERATED {kHz15, kHz30, kHz60, kHz120, ...},

dl-PRS-ResourceBandwidth-r16		INTEGER (1..63),

dl-PRS-StartPRB-r16					INTEGER (0..2176),

dl-PRS-PointA-r16					ARFCN-ValueNR-r15,

dl-PRS-CombSizeN-r16				ENUMERATED {n2, n4, n6, n12, ...},

dl-PRS-CyclicPrefix-r16				ENUMERATED {normal, extended, ...},

...

}

nrMaxFreqLayers	INTEGER ::= 4	-- Max freq layers

nrMaxTRPsPerFreq		INTEGER ::= 64		-- Max TRPs per freq layers

nrMaxResourceIDs INTEGER ::= 64		-- Max ResourceIDs

-- ASN1STOP

NR-DL-PRS-AssistanceData field descriptions

nr-DL-PRS-Config This field specifies the PRS configuration of the TRP.

nr-DL-PRS-ReferenceInfo This field indicates the IDs of the reference TRP.

nr-DL-PRS-ResourceID-List The list of nr DL PRS resource ID. Only a single NR-DL-PRS-ResourceId is included if the field is used in measurement reporting.

nr-TRP-UEB-refIndices The set of reference indices refers to TRPs in the corresponding lists defined by IEs NR-TRP-LocationInfo, NR-DL-PRS-BeamInfo, and RTD-InfoList.

# Annex 2: Text Proposal for 37.355 Subsection 6.4.1

### 6.4.1	Common Lower-Level IEs

[…]

#### –	NR-DL-PRS-BeamInfo

[…]

NR-PRS-Beam-Info-ResourceSet-r16 ::= SEQUENCE {

nr-PRS-BeamInfoList-r16			SEQUENCE (SIZE(0..63)) OF NR-PRS-BeamInfoElement-r16

lcs-To-GCS-Translation           SEQUENCE {

alpha                            INTEGER (0..359),

alpha-fine                       INTEGER (0..9),      OPTIONAL,  -- Need OP

beta                             INTEGER (0..359)     OPTIONAL,  -- Need OP

beta-fine                        INTEGER (0..9)       OPTIONAL,  -- Need OP

gamma                            INTEGER (0..359)     OPTIONAL   -- Need OP

gamma-fine                       INTEGER (0..9)       OPTIONAL   -- Need OP    },                                                       OPTIONAL,  -- Need OP

...

}

NR-PRS-BeamInfoElement-r16 ::= SEQUENCE {

nr-PRS-Azimuth-r16				INTEGER (0..359),

nr-PRS-Azimuth-fine-r16			INTEGER (0..9),				OPTIONAL,	-- Cond FineAngles

nr-PRS-Elevation-r16			INTEGER (0..180)				OPTIONAL,	-- Need ON

nr-PRS-Elevation-fine-r16		INTEGER (0..9)				OPTIONAL,	-- Cond FineAngles

...

}

-- ASN1STOP

# Annex 3: Text Proposal for 37.355 Subsection 6.4.3 (which should be considered for relocation to 6.4.1)

### 6.4.1	Common Lower-Level IEs

[…]

#### -	NR-TRP-LocationInfo

The IE NR-TRP-LocationInfo is used by the location server to provide the coordinates of the antenna reference points for a set of TRPs. For each TRP, the ARP location can be provided for each associated PRS Resource ID per PRS Resource Set.

-- ASN1START

NR-TRP-LocationInfo-r16 ::= SEQUENCE (SIZE (1..4)) OF NR-TRP-LocationInfoPerFreqLayer-r16

NR-TRP-LocationInfoPerFreqLayer-r16 ::= SEQUENCE {

referencePoint-r16			ReferencePoint-r16				OPTIONAL,	-- Cond NotSameAsPrev

trp-LocationInfoList-r16	SEQUENCE (SIZE (1..64)) OF TRP-LocationInfoElement-r16,

...

}

TRP-LocationInfoElement-r16 ::= SEQUENCE {

trp-id-r16						TRP-ID-r16,

trp-Location-r16				RelativeLocation-r16					OPTIONAL,	-- Need OP

trp-DL-PRS-ResourceSets-r16		SEQUENCE (SIZE(1..2)) OF

DL-PRS-ResourceSets-TRP-Element-r16	OPTIONAL,	-- Need OP

...

}

DL-PRS-ResourceSets-TRP-Element-r16 ::= SEQUENCE {

dl-PRS-ResourceSetARP-r16			RelativeLocation-r16				OPTIONAL,	-- Need OP

dl-PRS-Resource-ARP-List-r16		SEQUENCE (SIZE(1..64)) OF

DL-PRS-Resource-ARP-Element-r16	OPTIONAL,	-- Need OP

...

}

DL-PRS-Resource-ARP-Element-r16 ::= SEQUENCE {

dl-PRS-Resource-ARP-location-r16	RelativeLocation-r16				OPTIONAL,	-- Need OP

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| NotSameAsPrev | The field is mandatory present in the first entry of the NR-TRP-LocationInfoPerFreqLayer list; otherwise it is optionally present, need OP. |

NR-TRP-LocationInfo field descriptions

referencePoint This field specifies the reference point used to define the TRP location in the trp-LocationInfoList. If this field is absent, the reference point is the same as in the previous entry of the NR-TRP-LocationInfoPerFreqLayer list.

trp-LocationInfoList This field provides the antenna reference point locations of the DL-PRS Resources for the TRPs and comprises the following sub-fields: - trp-id: This field provides an identity of the TRP. - trp-Location: This field provides the location of the TRP relative to the referencePoint location. If this field is absent the TRP location coincides with the referencePoint location. - trp-DL-PRS-ResourceSets: This field provides the antenna reference point location(s) of the DL-PRS Resource Set(s) associated with this TRP. If this field is absent, the antenna reference point location(s) of the DL-PRS Resource Set(s) coincides with the trp-Location location. This field comprises the following sub-fields: - dl-PRS-ResourceSetARP: This field provides the antenna reference point location of the DL-PRS Resource Set relative to the trp-Location location. If this field is absent, the antenna reference point location of this DL-PRS Resource Set coincides with the trp-Location location. - dl-PRS-Resource-ARP-List: This field provides the antenna reference point location(s) of the DL-PRS Resource(s) associated with this resource set of the TRP. If this field is absent, the antenna reference point location(s) of the DL-PRS Resources coincides with the dl-PRS-ResourceSetARP location. This field comprises the following sub-fields: - dl-PRS-Resource-ARP-location: This field provides the antenna reference point location of the DL-PRS Resource associated with the DL-PRS Resource Set of the TRP relative to the dl-PRS-ResourceSetARP location. If this field is absent, the antenna reference point location of this DL-PRS Resource coincides with the dl-PRS-ResourceSetARP location.

#### –	ReferencePoint

The IE ReferencePoint provides a well defined location relative to which other locations may be defined.

-- ASN1START

# ReferencePoint-r16 ::= SEQUENCE {

referencePointGeographicLocation-r16 		CHOICE {

location3D-r16 			EllipsoidPointWithAltitudeAndUncertaintyEllipsoid,

ha-location3D-r16 		HighAccuracyEllipsoidPointWithAltitudeAndUncertaintyEllipsoid-r15,

...

},

...

}

-- ASN1STOP

ReferencePoint field descriptions

referencePointGeographicLocation This field provides the geodetic location of the reference point.

#### –	RelativeLocation

The IE RelativeLocation provides a location relative to some known reference location.

-- ASN1START

# RelativeLocationLoLaAlt-r16 ::= SEQUENCE {

milli-arc-second-units-r16 	ENUMERATED { mas0-03, mas0-3, mas3, mas30, ...},

height-units-r16			ENUMERATED {mm, cm, m, ...},

delta-latitude-r16			Delta-Latitude-r16,

delta-longitude-r16			Delta-Longitude-r16,

delta-height-r16			Delta-Height-r16,

locationUNC-r16				LocationUncertainty-r16				OPTIONAL,		-- Need OP

...

}

Delta-Latitude-r16 ::= SEQUENCE {

delta-Latitude-r16						INTEGER (-1024..1023),

coarse-delta-Latitude-r16				INTEGER (0..4095)		OPTIONAL,		-- Need OP

...

}

Delta-Longitude-r16 ::= SEQUENCE {

delta-Longitude-r16						INTEGER (-1024..1023),

coarse-delta-Longitude-r16				INTEGER (0..4095)		OPTIONAL,		-- Need OP

...

}

Delta-Height-r16 ::= SEQUENCE {

delta-Height-r16						INTEGER (-1024..1023),

coarse-delta-Height-r16					INTEGER (0..4095)		OPTIONAL,		-- Need OP

...

}

LocationUncertainty-r16 ::= SEQUENCE {

horizontalUncertainty-r15				INTEGER (0..255),

horizontalConfidence-r15				INTEGER (0..100),

verticalUncertainty-r15					INTEGER (0..255),

verticalConfidence-r15					INTEGER (0..100)

}

RelativeLocationXYZ-r16 ::= SEQUENCE {

xyz-units-r16				ENUMERATED {cm, dm, ...},

delta-x-r16					INTEGER (0..4095),

delta-y-r16					INTEGER (0..4095),

delta-z-r16					INTEGER (0..4095),

locationUNC-r16				LocationUncertainty-r16				OPTIONAL,		-- Need OP

...

}

RelativeLocation-r16 ::= CHOICE {

relativeLocationLoLaAlt-r16		RelativeLocation-r16,

relativeLocationxyz-r16			RelativeLocationXYZ-r16

}

-- ASN1STOP

RelativeLocation field descriptions

milli-arc-second-units This field provides the units and scale factor for the delta-latitude and delta-longitude fields. Enumerated values mas0-03, mas0-3, mas3, and mas30, correspond to 0.03, 0.3, 3, and 30 milliarcseconds, respectively.

height-units This field provides the units and scale factor for the delta-height field. Enumerated values mm, cm, and m correspond to 10-3 metre, 10-2 metre, and 1 metre, respectively.

delta-latitude This field specifies the delta value in latitude of the desired location, defined as "desired location" minus "reference point location" and comprises the following sub-fields: - delta-Latitude specifies the delta value in latitude in the unit provided in milli-arc-second-units field. - coarse-delta-Latitude specifies the delta value in latitude in 1024 times the size of the unit provided in milli-arc-second-units field and with the same sign as in the delta-Latitude field. If this field is absent, the value for coarse-delta-Latitude is zero. I.e., the full delta-latitude is given by: (delta-Latitude × milli-arc-second-units) ± (coarse-delta-Latitude × 1024 × milli-arc-second-units) [milli-arc-seconds]

delta-longitude This field specifies the delta value in longitude of the desired location, defined as "desired location" minus "reference point location" and comprises the following sub-fields: - delta-Longitude specifies the delta value in longitude in the unit provided in milli-arc-second-units field. - coarse-delta-Longitude specifies the delta value in longitude in 1024 times the size of the unit provided in milli-arc-second-units field and with the same sign as in the delta-Longitude field. If this field is absent, the value for coarse-delta-Longitude is zero. I.e., the full delta-longitude is given by: (delta-Longitude × milli-arc-second-units) ± (coarse-delta-Latitude × 1024 × milli-arc-second-units) [milli-arc-seconds]

delta-height This field specifies the delta value in ellipsoidal height of the desired location, defined as "desired location" minus "reference point location" and comprises the following sub-fields: - delta-Height specifies the delta value in ellipsoidal height in the unit provided in height-units field. - coarse-delta-Height specifies the delta value in ellipsoidal height in 1024 times the size of the unit provided in height-units field and with the same sign as in the delta-Height field. If this field is absent, the value for coarse-delta-Height is zero. I.e., the full delta-height is given by: (delta-Height × height-units) ± (coarse-delta-Height × 1024 × height-units) [metres]

xyz-units This field provides the units and scale factor for the delta-x, delta-y and delta-z fields. Enumerated values cm and dm corresponds to centimeter and decimeter, respectively.

delta-x, delta-y This field specifies the delta value in horizontal cartesian coordinates of the desired location, defined as "desired location" minus "reference point location"

delta-x, delta-y This field specifies the delta value in horizontal cartesian coordinates of the desired location, defined as "desired location" minus "reference point location"

delta-z This field specifies the delta value in vertical cartesian coordinates of the desired location, defined as "desired location" minus "reference point location"

locationUNC This field specifies the uncertainty of the location coordinates and comprises the following sub-fields: - horizontalUncertainty indicates the horizontal uncertainty of the ARP latitude/longitude. The ′horizontalUncertainty′ corresponds to the encoded high accuracy uncertainty as defined in TS 23.032 [15] and ′horizontalConfidence′ corresponds to confidence as defined in TS 23.032 [15]. - verticalUncertainty indicates the vertical uncertainty of the ARP altitude. The 'verticalUncertainty' corresponds to the encoded high accuracy uncertainty as defined in TS 23.032 [15] and 'verticalConfidence' corresponds to confidence as defined in TS 23.032 [15]. If this field is absent, the uncertainty is the same as for the associated reference point location.