3GPP TSG-RAN WG2 Meeting #110-e	DRAFT R2-2004599

Elbonia, 01 – 11 June 2020

Title:	[DRAFT] Reply LS on UE declaring beam failure due to LBT failures

Response to:	R4-2005365

Release:	Release 16

Work Item:	NR_unlic

Source:	Nokia [TSG RAN WG2]

To:	TSG RAN WG4

Cc:	TSG RAN WG1

Contact Person:

#### Name:	Samuli Turtinen

E-mail Address:	samuli.turtinen@nokia-bell-labs.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN2 would like to thank RAN4 for providing the LS in R4-2005365 and the question about whether the UE shall declare beam failure due to LBT failures when configured with RRC-based active TCI state switching.

RAN2 has discussed this issue and notes that unavailability of the reference signals (like SSB/CSI-RS) due to DL LBT failures after the TCI state switch would lead to beam failure instance indications from L1 to MAC in the beam failure detection (BFD) procedure. Beam failure will be declared after the number of beam failure instances (e.g., due to LBT failures in DL) reach a network configured threshold and beam failure recovery will be triggered.

RAN2 would also like to note that this does not require UE to detect DL LBT failures and also requires no specification changes to the existing beam failure detection procedure.

2. Actions:

To RAN4 group.

ACTION: 	RAN2 respectfully asks RAN4 to take the above information into account for their further discussions.

3. Date of Next TSG-RAN WG2 Meetings:

3GPP RAN2#111		24 – 28 August 2020			eMeeting