3GPP TSG-RAN WG2 Meeting #110e                                                            R2-2004748

Online, June 1 – June 12, 2020

Agenda item:       6.1.5.2

Source: 		Qualcomm Incorporated

Title: 	IAB - Establishment of F1-C over LTE

Document for:	Discussion

# Introduction

- RAN2 #107e-bis agreed on F1-C transport over LTE/X2 [1]

- The F1-C IP packet over the LTE leg includes the SCTP/IP header. - When an LTE leg is configured, it can be used for F1-C. It is out of RAN3 scope to design how to perform the configuration. - It is up to Donor-CU to decide to only configure LTE leg, or only configure NR leg, or configure both LTE leg and NR leg, for F1-C. - When both LTE leg and NR leg are configured, it is up to node implementation to select a leg for F1-C transfer. - The existing security requirements for F1-C apply to F1-C over LTE leg as well. The F1-C over LTE leg reuses the existing security mechanisms for F1-C interface. - Separate IP address pairs {IAB-DU’s IP address, CU’s IP address} should be used for NR leg and LTE leg. - FFS on whether same SA/NSA IP address allocation procedures for the IP address assignment for F1-C over LTE leg - No impact to X2 interface on setting up LTE leg for F1-C.

This paper discusses implications of the F1-C path establishment on RRC configuration.

# 2	Discussion

Based on RAN3 agreement, the IAB-node needs to receive an indication from the IAB-donor-CU if F1-C should use LTE only, NR only, or both LTE and NR. This indication must be sent via NR RRC, i.e., before F1-C has been established.

Proposal 1: The IAB-donor to send an explicit indication to the IAB-node via NR RRC if F1-C should use LTE only, NR only, or both LTE and NR.

It is most convenient to include the indication in the RRCReconfiguration message.

Proposal 2: RRCReconfiguration message to be used for this indication.

# 3	Conclusion

This paper discussed implications of the F1-C path establishment on RRC configuration. The following proposals have been made:

Proposal 1: The IAB-donor to send an explicit indication to the IAB-node via NR RRC if F1-C should use LTE only, NR only, or both LTE and NR.

Proposal 2: RRCReconfiguration message to be used for this indication.

# 4	References

[1] Chairman notes, 3GPP TSG RAN WG3 Meeting #107e-bis, Electronic Meeting, April 20 – April 30, 2020