3GPP TSG-RAN WG2 Meeting #110		R2-200xxxx

Electronic, 01 June - 12 June, 2020

Agenda item:	5.4.3.1

Source:	Apple

Title:	Summary of email discussion [AT110-e][015][NR15] UE cap FR2 Fallback (Apple)

Document for:	Discussion

# 1	Introduction

This is to summarize the following offline email discussion.

[AT110-e][015][NR15] UE cap FR2 Fallback (Apple)

Scope: Progress CRs, based on R2-2004754 and R2-2004754

Part 1: Can kick off email discussion to gather more comments on the CRs, awaiting on-line treatment.

Part 2: Technically Endorsed CRs for RP. Deadline: June 10, 0700 UTC

# 2   Discussion

From the summary on email discussion [Post109bis-e][921][NR15] CRs for FR2 CA Fallback (Apple), rapporteur list several open issues to gather more comments on the CRs (R2-2004754, R2-2004755).

Issue 1: For inter-node signaling (CG-Config, CG-ConfigInfo), current design from R2-2004754 is to keep one bandCombinationIndex, which follows the design made for supportedBandCombinationListNEDC-Only. We received some offline comments proposing whether to introduce a separate bandCombinationIndex for supportedbandcombinations in rf-Parameters-FR2-CA-Fallback and rf-ParametersMRDC-FR2-CA-Fallback, in order to make sure it's clear which band combination list each index the MN indicates refers to.

Q1: Do companies think the current change shown below to bandCombinationIndex is sufficient and clear enough?

BandCombinationInfoSN field descriptions

bandCombinationIndex In case of (NG)EN-DC and NR-DC, this field indicates the position of a band combination in the supportedBandCombinationList. In case of NE-DC, this field indicates the position of a band combination in the supportedBandCombinationList and/or supportedBandCombinationListNEDC-Only. Band combination entries in supportedBandCombinationList are referred by an index which corresponds to the position of a band combination in the supportedBandCombinationList. Band combination entries in supportedBandCombinationListNEDC-Only are referred by an index which corresponds to the position of a band combination in the supportedBandCombinationListNEDC-Only increased by the number of entries in supportedBandCombinationList. In case rf-Parameters-FR2-CA-Fallback and/or rf-ParametersMRDC-FR2-CA-Fallback exist, band combination entries in rf-Parameters-FR2-CA-Fallback and/or rf-ParametersMRDC-FR2-CA-Fallback are referred by an index which corresponds to the position of a band combination in the rf-Parameters-FR2-CA-Fallback and/or rf-ParametersMRDC-FR2-CA-Fallback increased by the number of band combination entries in rf-Parameters and/or rf-ParametersMRDC.

| Companies | Answer (Yes/No) | Comments |
| --- | --- | --- |

Issue 2: One comment received during email discussion [Post109bis-e][921][NR15] CRs for FR2 CA Fallback (Apple) was not captured into the draft CR submitted in R2-2004754 and R2-2004755. The proposed change is to add the highlighted text “in a cell group”. The purpose is to clarify that “UE can only fallback to one single carrier on FR2” means for intra-band FR2 SCG, UE only fallbacks to one single carrier. But the MCG part is not impacted. Otherwise, UE is not able to support DC anymore. Rapporter thinks this is a straightforward clarification.

Q2: Do companies agree with the additional text “in the cell group” (highlighted in yellow) in the field descriptions?

UE-MRDC-Capability field descriptions

featureSetCombinations A list of FeatureSetCombination:s for supportedBandCombinationList and supportedBandCombinationListNEDC-Only in UE-MRDC-Capability. The FeatureSetDownlink:s and FeatureSetUplink:s referred to from these FeatureSetCombination:s are defined in the featureSets list in UE-NR-Capability.

rf-ParametersMRDC-FR2-CA-Fallback A list of band combinations that the UE supports for MR-DC with FR2 intra-band non-contiguous CA, on which the UE only supports fallback band combinations resulting in a single FR2 carrier in the cell group. The band combinations that are allowed to be reported in this list are defined in [34] and [39]. The FeatureSetCombinationId:s in this list refer to the FeatureSetCombination entries in the featureSetCombinations list in the UE-MRDC-Capability IE.

UE-NR-Capability field descriptions

featureSetCombinations A list of FeatureSetCombination:s for supportedBandCombinationList in UE-NR-Capability. The FeatureSetDownlink:s and FeatureSetUplink:s referred to from these FeatureSetCombination:s are defined in the featureSets list in UE-NR-Capability.

rssi-CO-Measurements Indicates whether the UE supports performing RSSI and Channel Occupancy (CO) measurements for operation with shared spectrum channel access.

rf-Parameters-FR2-CA-Fallback A list of band combinations that the UE supports for NR (and NR-DC, if requested) with FR2 intra-band non-contiguous CA, on which the UE only supports fallback band combinations resulting in a singler FR2 carrier in a cell group. The band combinations that are allowed to be reported in this list are defined in [39]. The FeatureSetCombinationId:s in this list refer to the FeatureSetCombination entries in the featureSetCombinations list in the UE-NR-Capability IE.

| Companies | Answer (Yes/No) | Comments |
| --- | --- | --- |

Q3: If companies have other comments on the CR set, please bring them here.

| Companies | Comments |
| --- | --- |

# 3	Conclusions

xx