3GPP TSG-RAN WG2 Meeting #111-e                                                     R2-2006631

Electronic, 17th - 28th August, 2020

Source:	CATT

Title:	Discussion on MAC Enhancement and Impact for NTN

Agenda Item:	8.10.2.1

Document for:	Discussion and Decision

Introduction

In RAN#86 meeting, the NTN WID was approved [1]. In this contribution, we are going to discuss on MAC Enhancement for NTN according to the WID, in order to finalize the impact on MAC protocols.

Discussion

The following user plane procedures enhancements in MAC should be specified for NTN:

  - Random access

  - Enhancement on UL scheduling to reduce scheduling latency

  - DRX

  - Scheduling Request: Extension of the value range of sr-ProhibitTimer

We are going to discuss the impact and propose the value for these parameters one by one.

According to the WID, we assumed UE is aware of its location information by itself in this contribution. The exception that UE doesn’t know its location is not in the scope of this contribution.

Random access

The RTT is used a lot in this contribution. The parameters will refer to the contribution R2- 2007572 NR NTN Reference scenarios definition for Rel-17 normative phase [2] as baseline definition. Here Round Trip Delay is [8ms, 542ms] supporting the three typical scenarios as below.

Table 4.2-1 Reference satellite scenarios: key parameters [2]

| Scenarios | Scen C1: NGSO based network | Scen C2: NGSO based network | Scen A: GEO based network |
| --- | --- | --- | --- |
| Altitude | 600 km | 1200 km | 35,786 km |
| Max beam foot print size (edge to edge) regardless of the elevation angle | 1000km | 1000 km | 3500 km (note 5) |
| Min Elevation angle for both sat-gateway and user equipment | 10° for service link and 10° for feeder link | 10° for service link and 10° for feeder link | 10° for service link and 10° for feeder link |
| Max distance between satellite and user equipment at min elevation angle | 1932.0 km | 3132.0 km | 40,581 km |
| Max Round Trip Delay (propagation delay only) | 25.77 ms | 41.76 ms | 541.46 ms (service and feeder links) |
| Minimum propagation delay contribution to the Round Trip Delay on the radio interface between the gNB and the UE. | 8 ms | 16 ms | 477.48 ms |
| Max differential delay within a cell (Note 6) | 3.12 ms | 3.18 ms | 10.3 ms |

Offset for the start of the ra-ResponseWindow

The behaviour of ra-ResponseWindow should be modified to support NTN. So the offset of ra-ResponseWindow shall be evaluated at first within different scenarios.

Table 2-1 Reference satellite scenarios: Offset for the start of the ra-ResponseWindow

| Scenarios | Scen C1: NGSO based network | Scen C2: NGSO based network | Scen A: GEO based network |
| --- | --- | --- | --- |
| Max Round Trip Delay (propagation delay only) | 25.77 ms | 41.76 ms | 541.46 ms (service and feeder links) |
| Minimum propagation delay contribution to the Round Trip Delay on the radio interface between the gNB and the UE. | 8 ms | 16 ms | 477.48 ms |

There are two options to evaluate the offset for the start of the ra-ResponseWindow.

- Option1: The value of the offset for the start of the ra-ResponseWindow is broadcast by network.

- Option2: UE calculates the accurate offset by itself based on its location info and the broadcast RTDmin from network.

RTDmin is defined as Minimum propagation delay contribution to the Round Trip Delay on the radio interface between the gNB and the UE in [2].

the RTDmin is required from network within different scenarios in either of the solutions.

From the RAN2 perspective, the broadcast RTTmin is enough for all UEs. UE can calculate the accurate offset by itself based on its location info and RTTmin to save power.

Proposal 1: Network broadcast RTDmin which is the Minimum propagation delay contribution to the Round Trip Delay on the radio interface between the gNB and the UE in SI message to all UEs. UE calculates the accurate offset by itself based on its location info and RTDmin to save power.

Offset for the start of the ra-ContentionResolutionTimer

When the UE sends an RRC Connection Request (Msg3), it will monitor for Msg4 in order to resolve a possible random-access contention. The ra-ContentionResolutionTimer starts after Msg3 transmission. The maximum configurable value of ra-ContentionResolutionTimer is large enough to cover the Round Trip Delay in NTN. However, to save UE power, the behavior of ra-ContentionResolutionTimer should be modified to support NTN. The offset can be decided by UE in different scenarios according to the proposal1.

Proposal 2: UE can calculate the accurate offset of the ra-ContentionResolutionTimer by itself based on its location info and RTDmin to save power.

Solutions for resolving preamble ambiguity and extension of RAR window

Assuming the configured offset to delay the start of the RAR window equals to 2 * minimum delay and neglecting the process delay between reception of preamble and transmission of RA Response at gNB side, it can be observed that the RAR monitoring duration shall cover at least 2 * maximum differential delay. Otherwise RAR for UE will fall out of RAR window. The maximum differential delay is defined as maximum one way delay minus minimum one way delay. Furthermore, time flexibility is required for the NW to schedule the RARs which means several milliseconds should be added on top of the 2 * maximum differential delay. Usually, RAR window length = ProcessingDelaygNB + RTTmax – RTTmin.

Figure 2-1 RAR window offset and length

There are two options to define the ra-ResponseWindow.

- Option1: The value of the ra-ResponseWindow is sent from network.

- Option2: UE calculates the accurate offset by itself based on its location info, so there is no need to extend the ra-ResponseWindow.

According to the previous proposal 1 and proposal 2, UE can calculate the accurate offset by itself based on its location info.

Proposal 3: UE calculates the accurate offset by itself based on its location info, so there is no need to extend the ra-ResponseWindow.

Enhancement to timing advance

Based on WID scope, it is assumed that UE has GNSS capability and can be able to conduct time-frequency compensation based on UE position and ephemeris information. For the UE without pre-compensation capability, it is not focus of Rel-17 WID.

TA compensation based on full TA and UE specific differential TA had been discussed in SI state. One example of TA calculation in the bent-pipe payload case is illustrated in figure 2-2.

Figure 2-2: Common reference TA and UE specific differential TA calculation

- Full TA compensation

As shown in the in Figure 2-2, full TA=2*(d1+d0_F)/c, c is the speed of light. Based on the assumption of UE with GNSS capability, the TA for service link part can be calculated based on the UE location and satellite ephemeris, while the TA for feeder link part will be indicated by the gNB. Due to unknown Gateway position, UE is hard to track distance variation of the feeder link. Though some information indication may help UE to finish the TA calculation, frequent indication may increase signaling overhead and complicate UE behavior.

- UE-specific TA

Compared with the full-TA mode, the UE only needs to compensate for the differential TA, which reduces UE effort. The differential TA is equal to 2*(d1-d0/c, and the common TA is equal to 2*(d1+d0_F)/c.  Within this method, gNB will compensate the common part TA, based on the position information of reference point, satellite and Gateway. Considering the implementation flexibility, Reference point can be set in the satellite or on the earth. If reference point is in the satellite, UE is only required to compensate the service link TA, and gNB compensates the feeder link TA.

The outstanding drawbacks of full TA may include:

- UE calculates the service link TA by itself and should compensate the TA of feeder link. But UE is not aware of the distance of Gateway.

- gNB should broadcast the feeder link to UE. But the feeder link changes frequently which costs large overhead when updating the broadcast feedlink to UE from time to time.

On the contrary, no feeder link value is broadcast to UE in UE-Specific differential TA method.

- UE just calculates the service link TA by itself and gNB maintains the feeder link TA(common part for all UEs) which won’t broadcast to UE.

Observation 1: Compensation of full TA would require additional signaling overhead, and make behaviors between UE and gNB more complicated.

Proposal 4: UE only needs to compensate the UE-specific differential TA.

Enhancement on UL scheduling

Figure 7.2.1.5-1: Scheduling of UE transmission

In non-terrestrial networks the drawback of this procedure is that it would take at least 2 Round-trip times from data arriving in the buffer at the UE side until it can be properly scheduled with resources that would fit the data and the required QoS.

There are several potential solutions to enhance the UL scheduling, such as:

- Option1: Sending the buffer status by SR, BSR can be omitted.

- Option2: Sending data by Type 1 configured grant, instead of SR/BSR to reduce the large delay.

Option2 is already supported in existed mechanism. So there is no need to introduce new enhancement on UL scheduling.

Proposal 5: No need to introduce new enhancement on UL scheduling in NTN.

DRX

Offset for drx-HARQ-RTT-TimerDL and drx-HARQ-RTT-TimerUL

If HARQ feedback is enabled, offset for drx-HARQ-RTT-TimerDL and drx-HARQ-RTT-TimerUL will be introduced. The drx-HARQ-RTT-TimerDL is the minimum duration before a downlink assignment for HARQ retransmission is expected by the MAC entity. In terrestrial communications this is configurable in the range of a few ms, which is too small for a communication-link with a satellite. drx-HARQ-RTT-TimerUL is the same as drx-HARQ-RTT-TimerDL just for the uplink. If HARQ is supported by NTN, the handling of drx-HARQ-RTT-TimerDL and drx-HARQ-RTT-TimerUL, should be modified to support NTN.

Table 2-3 Reference satellite scenarios: Expected drx-HARQ-RTT-TimerDL and drx-HARQ-RTT-TimerUL

| Scenarios | Scen C1: NGSO based network | Scen C2: NGSO based network | Scen A: GEO based network |
| --- | --- | --- | --- |
| Max Round Trip Delay (propagation delay only) | 25.77 ms | 41.76 ms | 541.46 ms (service and feeder links) |
| Minimum propagation delay contribution to the Round Trip Delay on the radio interface between the gNB and the UE. | 8 ms | 16 ms | 477.48 ms |
| drx-HARQ-RTT-TimerDL drx-HARQ-RTT-TimerUL symbol@120KHz | 8*8*14 = 896 | 16*8*14=1792 | 477*8*14=53424 |
| drx-HARQ-RTT-TimerDL drx-HARQ-RTT-TimerUL symbol@240KHz | 8 ms*16 slot*14 symb =1792 | 3584 | 106848 |

Proposal 6: drx-HARQ-RTT-TimerDL and drx-HARQ-RTT-TimerUL should be extended to support NTN:

drx-HARQ-RTT-TimerDLExt                INTEGER (0.. 106848),

drx-HARQ-RTT-TimerULExt                	INTEGER (0.. 106848),

SR

A UE can use a Scheduling Request (SR) to request UL-SCH resources from the gNB for a new transmission or a transmission with a higher priority. SR transmission is configured by RRC. During the prohibit timer (sr-ProhibitTimer) is active, no further SR is initiated. The sr-ProhibitTimer will at latest expire after 128ms and initiate a SR. For GEO systems the value range is not sufficient because the RTD is larger.

So the sr-ProhibitTimer should be modified to support NTN.

Table 2-4 Reference satellite scenarios: Expected sr-ProhibitTimer

| Scenarios | Scen C1: NGSO based network | Scen C2: NGSO based network | Scen A: GEO based network |
| --- | --- | --- | --- |
| Max Round Trip Delay (propagation delay only) | 25.77 ms | 41.76 ms | 541.46 ms (service and feeder links) |
| Minimum propagation delay contribution to the Round Trip Delay on the radio interface between the gNB and the UE. | 8 ms | 16 ms | 477.48 ms |
| Expected sr-ProhibitTimer | >26ms | >42ms | >542ms |
| existed sr-ProhibitTimer | ENUMERATED {ms1, ms2, ms4, ms8, ms16, ms32, ms64, ms128} |  |  |

Proposal 7: sr-ProhibitTimer should be extended to support NTN: the extended range can be [ ms256, ms512, ms1024, ms2048].

Conclusion

According to the analysis in section 2, it is proposed:

Proposal 1: Network broadcast RTDmin which is the Minimum propagation delay contribution to the Round Trip Delay on the radio interface between the gNB and the UE in SI message to all UEs. UE calculates the accurate offset by itself based on its location info and RTDmin to save power.

Proposal 2: UE can calculate the accurate offset of the ra-ContentionResolutionTimer by itself based on its location info and RTDmin to save power.

Proposal 3: UE calculates the accurate offset by itself based on its location info, so there is no need to extend the ra-ResponseWindow.

Observation 1: Compensation of full TA would require additional signaling overhead, and make behaviors between UE and gNB more complicated.

Proposal 4: UE only needs to compensate the UE-specific differential TA.

Proposal 5: No need to introduce new enhancement on UL scheduling in NTN.

Proposal 6: drx-HARQ-RTT-TimerDL and drx-HARQ-RTT-TimerUL should be extended to support NTN:

drx-HARQ-RTT-TimerDLExt                INTEGER (0.. 106848),

drx-HARQ-RTT-TimerULExt                	INTEGER (0.. 106848),

Proposal 7: sr-ProhibitTimer should be extended to support NTN: the extended range can be [ ms256, ms512, ms1024, ms2048].

Reference

- RP-193234 Solutions for NR to support non-terrestrial networks (NTN)

- R2-2007572 NR NTN Reference scenarios definition for Rel-17 normative phase  Thales

- TR38.821 Solutions for NR to support non-terrestrial networks (NTN) V16.0.0