3GPP TSG-RAN WG2 Meeting #111 electronic	                                            R2-2006960

Online, August 17th - 28th, 2020

Agenda Item:	8.4.1

Source:	AT&T

Title:	Multi-hop scheduling and local routing enhancements for IAB

Document for:	Discussion/Approval

# Introduction

As part of the Work Item on Enhancements for Integrated Access and Backhaul for NR [1], 3GPP has agreed to support the following enhancements:

Topology, routing and transport enhancements [RAN2-led, RAN3]:

- Specifications of enhancements to improve topology-wide fairness, multi-hop latency and congestion mitigation

In order to support efficient and optimized operation, inter-IAB node metric exchange for IAB scheduling should be supported as well as local routing decisions beyond what is allowed in Rel-16. This contribution analyses the requirements for introduction of a scheduling benefit metric and local routing enhancements for IAB.

# Scheduling Metric for QoS Enforcement

Consider a multi-hop IAB relay architecture, where a UE accesses a serving IAB node, which then connects via IAB links to other IAB nodes to reach the IAB donor. This architecture is demonstrated in Figure 1 [2].

Data generated at a UE in Figure 1 must propagate through the network in a timely manner, subject to QoS of radio bearer established by the CU. The adaptation layer provides QoS-enforcement by the scheduler on DL and UL on the wireless backhaul link [2]. In a multi-hop IAB-network, there is a scheduler in the DU for each IAB-node.

Observation 1: Successful propagation of UE UP data relies on multiple scheduler decisions.

Figure 1: Example SA IAB architecture

All UE UP data arriving at the MT of an IAB-node must leave the IAB-node from the DU. To balance the transmission of data and optimize the scheduling, the parent IAB-node should have information beyond buffer status to prioritize bearers, subject to other QoS constraints. This concept has been used in 3GPP prior for CoMP in the X2-AP specification. A signed integer is exchanged to indicate the cost or benefit of CoMP hypotheses. Such a concept can be used to inform parent IAB-nodes of bearer status downstream.

Additionally, the scheduling of data for the DL IAB transmission (backhaul link to child node or access link) resides at the IAB-donor or IAB-node, while scheduling of data for the UL IAB transmission (backhaul link to parent node) is scheduled by the parent IAB-node. If a certain UE or downstream IAB-node is aggressively scheduled in the UL by the IAB-node, the parent-IAB node can be notified to provide load balancing.

Furthermore, there could be situations in which the scheduler at the parent-IAB node may not have all the information needed to make appropriate scheduling decisions for different bearers/RLC channels across multiple child-IAB nodes. For example, if the backhaul link to one child-IAB node supports three UE bearers with 1:1 mapping to RLC channels, and a second child-IAB node supports three UE bearers with 3:1 mapping to a single RLC channel, the scheduler at the parent-IAB node may see four RLC channels, and a normal proportional-fair metric for each RLC channel may not translate into more scheduling opportunities for the RLC channel that multiplexes three UE bearers. For such and other examples, it may be useful to provide some benefit metric from child-IAB nodes to parent-IAB nodes to allow the parent-IAB node to provide appropriate weighting of bearers as well. In addition, such a benefit metric could incorporate link quality across multiple hops for a given route, based on L1 (e.g. CSI) or L3 (e.g. RRM) measurements for example.

Figure 2: Comparison of E2E user throughput with or without IAB specific scheduling weights at IAB nodes/donors

Figure 2 illustrates the benefit of providing information about the downstream access/backhaul link radio conditions at the schedulers of higher-up IAB nodes or donor nodes. System simulations based on assumptions in the IAB TR [2] are used to compare the case where the scheduling metric at each parent node is only based on the reported feedback of the directly connected child nodes with the case where the scheduling metric is computed with additional weights applied depending on the E2E capacity of the route (e.g. the mutual information expressed as log2(1+SINR)). The SINR is derived from either L3 metrics (the backhaul/access link RSRP values) or L1 metrics (exponential effective SNR mapping values derived from MT/UE reported CSI).

Specifically, the benefit metric is computed as an additional weight applied by the parent node to the child MT’s k Proportional Fair (PF) scheduling metric:

PF_k(t) = IAB_bonus*(R_k,instantaneous(t)/R_k, average(t)) ,  (1)

where:

R_k,instantaneous(t) is the instantaneous throughput of the MT at time t,

R_k, average(t) is the average throughput of the MT up to time t, and

IAB_bonus is the additional weight applied to MTs compared to access UEs

Further:

IAB_bonus (L3 E2E Bonus)  = log2(1 + min(RSRP{k}_a/b))

IAB_bonus (L1 E2E Bonus) = log2(1 + min(EESM{k}_a/b))

where:

RSRP{k}_a/b = The set of RSRP values of the access and backhaul link(s) associated with the route selected by IAB node k derived from RRM measurements.

EESM{k}_a/b = The set of exponential effective SNR mapping (EESM) values of the access and backhaul link(s) associated with the route selected by IAB node k derived from CSI measurements.

The results in Figure 2 shows the significant benefits of this form of radio-aware scheduling, especially for the lower and median ranges of the L2 throughput distribution. The gains are mainly derived from the new ability for the higher-up node in the topology to be able to prioritize routes which may suffer significant congestion due to a large number of downstream nodes, and the large variances between the access and backhaul link quality which cannot be directly derived by the intermediate IAB node’s or donor node’s feedback from its serving links. It should also be noted that the larger gains are observed with the L1 E2E Bonus, which cannot be provided by a central entity such as the CU-CP as is possible in case of the L3 E2E Bonus which uses RRM measurements. Instead, the L1 Bonus requires tunnelling the feedback or the values of the weights to be applied in the PF scheduling via L2 signalling.

Observation 1: Compared to centralized approaches, IAB network performance can be improved by L2 signalling of a benefit metric which can be used to communicate weighting of bearers/RLC channels between schedulers of different IAB-nodes, e.g. taking L1/L3 link quality measurements and the topology associated with different routes into account.

Given that the BAP sublayer already supports the exchange of control information directly between parent and child nodes, it would be straightforward to add the BH scheduling benefit metric as one of the support functions, along with the existing functions captured in 38.300 for IAB:

The main service and functions of the BAP sublayer include:

-	Transfer of data;

-	Routing of packets to next hop;

-	Determination of BAP destination and path for packets from upper layers;

-	Determination of egress RLC channels for packets routed to next hop;

-	Differentiating traffic to be delivered to upper layers from traffic to be delivered to egress link;

-	Flow control feedback signalling;

-	BH RLF notification;

- 	BH scheduling benefit metric;

Proposal 1: Add the exchange of a backhaul scheduling benefit metric to the list of functions of the BAP sublayer in 38.300.

As discussed previously, depending on the scenario and different IAB node capabilities/scheduler algorithms, different formulations of the benefit metric can be derived (e.g. based on L1 vs. L3 metrics). In Rel-17 RAN2 should introduce BAP control PDUs with one or more fields which allow the introduction of different values of the scheduling metric.

Proposal 2: Field(s) in the BAP control PDU should be defined for carrying a backhaul scheduling benefit metric(s).

# Local routing

In a multi-hop IAB network where multiple routes can be configured to the same destination node, the donor CU can configure routing tables at IAB nodes. When there is a link failure at an IAB node, per agreement from RAN2#108, the IAB node performs a local path selection:

If there is a packet with a path ID with no matching entry in the routing table, routing is done based on destination address.

Packet re-routing when there is a matching path ID in the routing table is only supported in egress-link-not-available (RLF) scenario.

We do not introduce support for path ID modification not even for re-routing (modification is not strictly forbidden but no support)

In a multi-hop IAB network where multiple routes can be configured to the same destination node, the donor CU can configure routing tables, and even when there is no link failure, when at least two paths are configured at an IAB node for the same destination node, populating a donor-managed cost metric per path in routing tables at IAB nodes allows the donor CU to perform semi-static load balancing at any IAB node. Given that Rel-17 is considering additional scenarios for IAB deployments including scenarios where topology adaptation events are more frequent and may even span donor nodes, RAN2 should revisit the limitation on re-routing for only link failure scenarios.

Proposal 3: In Rel-17, RAN2 should consider support for local re-routing beyond link failure scenarios.

# Conclusion

This contribution discussed requirements and solutions for IAB radio-aware scheduling and local routing. The following proposals were made:

Proposal 1: Add the exchange of a backhaul scheduling benefit metric to the list of functions of the BAP sublayer in 38.300.

Proposal 2: Field(s) in the BAP control PDU should be defined for carrying a backhaul scheduling benefit metric(s).

Proposal 3: In Rel-17, RAN2 should consider support for local re-routing beyond link failure scenarios.

# References

[1] RP-201293 New WID on Enhancements to Integrated Access and Backhaul, Qualcomm

[2] TR 38.874 1.0.0, Study on Integrated Access and Backhaul