3GPP TSG-RAN WG2 #111-e	Tdoc R2-2007197

Online, 17 – 28 August 2020

Agenda Item:	8.6.2

Source:	Ericsson

Title:	2-step and 4-step based RACH Small Data transmission

Document for:	Discussion, Decision

# 1	Introduction

In RAN#86, a new work item on the support of NR small data transmissions in INACTIVE state was approved [1]. The first objective of the WI is the following:

- UL small data transmissions for RACH-based schemes (i.e. 2-step and 4-step RACH): - General procedure to enable UP data transmission for small data packets from INACTIVE state (e.g. using MSGA or MSG3) [RAN2] - Enable flexible payload sizes larger than the Rel-16 CCCH message size that is possible currently for INACTIVE state for MSGA and MSG3 to support UP data transmission in UL (actual payload size can be up to network configuration) [RAN2] - Context fetch and data forwarding (with and without anchor relocation) in INACTIVE state for RACH-based solutions [RAN2, RAN3] Note 1: The security aspects of the above solutions should be checked with SA3

In this contribution, we look at solutions for this objective and what the potential issues are.

# 2	Discussion

Transmission of small data is a scenario that has been scrutinized in many 3GPP releases, but so far not for NR. Looking at a basic NR case for transmitting a packet shows that a very large portion of the time of the transmission is the connection setup (left figure). Latency in this case is closely correlated to power consumption. The right figure shows the subsequent transmission, after connection setup has been completed.

Figure 1: Simulated end-to-end latency from transmission of one UL packet. (left) UE starts from RRC_INACTIVE  (right) UE is RRC_CONNECTED when transmission starts. The bar labelled “min” is a theoretical minimum.

For LTE several solutions have been introduced to reduce the control signaling overhead for small data transmissions. RRC Suspend/Resume, a.k.a. CIoT UP-optimization, which is similar to RRC INACTIVE state in NR, was introduced in Rel-13. Early Data Transmission (EDT) was introduced in Rel-15 for LTE-M and NB-IoT. NR Small Data transmission in INACTIVE state would be expected to be very similar to UP-EDT, that is the EDT solution specified for the CIoT user plane optimization. A larger UL grant for Msg3 transmission could be provided to the UE, e.g. indicated using separate preambles as for EDT, and the user-plane data payload could be multiplexed with the RRC message in Msg3.

In general, EDT consists of the following steps, which we think can be used as a baseline to continue the discussion and work towards NR small data solution:

- Indication of UEs intention to use small data transmission. This is achieved through preamble partitioning in LTE and it can be discussed further what alternatives we have for the NR case.

- Random access response indicating larger grant size is possible for Msg3 transmission including RRC signaling and data. In LTE, the UE may select between number of potential TB sizes according to configuration. This can increase the decoding complexity on NW side, and any such increase in complexity should be motivated for the NR case. It might be better to consider fixed TB size as provided by the NW in RAR.

- Multiplexing of data and RRC signaling in Msg3. This is the main feature enabled through the EDT functionality in LTE, and similar solution should be specified for NR. This can be further seen as a common building block for both RACH based and possible pre-configured PUSCH based solution (cf. PUR in LTE, which uses similar concept). Additionally, possible inclusion of a release assist indication should be considered.

- Msg4 transmission, possibly including payload as response to upload message in Msg3. The UE should be able to close the connection and remain in INACTIVE. With possible optimizations in signaling.

Figure 2: 4-step RACH-based Small Data

4-step RACH-based Small Data is similar to LTE UP-EDT.

2-step RACH was introduced in Rel-16, but without support for inclusion of user-plane data in MsgA when the UE is in RRC_IDLE or RRC_INACTIVE (CCCH LCH).  This was a result of the Rel-16 2-step RACH WID containing the following note:

Note 2: UP data transmission in RRC_IDLE and RRC_INACTIVE state is not in the scope. UP data transmission in RRC_CONNECTED mode as in Rel-15 NR is supported.

In principle it should be straight forward to introduce the support for resource selection and larger MsgA PUSCH to fit small data, and to allow for multiplexing of user-plane data for UEs in RRC INACTIVE.

Figure 3: 4-step RACH-based Small Data

Based on this we propose the following:

Use the four steps in Figure 2 as baseline for 4-step RACH based small data.  User plane data should be multiplexed with the RRC message in Msg3, further details of the steps and possible optimizations for NR should be discussed further.

Support multiplexing of user-plane data in MsgA in RRC_INACTIVE and larger MsgA PUSCH TBSs.

The last sub-bullet on the objective covers context fetch and data forwarding. For Small Data we think support for context fetch and data forwarding with anchor re-location must be supported and should be the base solution, e.g. support cases with subsequent data and maintain forward security. In our understanding RNA update is the primary reason for supporting context fetch and data forwarding without anchor re-location (i.e. path switch). But since for Small Data latency is not an issue, we see little motivation for supporting also this solution.

The details of context fetch and anchor re-location should be coordinated together with RAN3.

Context fetch and data forwarding with anchor re-location is used for RACH based schemes.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	4-step RACH-based Small Data is similar to LTE UP-EDT.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Use the four steps in Figure 2 as baseline for 4-step RACH based small data.  User plane data should be multiplexed with the RRC message in Msg3, further details of the steps and possible optimizations for NR should be discussed further.

Proposal 2	Support multiplexing of user-plane data in MsgA in RRC_INACTIVE and larger MsgA PUSCH TBSs.

Proposal 3	Context fetch and data forwarding with anchor re-location is used for RACH based schemes.

# References

RP-201305, Revised WID on NR small data transmissions in INACTIVE state, ZTE Corporation, Sanechips, RAN#88e, Online, June 2020.