3GPP TSG-RAN WG2 #111e	Tdoc R2-2007598

Electronic meeting, August 17th – 28th 2020

Agenda Item:	8.2.2

Source:	Ericsson

Title:	Efficient SCG/SCell (de)activation

Document for:	Discussion, Decision

# Introduction

This contribution is to discuss the following objective of the Rel-17 WI on MR-DC further enhancements [1]:

- Support efficient activation/de-activation mechanism for one SCG and SCells

- Support for one SCG applies to (NG)EN-DC, and NR-DC [RAN2, RAN3, RAN4]

- Support for SCells applies to NR CA, based on RAN1 leading mechanisms [RAN1, RAN2, RAN4]

- This objective applies to FR1 and FR2

As reference, the following justification is given for the objective:

“In EN-DC, UE and network power consumption is a big issue, due to maintaining two radio links simultaneously. In some cases, NR UE power consumption is 3 to 4 times higher than LTE. In EN-DC deployment, MN provides the basic coverage. When UE data rate requirement changes dynamically, e.g. from high to low, SN is worth considering to be (de)activated to save network and UE energy consumption. This issue has been discussed in Release 16 eDCCA WI and has significant support. But due to time limitation, it is not completed in Release 16. Therefore, an efficient SCG (de)activation mechanism should be specified in Release 17. This efficient SCG (de)activation mechanism can also be applied to other MR-DC options.”

# Discussion

## Rel-15 baseline

Before starting the discussion on Rel-17 improvements, it is good to review the Rel-15 baseline for reducing UE and network power consumption caused by SCG.

### SCG release/addition

UE and network power consumption can be reduced by releasing the SCG when it is not needed. However, adding the SCG back using the SCG addition procedure (see Figure 1) is a time consuming operation. RAN4 has defined the requirement on PSCell Addition Delay in [12] as follows:

Tconfig_PSCell = TRRC_delay + Tprocessing + Tsearch + T∆ + TPSCell_ DU + 2 ms

where

TRRC_delay is the RRC procedure delay. SCG addition TRRC_delay = 16 ms.

Tprocessing is the SW processing time needed by UE, including RF warm up period. Tprocessing = 40 ms.

Tsearch is the time for AGC settling and PSS/SSS detection. 	If the target cell is known, Tsearch = 0 ms. If the target cell is unknown, provided that the side condition Ês/Iot ≥ [-2]dB is fulfilled, Tsearch = 24* Trs ms.

T∆ is time for fine time tracking and acquiring full timing information of the target cell. T∆ = 1*Trs ms for a known and for an unknown PSCell.

TPSCell_ DU is the delay uncertainty in acquiring the first available PRACH occasion in the PSCell. TPSCell_ DU is up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in Table 8.1-1 of TS 38.213 [3].

In total, the full procedure can take around 100-300ms, depending a bit on network implementation, PSCell  SSB periodicity, whether PSCell is known and whether measurement information is available or not.

Figure 1: SCG addition procedure

### SCG DRX

UE and network power consumption can also be reduced without necessarily releasing the SCG (and adding when needed), but by configuring long DRX in the SCG. Same as for the SCG release/addition case there is also here a trade-off between power saving and latency to reactivate the SCG. In the DRX case, latency depends on the mean delay until next ON duration, for DL initiated traffic, see Figure 2. For long DRX, that delay may be significantly larger than that of SCG addition.

Figure 2: DRX Cycle

## Rel-16 discussions

SCG suspension was discussed as part of the Rel-16 WI on DCCA enhancements. Due to lack of time, it was decided not to complete the objective in Rel-16, but before that in RAN2#107bis, the following assumptions were made regarding SCG suspension:

- The UE supports network-controlled suspension of the SCG in RRC_CONNECTED.

- UE behaviour for a suspended SCG is FFS

- The UE supports at most one SCG configuration, suspended or not suspended, in Rel16.

- In RRC_CONNECTED upon addition of the SCG, the SCG can be either suspended or not suspended by configuration.

These above agreements serve as a reasonable starting point for the Rel-17 discussions.

- The agreements from Rel-16 can act as starting point for Rel-17 discussions, i.e.
a) In RRC_CONNECTED, the UE supports network controlled SCG activation and deactivation
b) The UE supports at most one SCG configuration, activated or deactivated.
c) In RRC_CONNECTED, an SCG can be added either activated or deactivated.

## Evaluation criteria

There may be different solutions being proposed in R17 for UE and network power saving for SCG activation/deactivation. Hence, it is good to define a set of evaluation criteria for the comparison of different SCG activation/deactivation solutions. Though the prime objective is to improve power saving performance, the following relevant criteria can be listed:

- Complexity. This may be subdivided into UE complexity, network complexity and specification complexity.

- Performance. This may be subdivided into power saving performance (UE/Network) and activation/deactivation latency.

- Scenarios that are supported, such as applicable deployment scenarios and applicable MR-DC architecture deployment options.

When comparing different alternatives for SCG (de)activation, the above listed criteria should be used.

Evaluation criteria for comparison of SCG (de)activation alternatives are:
a) Complexity (UE, network and specification)
b) Performance (UE/network power saving, latency)
c) Applicable scenarios (deployment scenarios, MR-DC architecture options)

## SCG (de)activation alternatives

During the Rel-16 discussions, the following general approaches for SCG deactivation were proposed:

- Long SCG DRX. This includes various solutions involving long DRX in the SCG. For example, in [2], it was proposed to configure long DRX for PSCell and switch SCells to dormant BWP. There were also proposals for activating the SCG using the MCG using RRC signalling.

  - Complexity: Low to Medium depending a bit on the realization. E.g. different DRX configurations for MCG and SCG is already supported, but activation via MCG can increase complexity.

  - Performance:

    - Power saving is negatively affected by ON duration every DRX cycle, during which also PDCCH is monitored.

    - Latency is affected by the DRX cycle (network has to wait until ON duration to activate the SCG) unless activation is done via MCG.

- Stored SCG. In it’s simplest form, stored SCG is a signaling optimization, where the UE simply stores the SCG configuration instead of releasing it during SCG release. On top of this, UE can be ordered to continue e.g. RRM and RLM of the deactivated SCG, in order to improve SCG activation. Storing and activating SCG is also related to conditional SCG addition (CPAC). Various solutions for storing SCG were proposed in [3][4][5].

  - Complexity: Low to Medium depending a bit on the realization.

  - Performance:

    - Power saving is equivalent to that of SCG release, depending a bit on the realization, e.g. whether and how RRM and/or RLM is supported.

    - Latency will depend on defined UE behavior.

- Dormant PSCell. In general this would mean to introduce dormancy for PSCell, similar to what was done in Rel-16 for dormant SCell, meaning that the UE stops monitoring PDCCH of dormant PSCell, but continues with CSI measurements, RRM measurements and beam management. Variants of this general approach were proposed in [6][7][8][9][10][11].

  - Complexity: Depends on solution selected, but can be expected to be higher than the two previous alternatives

  - Performance:

    - Power saving compared to released SCG would be negatively affected by e.g. CSI measurements and beam management.

    - Latency will depend on defined UE behavior.

In order to end up with a solution which will have a chance of being implemented by UEs and networks, it seems most important to consider a solution with moderate complexity, still providing acceptable performance improvement in terms of power saving and latency compared to legacy solutions. Especially, care must be taken not to create an overly complex solution for minimal performance gains. Further, only one solution should be specified to avoid market fragmentation.

Only one solution with low complexity should be specified for efficient SCG (de)activation.

## General aspects of deactivated SCG

Regardless of the alternative selected in section 2.3, this section lists a set of common topics for deactivated SCG that are good to discuss.

### Transmission

In any of the SCG deactivation options above, one common behavior is that the UE will not transmit/receive data via the SCG while the SCG is deactivated. Thus, it can be assumed that the SCG deactivation, regardless of which option is used, will result in the suspension of traffic for all SRBs and DRBs on the SCG. This follows the same principle as applied during MCG/SCG failure recovery.

When SCG is deactivated, the UE suspends SCG transmission for all SRBs and DRBs.

### Signalling

In general, RRC should be maintained as the protocol used to configure the UE. RRC signaling is generally considered to have slightly worse latency compared to lower layer solutions, like MAC and DCI. However, the RRC signaling delay only constitutes a small part of the total delay, as seen in the RAN4 formula in section 2.1.1. Even for known cell, the total delay may be ~100-200ms, depending on the SMTC out of which the RRC processing delay is 16ms. It needs also to be considered that the processing delay of an RRC message simply including the activation of an already stored SCG should be considerably smaller than that of and RRC message including a new SCG, as in SCG addition, making the difference in latency smaller compared to other signaling options. A common limitation of lower layer signaling is that only a very limited amount of information can be included, which limits the flexibility and possibilities for future enhancements of such a solution. In summary, we have a preference for using RRC signaling, but since the requirements on the signaling solutions will differ depending on the approach selected in section 2.3, our proposal is to decide the signaling protocol only once the overall solution has been agreed.

- Decide the signaling protocol (e.g. RRC or MAC CE) only after the overall solution is decided.

Another aspect to discuss is the inter node signaling. In general, both MN and SN will be involved in the activation and deactivation of SCG, so some form of inter node signaling will be needed for coordination. The WI description mentions deactivating the SCG during periods of low data activity. Given that MR-DC supports both MN and SN terminated DRBs, there seems to be a need for both MN and SN initiated procedures for activation and deactivation of SCG.

- Inter node procedures for both MN and SN initiated SCG (de)activation are defined.

### RLM&RRM

In R16 it was also discussed what should have been the UE behavior regarding RLM on the PSCell and RRM measurements based on the SCG measurement configuration while the SCG is deactivated.

Regardless of which solution is specified in R17, the network should at least be notified if the UE detects radio link problems with the PSCell even when the SCG is deactivated, to avoid a scenario where the network attempts to resume the operation with a deactivated SCG without success. That would require the UE to perform at least some form of S-RLM so the UE is able to detect an S-RLF while the SCG is deactivated, and the transmission of SCG Failure reports.

- UE performs some form of S-RLM and transmits SCG failure reports while the SCG is deactivated.

Regardless of which solution is specified in R17, the UE could move away from the PSCell coverage while the SCG is deactivated, and/or enter the coverage of a cell in the same frequency of the PSCell with better radio conditions. If this is not notified to the network, the network may attempt to activate/resume the deactivated SCG and fail.

Figure 3: UE mobility with deactivated PSCell

Hence, to avoid problems during the activation of the deactivated SCG, the UE shall continue performing at least some of the SCG RRM measurements and possibly transmit measurement reports (via MN as the SCG is deactivated). It could be debated whether RRM measurements associated to the SCG are enabled at all, as the intent of the feature is for the UE/network to save power. However, it is known that most of the savings comes from the absence of PDCCH monitoring. It is however not certain whether the UE would need to perform measurements according to the whole SCG measConfig (or if same requirements applied for measurements in RRC_CONNECTED would need to be applicable for deactivated SCG). For example, if only a PSCell Release and Add mechanism is supported, perhaps only PSCell would need to be measured. Hence, restrictions for further power savings regarding SCG RRM measurements can be left FFS.

- UE performs some form of SCG RRM measurements while the SCG is deactivated. FFS restrictions for further power savings.

Measurement reports could enable network decisions while the SCG is deactivated, depending on what kind of information is reported such as the release of a deactivated SCG, or/and an SCG mobility. In our view both uses cases should be supported.

- SCG release and SCG mobility are supported for a deactivated SCG.

A reasonable assumption is that while the SCG is deactivated, the UE continues to normally operate with the MCG e.g. UE continues to perform M-RLM and MCG RRM measurements. The UE could move away from the PCell coverage while the SCG is deactivated, and/or enter the coverage of a cell in the same frequency of the PCell with better radio conditions. Hence, it should be possible to perform a PCell change while the SCG is deactivated.

- MCG mobility while the SCG is deactivated is supported.

## SCell (de)activation

According to WI objective, support for SCells applies to NR CA, based on RAN1 leading mechanisms. Given the support for dormant SCell introduced in Rel-16, the main part for Rel-17 is the RAN1 driven work on temporary RS to speed up synchronization. Possible RAN2 impact can be evaluated once RAN1 has made more progress.

- Possible RAN2 impact for SCell deactivation to be evaluated once RAN1 has made more progress on temporary RS.

# Conclusion

Based on the discussion in section 2 we propose the following:

Proposal 1	The agreements from Rel-16 can act as starting point for Rel-17 discussions, i.e. a) In RRC_CONNECTED, the UE supports network controlled SCG activation and deactivation b) The UE supports at most one SCG configuration, activated or deactivated. c) In RRC_CONNECTED, an SCG can be added either activated or deactivated.

Proposal 2	Evaluation criteria for comparison of SCG (de)activation alternatives are: a) Complexity (UE, network and specification) b) Performance (UE/network power saving, latency) c) Applicable scenarios (deployment scenarios, MR-DC architecture options)

Proposal 3	Only one solution with low complexity should be specified for efficient SCG (de)activation.

Proposal 4	When SCG is deactivated, the UE suspends SCG transmission for all SRBs and DRBs.

Proposal 5	Decide the signaling protocol (e.g. RRC or MAC CE) only after the overall solution is decided.

Proposal 6	Inter node procedures for both MN and SN initiated SCG (de)activation are defined.

Proposal 7	UE performs some form of S-RLM and transmits SCG failure reports while the SCG is deactivated.

Proposal 8	UE performs some form of SCG RRM measurements while the SCG is deactivated. FFS restrictions for further power savings.

Proposal 9	SCG release and SCG mobility are supported for a deactivated SCG.

Proposal 10	MCG mobility while the SCG is deactivated is supported.

Proposal 11	Possible RAN2 impact for SCell deactivation to be evaluated once RAN1 has made more progress on temporary RS.

# References

RP-201040, Work Item on Further Multi-RAT Dual-Connectivity enhancements, Huawei

R2-194364, Further discussion on suspension of SCG, Qualcomm

R2-1914576, Practical scoping of SCG suspension operation in NR, Intel

R2-1914951, Discussion on Suspended SCG, MediaTek

R2-1915687, Open issues for suspended SCG behaviour, Ericsson

R2-1914531, Consideration on Dormant SCG State, CATT

R2-1914823, On SCG Suspension, ZTE

R2-1914944, UE behaviour for a suspended SCG, vivo

R2-1915933, SCG Suspension, Apple

R2-1916157, Clarification of suspended SCG, LG

R2-1916161, Resumption of suspended SCG, LG

3GPP TS 38.133, NR; Requirements for support of radio resource management