3GPP TSG-RAN WG2 Meeting #111	R2-2008091

# Electronic, 17 – 28 August 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 1987 | rev | - | Current version: | 15.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on re-establishment procedure (R15) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - When initiating the RRC re-establishment procedure, the spCellConfig and MCG SCells are released for the MCG if UE is not configured with conditionalReconfiguration while other MCG configuration of the source, e.g. mac-CellGroupConfig is not released. Considering that some fields in mac-CellGroupConfig are OPTIONAL M, if the target node does not support the configuration of the source node and configure the new mac-CellGroupConfig with these fields absent, UE will consider the configuration from the source node is still valid, leads to misunderstanding of the configuration. To avoid such misunderstanding, it is suggested to let UE release the whole masterCellGroup including the spCellConfig when initiating the RRC re establishment procedure. - When initating RRC re-establishment procedure, all the RBs except SRB0 are suspended while the radio bearer comfiguration is still stored at UE side. For reconfiguration with fullconfig after re-establishment, the srb-ToAddModList is mandatory present and it has been clarified in NOTE2 that this is to apply default configuration for SRB2, which means SRB1 is not included. 1> for each srb-Identity value included in the srb-ToAddModList (SRB reconfiguration): 2> apply the default SRB configuration defined in 9.2.1 for the corresponding SRB; NOTE 2: This is to get the SRBs (SRB1 and SRB2 for reconfiguration with sync and SRB2 for reconfiguration after re-establishment) to a known state from which the reconfiguration message can do further configuration. Upon receiving the RRCReconfiguration with full config and SRB1 is not included in srb-ToAddModList, the following options for SRB1 handling can be considered: - Option 1: Since UE has applied default SRB1 configuration when transmitting RRCRe-establishmentRequest, UE can continue to use the SRB1 with default configuration. - Option 2: Apply the SRB1 configuration from the source node if SRB1. Considering that the target node maynot be aware of the radio bearer configuration at the source side or may not support such configuration, option 2 is not preferable as the SRB1 may be an unknown state for the target. To prevent UE from going to option 2, it is suggested to let UE release SRB1 configuration from the source node when initiating RRC re-establishment procedure. Similar to the concern raised for UE storage of the MCG configuration, there is some OPTIONAL M fields in radioBearerConfig, if the target node does not support the configuration of the source node and configure the new radioBearerConfig with these fields absent, UE will consider the configuration from the source node is still valid, leads to misunderstanding of the configuration. With the above consideration, we suggest to let UE release the whole radioBearerConfig when initiating the RRC re establishment procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify that UE release the whole masterCellGroup including the spCellConfig when initiating the RRC re establishment procedure. - Clarify that UE release the whole radioBearerConfig when initiating the RRC re establishment procedure. Impact Analysis Impacted 5G architecture options: SA, NE-DC, NR-DC Impacted functionality: RRC Re-establishment procedure Inter-operability: No inter-oprability issue is forseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the target node does not support the configuration of the source node and configure the new radioBearerConfig or masterCellGroup with some OPTION M fields absent, UE will consider the configuration from the source node is still valid, leads to misunderstanding of the configuration. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of change

#### 5.3.7.2	Initiation

The UE initiates the procedure when one of the following conditions is met:

1>	upon detecting radio link failure of the MCG, in accordance with 5.3.10; or

1>	upon re-configuration with sync failure of the MCG, in accordance with sub-clause 5.3.5.8.3; or

1>	upon mobility from NR failure, in accordance with sub-clause 5.4.3.5; or

1>	upon integrity check failure indication from lower layers concerning SRB1 or SRB2, except if the integrity check failure is detected on the RRCReestablishment message; or

1>	upon an RRC connection reconfiguration failure, in accordance with sub-clause 5.3.5.8.2.

Upon initiation of the procedure, the UE shall:

1>	stop timer T310, if running;

1>	stop timer T304, if running;

1>	start timer T311;

1>	suspend all RBs, except SRB0;

- release radioBearerConfig, if configured;

1>	reset MAC;

1>	release the MCG SCell(s), if configured;

1>	release masterCellGroup, if configured;

1>	if MR-DC is configured:

2>	perform MR-DC release, as specified in clause 5.3.5.10;

1>	release delayBudgetReportingConfig, if configured, and stop timer T342, if running;

1>	release overheatingAssistanceConfig, if configured, and stop timer T345, if running;

1>	perform cell selection in accordance with the cell selection process as specified in TS 38.304 [20], clause 5.2.6.

End of change