3GPP TSG-RAN WG2 #111-e	R2-20xxxxx

Electronic Meeting, 17th – 28th August 2020

Agenda Item:	6.1.1

Source:	Ericsson

Title:	[AT111-e][014][NR16] RRC Misc II

Document for:	Discussion, Decision

# 1	Introduction

This document is to kick off the following email discussion:

- [AT111-e][014][NR16] RRC Misc II (Ericsson)

Scope: Treat R2-2007275, R2-2007276, R2-2007077, R2-2006915, R2-2006934 (proponents to drive)

Part 1: Decision whether to make corrections, identify agreeable parts.

Deadline: Aug 20, 0900 UTC.

Part 2: For agreeable parts, continuation to agree CRs.

Deadline: Aug 26, 0900 UTC.

# 2	Discussion

Companies are requested to add their comments for each of the treated CRs of this email discussion in the boxes below (one for each CR to be treated).

### 2.1.1	Misc corrections for on-demand SIB in connected

R2-2007275	Miscellaneous correction regarding on demand SIB in CONNECTED	Ericsson	CR	Rel-16	38.331	16.1.0	1820	-	F	5G_V2X_NRSL-Core, NR_pos-Core

| Company | Agree? (Yes or No) | Comments |
| --- | --- | --- |
| Apple | Yes | There is one additonal typo needs tob e fixed in 5.2.2.3.2, the si-periodicity needs to be posSI-periodicity. 2> else if the concerned SI message is configured in the pos-SchedulingInfoList and offsetToSI-Used is not configured: 3> create a concatented list of SI messages by appending the pos-SchedulingInfoList in posSI-SchedulingInfo in SIB1 to schedulingInfoList in si-SchedulingInfo in SIB1 3> for the concerned SI message, determine the number n which corresponds to the order of entry in the concatenated list; 3> determine the integer value x = (n – 1) × w, where w is the si-WindowLength; 3> the SI-window starts at the slot #a, where a = x mod N, in the radio frame for which SFN mod T = FLOOR(x/N), where T is the si-Periodicity of the concerned SI message and N is the number of slots in a radio frame as specified in TS 38.213 [13]; |

### 2.1.2	Redundant procedural text of on-demand SIB

R2-2007276	Redundant procedural text of on demand SIB in CONNECTED	Ericsson	CR	Rel-16	38.331	16.1.0	1821	-	F	5G_V2X_NRSL-Core, NR_pos-Core

| Company | Agree? (Yes or No) | Comments |
| --- | --- | --- |
| Apple | Yes |  |

### 2.1.3	Correction to on-demand SI acquisition in RRC_CONNECTED

R2-2007077	Corrections to on demand SI acquisition in RRC_CONNECTED	Samsung Electronics Co., Ltd	CR	Rel-16	38.331	16.1.0	1780	-	F	5G_V2X_NRSL-Core, NR_pos-Core

| Company | Agree? (Yes or No) | Comments |
| --- | --- | --- |
| Apple | No | If Ericsson CR R2-2007276 is agreed, the change proposed in R2-2007077 is not needed. |

### 2.1.4	Handling of CPC in fast MCG recovery

R2-2006934	Handling of CPC in fast MCG recovery	Intel Corporation	CR	Rel-16	38.331	16.1.0	1755	-	F	NR_Mob_enh-Core, LTE_NR_DC_CA_enh-Core

| Company | Agree? (Yes or No) | Comments |
| --- | --- | --- |
| Apple | Yes | A comment on Minor editorial issue: The sentence below needs tob e ended with a colon : 2> if the RRCReconfiguration message was received via SRB3 within DLInformationTransferMRDC; |

# Conclusion

In the previous sections we made the following observations:

Based on the discussion in the previous sections we propose the following:

# References

[1]