3GPP TSG-RAN WG2 #111-e	R2-20xxxxx

Electronic Meeting, 17th – 28th August 2020

Agenda Item:	6.1.1

Source:	Ericsson

Title:	[AT111-e][014][NR16] RRC Misc II

Document for:	Discussion, Decision

# 1	Introduction

This document is to kick off the following email discussion:

- [AT111-e][014][NR16] RRC Misc II (Ericsson)

Scope: Treat R2-2007275, R2-2007276, R2-2007077, R2-2006915, R2-2006934 (proponents to drive)

Part 1: Decision whether to make corrections, identify agreeable parts.

Deadline: Aug 20, 0900 UTC.

Part 2: For agreeable parts, continuation to agree CRs.

Deadline: Aug 26, 0900 UTC.

# 2	Discussion

Companies are requested to add their comments for each of the treated CRs of this email discussion in the boxes below (one for each CR to be treated).

### 2.1.1	Misc corrections for on-demand SIB in connected

R2-2007275	Miscellaneous correction regarding on demand SIB in CONNECTED	Ericsson	CR	Rel-16	38.331	16.1.0	1820	-	F	5G_V2X_NRSL-Core, NR_pos-Core

| Company | Agree? (Yes or No) | Comments |
| --- | --- | --- |
| Apple | Yes | There is one additonal typo needs tob e fixed in 5.2.2.3.2, the si-periodicity needs to be posSI-periodicity. 2> else if the concerned SI message is configured in the pos-SchedulingInfoList and offsetToSI-Used is not configured: 3> create a concatented list of SI messages by appending the pos-SchedulingInfoList in posSI-SchedulingInfo in SIB1 to schedulingInfoList in si-SchedulingInfo in SIB1 3> for the concerned SI message, determine the number n which corresponds to the order of entry in the concatenated list; 3> determine the integer value x = (n – 1) × w, where w is the si-WindowLength; 3> the SI-window starts at the slot #a, where a = x mod N, in the radio frame for which SFN mod T = FLOOR(x/N), where T is the si-Periodicity of the concerned SI message and N is the number of slots in a radio frame as specified in TS 38.213 [13]; |
| Nokia | Yes | Agree also with Apple comment |

### 2.1.2	Redundant procedural text of on-demand SIB

R2-2007276	Redundant procedural text of on demand SIB in CONNECTED	Ericsson	CR	Rel-16	38.331	16.1.0	1821	-	F	5G_V2X_NRSL-Core, NR_pos-Core

| Company | Agree? (Yes or No) | Comments |
| --- | --- | --- |
| Apple | Yes |  |
| Nokia | Yes |  |

### 2.1.3	Correction to on-demand SI acquisition in RRC_CONNECTED

R2-2007077	Corrections to on demand SI acquisition in RRC_CONNECTED	Samsung Electronics Co., Ltd	CR	Rel-16	38.331	16.1.0	1780	-	F	5G_V2X_NRSL-Core, NR_pos-Core

| Company | Agree? (Yes or No) | Comments |
| --- | --- | --- |
| Apple | No | If Ericsson CR R2-2007276 is agreed, the change proposed in R2-2007077 is not needed. |
| Nokia | Yes | But both this and 7276 are not needed. Only one. |

### 2.1.4	Handling of CPC in fast MCG recovery

R2-2006934	Handling of CPC in fast MCG recovery	Intel Corporation	CR	Rel-16	38.331	16.1.0	1755	-	F	NR_Mob_enh-Core, LTE_NR_DC_CA_enh-Core

| Company | Agree? (Yes or No) | Comments |
| --- | --- | --- |
| Apple | Yes | A comment on Minor editorial issue: The sentence below needs tob e ended with a colon : 2> if the RRCReconfiguration message was received via SRB3 within DLInformationTransferMRDC; |
| Nokia | No | Issue 1: Yes the procedural combination exists, but that does not mean it will ever happen. 37.340 captures "CPC configuration in HO command, ...is not supported.", and the message in DLInfoTransfer in response to MCGFailureInfo is a HO command. If anything, RRC spec could capture this explicitly, instead of proposed tabular text which only addresses a sub-case. Issue 2: As a general principle, on SRB3 37.340 captures "SN RRC Reconfiguration Complete messages are mapped to the same SRB as the message initiating the procedure", and this principle is implemented by the current RRC procedure. |
| Intel | Yes | To Nokia, it is also ok to us if in stage 3 specification, we can clarify “The RRCReconfiguration message contained in DLInformationTransferMRDC includes ReconfigurationWithSync if it is used for fast MCG failure recovery.” |

# Conclusion

In the previous sections we made the following observations:

Based on the discussion in the previous sections we propose the following:

# References

[1]