# 3GPP TSG RAN WG2 Meeting #111-e      	                                               R2-200xxxx
E-Conference, 17th August – 28th August 2020

Agenda item:	6.1.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Summary of [AT111-e][016][NR16] UE cap TRS bandwidth (Nokia)

Document for:	Discussion and Decision

# 1.	Introduction

This is a summary of below offline discussion:

TEI16

TRS bandwidth

[AT111-e][016][NR16] UE cap TRS bandwidth (Nokia)

Scope: Treat R2-2007498, R2-2007499, R2-2008089, R2-2008090 (proponents to drive)

Deadlines: Short NR UE cap

# 2. Discussions

## 2.0 Contact list of delegates

To make it easier to find the correct contact delegate in each company for potential follow-up questions, the rapporteur encourages the delegates who provide input to provide their contact information in this table:

| Company | Delegate contact |
| --- | --- |
| Nokia, Nokia Shanghai Bell | Amaanat Ali (amaanat.ali@nokia.com) |
| Huawei, HiSilicon | Yang Zhao (zhaoyang@huawei.com) |
| Ericsson | Mattias Bergström (Mattias.a.bergstrom@ericsson.com) |

## 2.1 Discussion on R2-2007498 and R2-2007499

R2-2007498	Capability signalling for limited TRS bandwidth for 10 MHz bandwidth with 15 kHz SCS	Nokia, Nokia Shanghai Bell	CR	Rel-16	38.306	16.1.0	0381	-	B	TEI16

R2-2007499	Capability signalling for limited TRS bandwidth for 10 MHz bandwidth with 15 kHz SCS	Nokia, Nokia Shanghai Bell	CR	Rel-16	38.331	16.1.0	1848	-	B	TEI16

| Company | Comments |
| --- | --- |
| Nokia, Nokia Shanghai Bell | Proponent |
| Huawei, HiSilicon | There is no big difference between the two sets of CRs, and we slightly prefer our version as this has already been discussed with other cosigners. To be more specific, we prefer the naming in our version as this looks more generic, while for choice or enumeration, we don’t have strong view and are open to hear others. |
| Ericsson | To have a capability bit saying that the UE supports "all" of something is not suitable since in case more (not yet anticipated) options are added in the future, "all" would no longer be true. Hence we think the other CRs are more suitable. |

## 2.2 Discussion on R2-2008089 and R2-2008090

The following documents are relevant for the discussion:

R2-2008089	Support of new newly defined TRS bandwidth sizes	Huawei, HiSilicon, Ericsson, Vodafone	CR	Rel-16	38.331	16.1.0	1910	1	F	TEI16	R2-2007803	Late

R2-2008090	Support of new newly defined TRS bandwidth sizes	Huawei, HiSilicon, Ericsson, Vodafone	CR	Rel-16	38.306	16.1.0	0391	1	F	TEI16	R2-2007804	Late

| Company | Comments |
| --- | --- |
| Nokia, Nokia Shanghai Bell | Intention looks fine. There are few differences e.g. wording compared to Nokia CR and also choice of how the fields are set in the xDD and FRx differentiation. Then the choice of enumeration is also aligned to each proponent’s 306 changes. We would prefer our version or request to update as per our CR and we can co-sign. |
| Huawei, HiSilicon | Proponents and we are happy to add co-signers, in addition to what we explained above, we actually see for 306 our version is more accurate on when to apply this capability. Surely open to hear any specific wording improvement. |
| Ericsson | Supportive. |

# 3. Conclusion

Summary to be provided at end of the discussion.

# References

TEI16

TRS bandwidth

[AT111-e][016][NR16] UE cap TRS bandwidth (Nokia)

Scope: Treat R2-2007498, R2-2007499, R2-2008089, R2-2008090 (proponents to drive)

Deadlines: Short NR UE cap

R2-2007498	Capability signalling for limited TRS bandwidth for 10 MHz bandwidth with 15 kHz SCS	Nokia, Nokia Shanghai Bell	CR	Rel-16	38.306	16.1.0	0381	-	B	TEI16

R2-2007499	Capability signalling for limited TRS bandwidth for 10 MHz bandwidth with 15 kHz SCS	Nokia, Nokia Shanghai Bell	CR	Rel-16	38.331	16.1.0	1848	-	B	TEI16

4 moved from 6.16

R2-2007803	Support of flexible TRS bandwidth sizes	Huawei, HiSilicon	CR	Rel-16	38.331	16.1.0	1910	-	F	TEI16	Revised

R2-2008089	Support of new newly defined TRS bandwidth sizes	Huawei, HiSilicon, Ericsson, Vodafone	CR	Rel-16	38.331	16.1.0	1910	1	F	TEI16	R2-2007803	Late

R2-2007804	Support of flexible TRS bandwidth sizes	Huawei, HiSilicon	CR	Rel-16	38.306	16.1.0	0391	-	F	TEI16	Revised

R2-2008090	Support of new newly defined TRS bandwidth sizes	Huawei, HiSilicon, Ericsson, Vodafone	CR	Rel-16	38.306	16.1.0	0391	1	F	TEI16	R2-2007804	Late