# 3GPP TSG RAN WG2 Meeting #111-e      	                                               R2-200xxxx
E-Conference, August 17th to August 28th 2020

Agenda item:	6.1.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Summary of [AT111-e] [020] [NR16] UE cap RF FR2 (Nokia)

Document for:	Discussion and Decision

# 1.	Introduction

RF FR2

[AT111-e][020][NR16] UE cap RF FR2 (Nokia)

Scope: Treat R2-2007403, R2-2007082, R2-2007083, R2-2007380, R2-2007381

Deadlines: Short UE Caps

Dl only spectrum, moved from 6.15

R2-2007403	DL-only spectrum	Ericsson, Apple	discussion	Rel-16	NR_RF_FR2_req_enh-Core

R2-2007082	Introduction on frequency separation class for DL-only FR2 spectrum	Apple, Ericsson	CR	Rel-16	38.306	16.1.0	0371	-	F	TEI16, NR_RF_FR2_req_enh

R2-2007083	Introduction on frequency separation class for DL-only FR2 spectrum	Apple, Ericsson	CR	Rel-16	38.331	16.1.0	1784	-	F	TEI16, NR_RF_FR2_req_enh

Suspend IBE requirements, moved from 6.15

R2-2007380	Uplink power boosting via suspended IBE requirements	Nokia, Nokia Shanghai Bell	CR	Rel-16	38.331	16.1.0	1840	-	B	NR_RF_FR2_req_enh

R2-2007381	Uplink power boosting via suspended IBE requirements	Nokia, Nokia Shanghai Bell	CR	Rel-16	38.306	16.1.0	0379	-	B	NR_RF_FR2_req_enh

# 2. Discussions

Contact list of delegates

To make it easier to find the correct contact delegate in each company for potential follow-up questions, the rapporteur encourages the delegates who provide input to provide their contact information in this table:

| Company | Delegate contact |
| --- | --- |
| Nokia, Nokia Shanghai Bell | Amaanat Ali (amaanat.ali@nokia.com) |
| Ericsson | Mattias Bergström (mattias.a.bergstrom@ericsson.com |
| Qualcomm Incorporated | Masato Kitazoe (mkitazoe [at] qti.qualcomm.com |
| Huawei, HiSilicon | Yiru Kuang (kuangyiru@huawei.com) |
| Intel | Youn Heo (young.hyoung.heo@intel.com) |
| Futurewei | Hao Bi (hao.bi@futuewei.com) |
| CATT | Da Wang (wangda@catt.cn) |
| OPPO | Qianxi Lu (qianxi.lu@oppo.com) |

2.0 Discussion on R2-2007403

R2-2007403	DL-only spectrum	Ericsson, Apple	discussion	Rel-16	NR_RF_FR2_req_enh-Core

| Company | Feedback |
| --- | --- |
| Ericsson | Background to the paper: The intention of the paper is to ensure that companies in RAN2 have a common understanding of what RAN4 is requesting. If they agree to what is written in this contribution, we assume the following CRs will be agreeable too. |
| Nokia | Agree with the intention |

2.1 Discussion on CRs R2-2007082 and R2-2007083

The following documents are relevant for the discussion:

Dl only spectrum, moved from 6.15

R2-2007082	Introduction on frequency separation class for DL-only FR2 spectrum	Apple, Ericsson	CR	Rel-16	38.306	16.1.0	0371	-	F	TEI16, NR_RF_FR2_req_enh

R2-2007083	Introduction on frequency separation class for DL-only FR2 spectrum	Apple, Ericsson	CR	Rel-16	38.331	16.1.0	1784	-	F	TEI16, NR_RF_FR2_req_enh

| Company | Feedback |
| --- | --- |
| Nokia | CRs intent is fine for us. Good proposals from Intel and Huawei which help clarifying the purpose of the additional capability. |
| Ericsson | Agree @Intel: It is also our understanding that if the UE indicates intraBandFreqSeparationDL-Only-r16 the UE shall also indicate intraBandFreqSeparationDL. We actually think it is already clear from the current CR since it says that the DL-only field extends the legacy field. But if companies want to include such a statement, that would be fine. |
| Qualcomm Incorporated | Agree to the content. The title of the CR should be corrected to cover paired bands. |
| Huawei, HiSilicon | Generally fine with the CRs, please see a wording change suggested as below, since it is an optional feature, if the UE does not report the field intraBandFreqSeparationDL-Only-r16, it means UE does not support frequency separation class for DL only frequency spectrum, instead of applying any default value. “Indicates whether the UE supports frequency separation class of DL only extension. If present, the field extends the maximum frequency separation between the lower edge of lowest CC and the upper edge of highest CC in a frequency band that the UE supports according to intraBandFreqSeparationDL.The frequency range extension is either above or below the frequency range indicated by intraBandFreqSeparationDL and extends it in contiguous manner with no frequency gap, and the network may configure contiguous or non-contiguous downlink serving cells in that extended range.” |
| Intel | Agree. One question for clarification is whether the UE should indicate intraBandFreqSeparationDL if the UE supports intraBandFreqSeparationDL-Only-r16. If yes, it might be good to explicitly mention. |
| Futurewei | Would like to have some clarification to understand the sentence “The UE sets the same value in the FeatureSetDownlink of each band entry within a band” – would there be multiple band entries within a band? My understanding is that a band entry is used to point to a band in a band combination. Is the intention of the sentence – - All bands in a band combination use the same value, i.e., “The UE sets the same value in the FeatureSetDownlink of each band entry within a band combination”; or - The same value is used for a band in all band combinations that involve this band? |
| CATT | We are generally fine with the CRs and agree the suggestion changes made by Huawei. |
| OPPO | Agree. On the other hand, we wonder if there is a need to highlight the network configuration flexibility here “the network may configure contiguous or non-contiguous downlink serving cells in that extended range”, although we share the same understanding of (non)contiguous feasibility. In other words, we anyway cannot exhaust all feasible configuration options by network. For the issue raised by Futurewei, our understanding is it is for the band entries for the same band in a BC entry, i.e., intra-band non-contiguous CA scenario. And we wonder if the following text “The sum of intraBandFreqSeparationDL and intraBandFreqSeparationDL-Only shall not exceed 2400 MHz. If the UE sets this field, the sum of intraBandFreqSeparationDL and intraBandFreqSeparationDL-Only shall be larger than 1400 MHz.” needs to be further clarified on whether we meant for intraBandFreqSeparationDL with and/or without suffix |

2.2 Discussion on CRs R2-2007380 and R2-2007381

The following documents are relevant for the discussion:

Suspend IBE requirements, moved from 6.15

R2-2007380	Uplink power boosting via suspended IBE requirements	Nokia, Nokia Shanghai Bell	CR	Rel-16	38.331	16.1.0	1840	-	B	NR_RF_FR2_req_enh

R2-2007381	Uplink power boosting via suspended IBE requirements	Nokia, Nokia Shanghai Bell	CR	Rel-16	38.306	16.1.0	0379	-	B	NR_RF_FR2_req_enh

| Company | Comments |
| --- | --- |
| Nokia, Nokia Shanghai Bell | Proponent |
| Ericsson | Agree |
| Qualcomm Incorporated | We should wait for RAN4's feature list (FG 8-7 currently FFS) and RRC parameter list. We prefer per band UE capability signalling, and propose the same in RAN4. General approach to have network configuration and UE capability looks fine. |
| Huawei, HiSilicon | Based on the feedback from our RAN4 colleague, the RAN4 work on this topic is not finished, suggest to postpone and wait for RAN4 progress. |
| Intel | Agree to wait until RAN4 conclude it. |
| Futurewei | Share the same view that we can wait for RAN4 conclusion. |
| CATT | Same view that we should wait for RAN4. |
| OPPO | Share the same view that we can wait for RAN4 conclusion. |

3. Conclusion

Summary to be provided at end of the discussion.

References

RF FR2

[AT111-e][020][NR16] UE cap RF FR2 (Nokia)

Scope: Treat R2-2007403, R2-2007082, R2-2007083, R2-2007380, R2-2007381

Deadlines: Short UE Caps

Dl only spectrum, moved from 6.15

R2-2007403	DL-only spectrum	Ericsson, Apple	discussion	Rel-16	NR_RF_FR2_req_enh-Core

R2-2007082	Introduction on frequency separation class for DL-only FR2 spectrum	Apple, Ericsson	CR	Rel-16	38.306	16.1.0	0371	-	F	TEI16, NR_RF_FR2_req_enh

R2-2007083	Introduction on frequency separation class for DL-only FR2 spectrum	Apple, Ericsson	CR	Rel-16	38.331	16.1.0	1784	-	F	TEI16, NR_RF_FR2_req_enh

Suspend IBE requirements, moved from 6.15

R2-2007380	Uplink power boosting via suspended IBE requirements	Nokia, Nokia Shanghai Bell	CR	Rel-16	38.331	16.1.0	1840	-	B	NR_RF_FR2_req_enh

R2-2007381	Uplink power boosting via suspended IBE requirements	Nokia, Nokia Shanghai Bell	CR	Rel-16	38.306	16.1.0	0379	-	B	NR_RF_FR2_req_enh

MPE, copied here from 6.15 only for reference – Treated separately in AI 6.15

R2-2008096	Implementing MPE enhancements	Ericsson	CR	Rel-16	38.306	16.1.0	0322	1	B	NR_RF_FR2_req_enh	R2-2004939 	Late