# 3GPP TSG RAN WG2 Meeting #111-e      	                                               R2-200xxxx
E-Conference, August 17th – August 28th 2020

Agenda item:	5.4.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Summary of [AT111-e][042][NR15] LTE Other (Nokia)

Document for:	Discussion and Decision

# 1.	Introduction

This is a summary of below offline discussion:

### 5.4.2	LTE changes related to NR

LTE Other

[AT111-e][042][NR15] LTE Other (Nokia)

Scope: Treat R2-2006997, R2-2006998, R2-2007350, R2-2007351, R2-2008040, R2-2008041 (proponents to drive)

Part 1: Decision whether to make corrections, identify agreeable parts. Identify Controversial issues for on-line treatment (if any).

Deadline: Aug 20, 0900 UTC.

Part 2: For agreeable parts, continuation to agree CRs.

Deadline: Aug 26, 0900 UTC.

R2-2006997	Correction on the Presence Condition for drb-ToAddModList	CATT	CR	Rel-15	36.331	15.10.0	4363	-	F	NR_newRAT-Core

R2-2006998	Correction on the Presence Condition for drb-ToAddModList	CATT	CR	Rel-16	36.331	16.1.1	4364	-	F	NR_newRAT-Core

Moved from 5.4.2

R2-2007350	Clarification about UL 256QAM	Nokia, Nokia Shanghai Bell	CR	Rel-15	36.331	15.10.0	4382	-	F	NR_newRAT-Core

R2-2007351	Clarification about UL 256QAM	Nokia, Nokia Shanghai Bell	CR	Rel-16	36.331	16.1.0	4383	-	A	NR_newRAT-Core

Moved from 7.x

R2-2008040	Correction for Qrxlevmin description in SIB24     Qualcomm Incorporated   CR       Rel-15 36.331 15.10.0   4420    -           F          LTE_eMob-Core

R2-2008041	Correction for Qrxlevmin description in SIB24     Qualcomm Incorporated   CR       Rel-16 36.331 16.1.1  4421    -   A          LTE_eMob-Core

# 2. Discussions

## 2.1 Discussion on CRs R2-2006997 and R2-2006998

R2-2006997	Correction on the Presence Condition for drb-ToAddModList	CATT	CR	Rel-15	36.331	15.10.0	4363	-	F	NR_newRAT-Core

R2-2006998	Correction on the Presence Condition for drb-ToAddModList	CATT	CR	Rel-16	36.331	16.1.1	4364	-	F	NR_newRAT-Core

| Company | Feedback |
| --- | --- |
| Nokia, Nokia Shanghai Bell | The initial feedback from our side is that the use case is not clear. For example, what was the scenario in which the issue is observed and what is particularly broken in the spec which you would like to fix? |

## 2.2 Discussion on CRs R2-2007350 and R2-2007351

The following documents are relevant for the discussion:

R2-2007350	Clarification about UL 256QAM	Nokia, Nokia Shanghai Bell	CR	Rel-15	36.331	15.10.0	4382	-	F	NR_newRAT-Core

R2-2007351	Clarification about UL 256QAM	Nokia, Nokia Shanghai Bell	CR	Rel-16	36.331	16.1.0	4383	-	A	NR_newRAT-Core

| Company | Comments |
| --- | --- |
| Nokia, Nokia Shanghai Bell | [Proponent] This is a real issue to fix as this caused some potential concern during IODT discussion. |

## 2.3 Discussion on CRs R2-2008040 and R2-2008041

The following documents are relevant for the discussion:

R2-2008040	Correction for Qrxlevmin description in SIB24     Qualcomm Incorporated   CR       Rel-15 36.331 15.10.0   4420    -           F          LTE_eMob-Core

R2-2008041	Correction for Qrxlevmin description in SIB24     Qualcomm Incorporated   CR       Rel-16 36.331 16.1.1  4421    -   A          LTE_eMob-Core

| Company | Comments |
| --- | --- |
| Nokia, Nokia Shanghai Bell | Looks correct. Error was caused because in NR this *2 was defined in IE description not in field description and LTE does not use IE. But in order to improve we would introduce IE and this behaviour similarly as in NR. |

# 3. Conclusion

Summary to be provided at end of the discussion.

# References

### 5.4.2	LTE changes related to NR

LTE Other

[AT111-e][042][NR15] LTE Other (Nokia)

Scope: Treat R2-2006997, R2-2006998, R2-2007350, R2-2007351, R2-2008040, R2-2008041 (proponents to drive)

Part 1: Decision whether to make corrections, identify agreeable parts. Identify Controversial issues for on-line treatment (if any).

Deadline: Aug 20, 0900 UTC.

Part 2: For agreeable parts, continuation to agree CRs.

Deadline: Aug 26, 0900 UTC.

R2-2006997	Correction on the Presence Condition for drb-ToAddModList	CATT	CR	Rel-15	36.331	15.10.0	4363	-	F	NR_newRAT-Core

R2-2006998	Correction on the Presence Condition for drb-ToAddModList	CATT	CR	Rel-16	36.331	16.1.1	4364	-	F	NR_newRAT-Core

Moved from 5.4.2

R2-2007350	Clarification about UL 256QAM	Nokia, Nokia Shanghai Bell	CR	Rel-15	36.331	15.10.0	4382	-	F	NR_newRAT-Core

R2-2007351	Clarification about UL 256QAM	Nokia, Nokia Shanghai Bell	CR	Rel-16	36.331	16.1.0	4383	-	A	NR_newRAT-Core

Moved from 7.x

R2-2008040	Correction for Qrxlevmin description in SIB24     Qualcomm Incorporated   CR       Rel-15 36.331 15.10.0   4420    -           F          LTE_eMob-Core

R2-2008041	Correction for Qrxlevmin description in SIB24     Qualcomm Incorporated   CR       Rel-16 36.331 16.1.1  4421    -   A          LTE_eMob-Core