3GPP TSG-RAN WG2 #111-e	draft_R2-200xxxx

Online, 17 – 28 August 2020

Agenda Item:	9.1.3

Source:	Ericsson

Title:	[AT111-e][309][NBIOT/eMTC R17] Carrier selection (Ericsson)

Document for:	Discussion, Decision

# 1	Introduction

This document is to kick off the following email discussion:

- [AT111-e][309][NBIOT/eMTC R17] Carrier selection (Ericsson)

Status:

Scope: To clarify the scope of this objective in terms of what could be enhanced.

Intended outcome: Report in R2-2008311

Deadline: Wednesday 26 1100 UTC.

# 2	Submitted Documents

R2-2006832	NB-IoT carrier selection and configuration based on coverage level	Ericsson	discussion	Rel-17

R2-2006835	Enhancements on multi carrier configuration and selection	ZTE Corporation, Sanechips	discussion	Rel-17	NB_IOTenh4_LTE_eMTC6-Core

R2-2007343	Use cases and scenarios of carrier specific configuration	Huawei, HiSilicon	discussion	Rel-17	NB_IOTenh4_LTE_eMTC6-Core

R2-2007354	Analysis on carrier selection options	Nokia, Nokia Shanghai Bell	discussion	Rel-17

R2-2007570	Support for NB-IoT carrier selection based on the coverage level	Qualcomm Incorporated	discussion	Rel-17	NB_IOTenh4_LTE_eMTC6-Core

R2-2007957	Carrier selection enhancement	Shanghai Chen Si Electronics	discussion	Rel-17	NB_IOTenh4_LTE_eMTC6-Core

# 3	Discussion

## 3.1 Scope of WID

Based upon submitted documents, the enhancements desired can be categorized in below three different areas

- Paging carrier selection Improvements

- UL NPRACH Carrier Selection Improvements

- Service base carrier selection Improvements

Companies are requested to provide their comments and percentage allocation; so it may help to prioritize or downselect. Where would companies like to focus and prioritize?

| Company | Paging | NPRACH | Service Based | Any Other |
| --- | --- | --- | --- | --- |
| Ericsson | Yes. 70% We are fine to enhance Paging carrier selection. However, the solution should be reasonable and implementable; i.e not big impact on the NW and UE power consumption. | Yes. 30% We are fine to discuss any enhancements that companies think could be desired. | No. We think Paging and NPRACH carrier selection should be sufficient and may address service-based selection. |  |

## 3.2 Scope of Paging Improvements

Further for paging carrier selection the below parameters have been proposed to study/discuss

- CE Level Rmax (Latency)

- Carrier Specific DRX

- WUS

- GWUS

Companies are requested to provide their prioritization; in which particlualr parameter they would like to focus.

| Company | CE Level Rmax | DRX | WUS | GWUS |
| --- | --- | --- | --- | --- |
| Ericsson | Yes. 100%. We would like to focus paging improvements based upon CE level. | No. Having too many parameters for paging carrier selection improvement may risk or complicate so we would like to have only one focus area. | No. Having too many parameters for paging carrier selection improvement may risk or complicate so we would like to have only one focus area | No. Having too many parameters for paging carrier selection improvement may risk or complicate so we would like to have only one focus area |

## 3.3 Other

Any other rcomments

Companies are requested to provide their view:

| Company | Comments |
| --- | --- |

# 4	Conclusion

Based on the discussion in the previous sections we propose the following proposal as:

# 5 	References

- R2-201306, Additional enhancements for NB-IoT and LTE-MTC, RAN#88e, Reno, June 2020.