3GPP TSG-RAN WG2 #111-e	draft_R2-200xxxx

Online, 17 – 28 August 2020

Agenda Item:	9.1.3

Source:	Ericsson

Title:	[AT111-e][309][NBIOT/eMTC R17] Carrier selection (Ericsson)

Document for:	Discussion, Decision

1	Introduction

This document is to kick off the following email discussion:

- [AT111-e][309][NBIOT/eMTC R17] Carrier selection (Ericsson)

Status:

Scope: To clarify the scope of this objective in terms of what could be enhanced.

Intended outcome: Report in R2-2008311

Deadline: Wednesday 26 1100 UTC.

2	Submitted Documents

R2-2006832	NB-IoT carrier selection and configuration based on coverage level	Ericsson	discussion	Rel-17

R2-2006835	Enhancements on multi carrier configuration and selection	ZTE Corporation, Sanechips	discussion	Rel-17	NB_IOTenh4_LTE_eMTC6-Core

R2-2007343	Use cases and scenarios of carrier specific configuration	Huawei, HiSilicon	discussion	Rel-17	NB_IOTenh4_LTE_eMTC6-Core

R2-2007354	Analysis on carrier selection options	Nokia, Nokia Shanghai Bell	discussion	Rel-17

R2-2007570	Support for NB-IoT carrier selection based on the coverage level	Qualcomm Incorporated	discussion	Rel-17	NB_IOTenh4_LTE_eMTC6-Core

R2-2007957	Carrier selection enhancement	Shanghai Chen Si Electronics	discussion	Rel-17	NB_IOTenh4_LTE_eMTC6-Core

3	Discussion

3.1 Scope of WID

Based upon submitted documents, the enhancements desired can be categorized in below three different areas

- Paging carrier selection Improvements

- UL NPRACH Carrier Selection Improvements

- Service base carrier selection Improvements

Companies are requested to provide their comments and percentage allocation; so it may help to prioritize or downselect. Where would companies like to focus and prioritize?

| Company | Paging | NPRACH | Service Based | Any Other |
| --- | --- | --- | --- | --- |
| Ericsson | Yes. 70% We are fine to enhance Paging carrier selection. However, the solution should be reasonable and implementable; i.e not big impact on the NW and UE power consumption. | Yes. 30% We are fine to discuss any enhancements that companies think could be desired. | No. We think Paging and NPRACH carrier selection should be sufficient and may address service-based selection. |  |
| Huawei, HiSilicon | Yes. According to the submitted documents, it is clear that it is beneficial to have carrier specific paging configuration and assign UEs on specific carriers based on latency requirement, coverage, etc. | TBD. We are open to discuss possible use case and scenario. But we do not see clear benefit to do further optimization for NPRACH as it is already possible to have very flexible NPRACH configuration among carriers. | - We think “service” is a high level concept which is very difficult to use in RAN side. Thus, we should focus on service requirement (latency requirement, coverage, etc.) instead of service concept. Service requirement can be covered by the first two columns. |  |
| ZTE | Yes, we agree that paging improvements are the main objective that needs to be done. What need to be studied for this objective can be found in our comments for next question. | No strong view. As in our view, NPRACH carriers and resources can already be configured for different CELs, which seems enough. If some further improvement points can be proposed, we are fine to discuss them. | We can see benefit and feasibility of service based carrier selection scheme for some known service types, e.g., mobility/stationary, paging probability, enhanced coverage restriction. We’d better to do some research on them. | After introduction of CEL-based carrier selection, we suggest to also study what’s the coordination between DL carrier for paging and UL carrier for paging response and coordination between UL carrier for preamble transmission and DL carrier for RAR reception. |

3.2 Scope of Paging Improvements

Further for paging carrier selection the below parameters have been proposed to study/discuss

- CE Level Rmax (Latency)

- Carrier Specific DRX

- WUS

- GWUS

Companies are requested to provide their prioritization; in which particlualr parameter they would like to focus.

| Company | CE Level Rmax | DRX | WUS | GWUS |
| --- | --- | --- | --- | --- |
| Ericsson | Yes. 100%. We would like to focus paging improvements based upon CE level. | No. Having too many parameters for paging carrier selection improvement may risk or complicate so we would like to have only one focus area. | No. Having too many parameters for paging carrier selection improvement may risk or complicate so we would like to have only one focus area | No. Having too many parameters for paging carrier selection improvement may risk or complicate so we would like to have only one focus area |
| Huawei, HiSilicon | Yes It is beneficial on paging latency for the UEs in normal coverage. Note that if carrier specific Rmax is supported, we may need to have carrier specific nB also so that the density of paging occasions on the paging carrier with smaller Rmax can be higher. | Yes Combining with carrier specific Rmax, it is possible to support both short paging latency and extreme coverage enhancement very well at the same time in a cell (i.e. the UEs with short DRX cycle can be assigned to the paging carrier with smaller Rmax). | Yes. Only focus on enabling/disabling WUS is always enabled or disabled on all paging carriers. However, not all UEs benefit from WUS (e.g. UE in very good coverage or UE always paged). | No GWUS can already be enabled on a carrier basis and paging based group selection already take into account some service aspect. |
| ZTE | Yes. CEL-based carrier configuration and carrier selection should be a priority. Accordingly, for carriers configured for different CELs, some PDCCH parameters, e.g., nB, repetitionNum, DRX cycle can be configured differently for them. The carrier selection formula should also be changed accordingly. Moreover, in order try to avoid paging failure, the scenario of CEL change and necessity of explicitly reporting CEL from UE to network or UE fallback to legacy paging carrier selection still need to be studied. | No or depends. In our view, different defaultDRXCycle can be configured for different carrier groups with the benefit of making full use of the small UE specific DRX cycle and achieving less paging latency. But the pre-condition of this is that a certain carrier group is configured for a certain CEL. In other word, DRX cycle should only be part of CEL-based carrier configuration. For the carriers that are configured for a certain CEL, the DRX cycle for these carriers should be same. If we allow purely per carrier DRX cycle configuration, we may need to study whether MME needs to aware such configuration and how and what’s impact on MME paging strategy. That’s undesired. Moreover, it may be unreasonable that for a carrier group configured for a certain CEL (bad CEL), some carriers in this group are with small DRX cycle while other carriers with large DRX cycle. | No. we are not crystal clear the motivation for further enhancement on WUS. We think WUS can be beneficial on power saving for all the UE, no matter in good coverage or bad coverage. | No. R16 GWUS already can be configured per carrier. That’s enough. No other motivation for further enhancement. |

3.3 Other

Any other rcomments

Companies are requested to provide their view:

| Company | Comments |
| --- | --- |

4	Conclusion

Based on the discussion in the previous sections we propose the following proposal as:

5 	References

- R2-201306, Additional enhancements for NB-IoT and LTE-MTC, RAN#88e, Reno, June 2020.