3GPP TSG-RAN WG2 Meeting #112-e	R2-2010405

# Electronic Meeting, 2nd – 13th Nov 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 0982 | rev |  | Current version: | 16.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on DELTA_PREAMBLE for 2-step RA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_2step_RACH-Core |  |  |  |  |  | Date: |  |  | 2020-11-2 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In clause 7.3 of TS 38.321, DELTA_PREAMBLE preamble format based power offset values has been defined. For short preamble formats, the DELTA_PREAMBLE value is calculated based on the each preamble format and sub-carrier spacing configured by network as shown follows. Table 7.3-2: DELTA_PREAMBLE values for short preamble formats. Preamble Format; DELTA_PREAMBLE values (dB). A1; 8 + 3 × μ. A2; 5 + 3 × μ. A3; 3 + 3 × μ. B1; 8 + 3 × μ. B2; 5 + 3 × μ. B3; 3 + 3 × μ. B4; 3 × μ. C0; 11 + 3 × μ. C2; 5 + 3 × μ. where μ is the sub-carrier spacing configuration determined by msg1-SubcarrierSpacing and Table 4.2-1 in TS 38.211 [8], and the preamble formats are given by prach-ConfigurationIndex and Tables 6.3.3.2-2 and 6.3.3.2-3 in TS 38.211 [8].. In 4-step RA, the short preamble format is determined based on prach-ConfigurationIndex while the sub-carrier spacing is dertermined by msg1-SubcarrierSpacing. However in 2-step RA, two new parameters have been introduced for separate RO between 2-step RA type and 4-step RA type, which are msgA-SubcarrierSpacing and msgA-prach-ConfigurationIndex as shown bellows. RACH-ConfigCommonTwoStepRA-r16 ::= SEQUENCE { rach-ConfigGenericTwoStepRA-r16 RACH-ConfigGenericTwoStepRA-r16, [OMIT] msgA-SubcarrierSpacing-r16 SubcarrierSpacing OPTIONAL, -- Cond 2StepOnlyL139 RACH-ConfigGenericTwoStepRA-r16 ::= SEQUENCE { msgA-PRACH-ConfigurationIndex-r16 INTEGER (0..262) OPTIONAL, -- Cond 2StepOnly. In clause 6.3.3 of TS 38.211, the msgA-prach-ConfigurationIndex is used to determine the PRACH configuration for 2-step RA type as shown below. Based on the Tables 6.3.3.2-2 to 6.3.3.2-4, a preamble format is associated for each RPACH configuration. Random access preambles can only be transmitted in the time resources obtained from Tables 6.3.3.2-2 to 6.3.3.2-4 and depends on FR1 or FR2 and the spectrum type as defined in [8, TS38.104]. The PRACH configuration index in Tables 6.3.3.2-2 to 6.3.3.2-4 is - for Table 6.3.3.2-3 given by the higher-layer parameter prach-ConfigurationIndexNew if configured, otherwise by the higher-layer parameter prach-ConfigurationIndex, or by msgA-prach-ConfigurationIndex and msgA-prach-ConfigurationIndexNew if configured; and - for Tables 6.3.3.2-2 and 6.3.3.2-4 given by the higher-layer parameter prach-ConfigurationIndex, or by msgA-prach-ConfigurationIndex and msgA-prach-ConfigurationIndexNew if configured.. It is more reasonable to use these new parameters (i.e. msgA-SubcarrierSpacing and msgA-prach-ConfigurationIndex) to determine DELTA_PREAMBLE for 2-step RA configured with separate RO due to the fact that both msg1-SubcarrierSpacing and prach-ConfigurationIndex are not configured for 2-step RA any more. |
| Summary of change: |  | update the descption in clause 7.3 of TS 38.321 to determine DELTA_PREAMBLE for 2-step RA by taking the msgA-SubcarrierSpacing and msgA-prach-ConfigurationIndex into account. Impact analysis Impacted functionality RA Inter-operability: If the network is implemented according to the CR and the UE is not, or vice-versa, there will not be inter-operability problems. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | DELTA_PREAMBLE is incomplete for 2-step RA. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

7.3	DELTA_PREAMBLE values

The DELTA_PREAMBLE preamble format based power offset values are presented in Tables 7.3-1 and 7.3-2.

Table 7.3-1: DELTA_PREAMBLE values for long preamble formats.

| Preamble Format | DELTA_PREAMBLE values |
| --- | --- |
| 0 | 0 dB |
| 1 | -3 dB |
| 2 | -6 dB |
| 3 | 0 dB |

Table 7.3-2: DELTA_PREAMBLE values for short preamble formats.

| Preamble Format | DELTA_PREAMBLE values (dB) |
| --- | --- |
| A1 | 8 + 3 × μ |
| A2 | 5 + 3 × μ |
| A3 | 3 + 3 × μ |
| B1 | 8 + 3 × μ |
| B2 | 5 + 3 × μ |
| B3 | 3 + 3 × μ |
| B4 | 3 × μ |
| C0 | 11 + 3 × μ |
| C2 | 5 + 3 × μ |

where μ is the sub-carrier spacing configuration determined by msg1-SubcarrierSpacing or msgA-SubcarrierSpacing and Table 4.2-1 in TS 38.211 [8], and the preamble formats are given by prach-ConfigurationIndex or msgA-prach-ConfigurationIndex and Tables 6.3.3.2-2 and 6.3.3.2-3 in TS 38.211 [8].

End OF CHANGE