3GPP TSG-RAN WG2 Meeting #112 Electronic	R2-2011002

Online, 2– 13 November 2020

Agenda item:	6.2.5

Source:	Nokia (Summary Rapporteur)

Title:	Summary of IAB UE capabilities under AI 6.2.5

WID/SID:	NR_IAB - Release 16

Document for:	Discussion and Decision

# 1	Introduction

Based on the LS in R2-200844 RAN4 agreed:

- Power class is not applicable to the IAB-MT

- IAB-MT can ignore the advertised NS values

This contribution is to provide a summary of TDocs submitted for IAB UE capabilities under 6.2.5 which address the agreement and include [1-5].

# 2	Summary

## 2.1 CATT proposal to feedback RAN4 and remove multipleNS-And-Pmax-IAB-r16‎ in both 38.331 and 38.306

The discussion paper in [1] proposes the following:

Proposal 1: Suggest RAN2 to feedback RAN4 that from RAN2 perspective, there is no any impact to the RAN2 design/signalling if Feature 2-8 (Power ‎class) is not applicable.‎

Proposal 2: Suggest RAN2 to feedback RAN4 that from RAN2 perspective, it’s feasible that IAB-MT doesn’t support the NS signalling and P-max.‎

Proposal 3: Suggest RAN2 to delete the parameter multipleNS-And-Pmax-IAB-r16‎ in both 38.331 and 38.306 and ‎wait for RAN4 decision on how to define the Feature 2-12 (Multiple NS/P-max).‎

Companies are invited to provide their views whether they agree with the proposals.

Q1: Do you agree with the proposals in [1]? If not, please provide comments.

| Company | Preference (Y/N) | Detailed Comments |
| --- | --- | --- |

## 2.2 Nokia CRs to remove multipleNS-And-Pmax-IAB-r16‎ in both 38.331 and 38.306

The CR in [2] and [3] proposes changes to TS 38.306 and TS 38.331 as follows:

Changes of the CR to TS38.331 contain:

- Adding reference to TS37.174 on IAB radio transmission and reception requirements

- In SIB1,SIB2, SIB4 acquisition procedure, clarification to apply maximum output power to IAB-MT, according to specified IAB requirements in TS37.174

- In p-max field description, clarification the field is ignored for IAB-MT

- In nr-NS-PmaxList description, clarification the field is ignored for IAB-MT

- In MeasandMobParameters UE capability, multipleNS-And-Pmax-IAB capability is dummified

Changes of the CR to TS38.306 contain:

- multipleNS-And-Pmax-IAB capability removal, as no requirements exist to support NS values by IAB

The changes in TS38.331 CR intend to clarify IAB radio transmission and reception requirements are defined in a separate RAN4 TS, and regular UE procedures do not apply for IAB-MT, in context of p-max and multiple nr-NS use.

Companies are invited to provide their views whether they agree with the proposed changes.

Q2: Do you agree with the changes proposed in [2]? If not, please provide comments.

| Company | Preference (Y/N) | Detailed Comments |
| --- | --- | --- |

Since, the multiple NS are not defined for IAB-MT in RAN4 specifications, the changes in TS38.306 CR are to remove the capability that was previously agreed as a placeholder for potentially defined requirements.

Companies are invited to provide their views whether they agree with the proposed changes.

Q3: Do you agree with the changes proposed in [3]? If not, please provide comments.

| Company | Preference (Y/N) | Detailed Comments |
| --- | --- | --- |

## 2.3 Huawei CRs to remove multipleNS applicability during  SIB1 acquisition in 38.331 and add clarification on power class inapplicability for IAB-MT

The CR in [4] and [5] proposes changes to TS 38.306 and TS 38.331 as follows:

Changes of the CR to TS38.331 contain:

- In section 5.2.2.4.2, for SIB1 acquisition, the condition bullet related to the NR-NS-PmaxList, is updated to allow the case that IAB-MT can ignore the advertised NS values.

Changes of the CR to TS38.306 contain:

- In section 4.2.7, the field description of ue-PowerClass and powerClass is updated to add clarification “This capability is not applicable to IAB-MT.”

Q4: Do you agree with the changes proposed in [4]? If not, please provide comments.

| Company | Preference (Y/N) | Detailed Comments |
| --- | --- | --- |

Q5: Do you agree with the changes proposed in [4]? If not, please provide comments.

| Company | Preference (Y/N) | Detailed Comments |
| --- | --- | --- |

# 3	Conclusion

This summary will be used to discuss way forward for IAB-MT capabilities based on LS in R2-200844, according to the following email discussion:

[AT112-e][021][IAB] UE capabilities (Nokia)

Treat tdocs under 6.2.5

Intended outcome: Intermediate: Determine agreeable parts. Final: For agreeable parts, agreed CRs.

# References

[1] R2-2008954, Discussion on the Issues from RAN4 LS on IAB-MT Feature List, CATT

[2] R2-2009417	Clarification on IAB-MT capability for Multiple NS	Nokia, Nokia Shanghai Bell

[3] R2-2009418	Clarification on Multiple NS and Pmax applicability to IAB-MT	Nokia, Nokia Shanghai Bell

[4] R2-2010352	Corrections based on RAN4 LS about IAB-MT feature	Huawei, HiSilicon

[5] R2-2010353	Corrections based on RAN4 LS about IAB-MT feature	Huawei, HiSilicon