3GPP TSG-RAN WG2 #112 electronic	R2-20xxxxx

Electronic Meeting, Nov 2-13, 2020

Agenda Item:	5.4.3

Source:	ZTE, Sanechips

Title:	Summary of offline [AT112-e][012][NR15] UE caps II (ZTE)

Document for:	Discussion, Decision

# 1	Introduction

This contribution summarizes the following discussion:

[AT112-e][012][NR15] UE caps II (ZTE)

Treat R2-2008710, R2-2009238, R2-2009239, R2-2009162, R2-2009163, R2-2009516, R2-2009517, R2-2010537, R2-2010536, R2-2010541, R2-2010540, R2-2009944

Intended outcome: Intermediate: Determine agreeable parts. Final: For agreeable parts, agreed CRs.

Deadline: Intermediate deadline(s) by Rapporteur, Final: Discussion stop at Wed Nov 11, 1200 UTC

Contact from companies

| Company | Email |
| --- | --- |
| Ericsson (Lian) | lian.araujo@ericsson.com |
| Ericsson (Martin) | martin.van.der.zee@ericsson.com |
| Qualcomm Incorporated | mkitazoe [at] qti.qualcomm.com |
| Intel | Seau.s.lim@intel.com |

# Discussion

## 2.1	Part 1: Intended to determine agreeable parts

Part 1 discussion is focusing on reaching conclusion whether the proposals/CRs can be agreed in principle, and Part 2 discussion would then focus on detailed changes for those agreeable contributions.

### 2.1.1 Clarify UE capability in case of cross-carrier operation

R2-2008710	LS on Interpretation of UE Features in Case of Cross-Carrier Operation (R1-2007334; contact: ZTE)	RAN1	LS in	Rel-15	NR_newRAT-Core	To:RAN2

R2-2009238	CR to clarify UE capability in case of cross-carrier operation	ZTE Corporation, Sanechips, Ericsson	CR	Rel-15	38.306	15.11.0	0418	-	F	NR_newRAT-Core

R2-2009239	CR to clarify UE capability in case of cross-carrier operation	ZTE Corporation, Sanechips, Ericsson	CR	Rel-16	38.306	16.2.0	0419	-	A	NR_newRAT-Core

Q1 Do companies agree with the CRs above?

| Company | Agree? (Yes or No) | Comments |
| --- | --- | --- |
| Ericsson (Lian) | Yes (Proponent) |  |
| Qualcomm Incorporated | Yes, but | We agree to the intention of the CRs. The category “Per serving cell” does not seem to be scalable for future extension, because in the future there can be UE capabilities which should be indicated for the serving cell triggering the command. It can be something like, “Triggered serving cell”. |
| Intel | Yes | We are fine with adding a new Annex for this. The future proofing from Qualcomm is also good in our view. |

### 2.1.2 Correction to BWP capability descriptions

R2-2009162	Correction to BWP capability descriptions	Nokia, Nokia Shanghai Bell	CR	Rel-15	38.306	15.11.0	0416	-	F	NR_newRAT-Core

R2-2009163	Correction to BWP capability descriptions	Nokia, Nokia Shanghai Bell	CR	Rel-16	38.306	16.2.0	0417	-	A	NR_newRAT-Core

Q2 Do companies agree with the CRs above?

| Company | Agree? (Yes or No) | Comments |
| --- | --- | --- |
| Ericsson (Lian) | Yes | On the change to clarify the relation between bwp-DiffNumerology and bwp-SameNumerology, wouldn’t it be simpler to say that a UE reporting bwp-DiffNumerology shall also report bwp-SameNumerology? |
| Qualcomm Incorporated | No | We do not agree to the first change. bwp-DiffNumerology should not include the UE capability for the same numerology. The rest can be release-16 correction only. |
| Intel | Yes | For the change in bwp-DiffNumerology, we also prefer to include a pre-requisite like ‘UE indicating support of this feature shall also indicate support of bwp-SameNumerology’. If this is not possible to be done in Rel-15 because of functional NBC, could check whether this can be done from Rel-16? |

### 2.1.3 Correction of the description of ue-SpecificUL-DL-Assignment

R2-2009516	Correction of the description of ue-SpecificUL-DL-Assignment	Apple	CR	Rel-15	38.306	15.11.0	0430	-	F	NR_newRAT-Core

R2-2009517	Correction of the description of ue-SpecificUL-DL-Assignment	Apple	CR	Rel-16	38.306	16.2.0	0431	-	A	NR_newRAT-Core

Q3 Do companies agree with the CRs above?

| Company | Agree? (Yes or No) | Comments |
| --- | --- | --- |
| Ericsson (Lian) |  | This seems more editorial change, so it could be merged. But we would be fine with the intention, maybe we could avoid referring to this parameter at all and have something as: “…and associated higher layer configured parameter TDD-UL-DL-ConfigDedicated as specified in TS 38.213 [11]”. |
| Qualcomm Incorporated | Yes, but | We think clarification in release-16 is sufficient. |
| Intel | Yes, but | It would be good to clarify from Rel-15 but merge with other Rel-15 CR |

### 2.1.4 Correction to the use of simultaneous CSI-RS resources

R2-2010537	Correction to the use of simultaneous CSI-RS resources	Ericsson	CR	Rel-15	38.306	15.11.0	0455	-	F	NR_newRAT-Core

R2-2010536	Correction to the use of simultaneous CSI-RS resources	Ericsson	CR	Rel-16	38.306	16.2.0	0454	-	A	NR_newRAT-Core

Q4 Do companies agree with the CRs above?

| Company | Agree? (Yes or No) | Comments |
| --- | --- | --- |
| Ericsson (Lian) | Yes (Proponent) |  |
| Qualcomm Incorporated | Yes |  |
| Intel | Yes |  |

### 2.1.5 Correction to pdcch-MonitoringSingleOccasion

R2-2010541	Correction to pdcch-MonitoringSingleOccasion	Ericsson	CR	Rel-15	38.306	15.11.0	0459	-	F	NR_newRAT-Core

R2-2010540	Correction to pdcch-MonitoringSingleOccasion	Ericsson	CR	Rel-16	38.306	16.2.0	0458	-	A	NR_newRAT-Core

Q5 Do companies agree with the CRs above?

| Company | Agree? (Yes or No) | Comments |
| --- | --- | --- |
| Ericsson (Lian) | Yes (Proponent) |  |
| Qualcomm Incorporated | Yes |  |
| Intel | Yes |  |

### 2.1.6 UE capability and cross-slot scheduling for Paging

R2-2009944	UE capability and cross-slot scheduling for Paging	Ericsson	discussion	Rel-15	NR_newRAT-Core

Observation 1: The UE is required to support K0>0 for DL PDSCH, but the UE may not have IOT tested this, and logs show that (some) REL-15 UEs do not support dl-SchedulingOffset-PDSCH-TypeA or dl-SchedulingOffset-PDSCH-TypeB. Observation 2: Rel-15 supports the default configurations Observation 3: Default PDSCH time domain resource allocation B for Paging and System Information includes both K0 = 0 and 1. Therefore it should be assumed that the UE supports K0 = 0 and 1 for Paging and System Information, even when the UE does not indicate support for dl-SchedulingOffset-PDSCH-TypeA or dl-SchedulingOffset-PDSCH-TypeB: Proposal 1: RAN2 to confirm that Rel-15 UE supports K0 = 0 and 1 for Paging and System Information. In case proposal 1 is agreeable, it can be discussed further if a clarification is needed (e.g. clarify that the UE supports the default configuration independent from the IOT capability signalling). In case proposal 1 is not agreeable, RAN2 should discuss if legacy UE supports K0 values in the pdsch-TimeDomainAllocationList provided in pdsch-ConfigCommon in SIB1 that have not been IOT tested by the UE, but the UE is only paged with K0=0 in the Paging PDCCH. This would enable the NW to use cross-slot scheduling for UEs that have indicated to support it, while using legacy scheduling for UEs that did not indicate support. But then dl-SchedulingOffset-PDSCH-TypeA and dl-SchedulingOffset-PDSCH-TypeB should be added to the UERadioPagingInformation message.

| dl-SchedulingOffset-PDSCH-TypeA Indicates whether the UE supports DL scheduling slot offset (K0) greater than 0 for PDSCH mapping type A. | UE | Yes | Yes | Yes |
| --- | --- | --- | --- | --- |
| dl-SchedulingOffset-PDSCH-TypeB Indicates whether the UE supports DL scheduling slot offset (K0) greater than 0 for PDSCH mapping type B. | UE | Yes | Yes | Yes |

Q6-1 Do companies agree that “the UE supports K0 = 0 and 1 for Paging and System Information, even when the UE does not indicate support for dl-SchedulingOffset-PDSCH-TypeA or dl-SchedulingOffset-PDSCH-TypeB?”

| Company | Agree? (Yes or No) | Comments |
| --- | --- | --- |
| Ericsson (Martin) | Yes | RAN1 specified in the L1-UE feature lists that the UE is required to support K0=1 for paging (38.822) independent from FR1 and FR2: 11) DL scheduling slot offset K0=1 for type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSS |
| Qualcomm Incorporated | Yes |  |
| Intel | Yes | Agree with Proposal 1 in the discussion document. |

Q6-2 If companies agree with Q6-1, do companies agree that “a clarification is needed (e.g. clarify that the UE supports the default configuration independent from the IOT capability signalling).”

| Company | Agree? (Yes or No) | Comments |
| --- | --- | --- |
| Ericsson (Martin) | Yes | In our understanding the RAN1 requirement specified in 38.822 for K0=1 was “lost in translation”, i.e. this requirements should be captured. |
| Qualcomm Incorporated | Yes, but | In light of the observation 1 in the document, which we agree to, we would like to get the confirmation from RAN2 that it is left to operators’ deployment to make sure there is no IOT problems with legacy UEs that are not IOTed for K0>0. |
| Intel |  | This feature is mandatory without signalling. |

Q6-3 If companies disagree with Q6-1, do companies agree that some spec modification is needed, e.g “add dl-SchedulingOffset-PDSCH-TypeA and dl-SchedulingOffset-PDSCH-TypeB to the UERadioPagingInformation message.”

| Company | Agree? (Yes or No) | Comments |
| --- | --- | --- |
| Ericsson (Martin) | Yes | In case the gNB would like to use K0>1 then the IOT capabilities should be included in the radio paging capabilities. |

## 2.2	Part 2: Intended to progress discussion on agreeable parts

- To be updated after discussion on part 1 -

# 3	Conclusion

- To be updated after discussion on part 1 -

# 4	References

- R2-2008710	LS on Interpretation of UE Features in Case of Cross-Carrier Operation (R1-2007334; contact: ZTE)	RAN1	LS in	Rel-15	NR_newRAT-Core	To:RAN2

- R2-2009238	CR to clarify UE capability in case of cross-carrier operation	ZTE Corporation, Sanechips, Ericsson	CR	Rel-15	38.306	15.11.0	0418	-	F	NR_newRAT-Core

- R2-2009239	CR to clarify UE capability in case of cross-carrier operation	ZTE Corporation, Sanechips, Ericsson	CR	Rel-16	38.306	16.2.0	0419	-	A	NR_newRAT-Core

- R2-2009162	Correction to BWP capabiltiy descriptions	Nokia, Nokia Shanghai Bell	CR	Rel-15	38.306	15.11.0	0416	-	F	NR_newRAT-Core

- R2-2009163	Correction to BWP capabiltiy descriptions	Nokia, Nokia Shanghai Bell	CR	Rel-16	38.306	16.2.0	0417	-	A	NR_newRAT-Core

- R2-2009516	Correction of the description of ue-SpecificUL-DL-Assignment	Apple	CR	Rel-15	38.306	15.11.0	0430	-	F	NR_newRAT-Core

- R2-2009517	Correction of the description of ue-SpecificUL-DL-Assignment	Apple	CR	Rel-16	38.306	16.2.0	0431	-	A	NR_newRAT-Core

- R2-2010537	Correction to the use of simultaneous CSI-RS resources	Ericsson	CR	Rel-15	38.306	15.11.0	0455	-	F	NR_newRAT-Core

- R2-2010536	Correction to the use of simultaneous CSI-RS resources	Ericsson	CR	Rel-16	38.306	16.2.0	0454	-	A	NR_newRAT-Core

- R2-2010541	Correction to pdcch-MonitoringSingleOccasion	Ericsson	CR	Rel-15	38.306	15.11.0	0459	-	F	NR_newRAT-Core

- R2-2010540	Correction to pdcch-MonitoringSingleOccasion	Ericsson	CR	Rel-16	38.306	16.2.0	0458	-	A	NR_newRAT-Core

- R2-2009944	UE capability and cross-slot scheduling for Paging	Ericsson	discussion	Rel-15	NR_newRAT-Core