3GPP TSG-RAN WG2 Meeting #112-e		R2-200xxxx

Electronic, 2nd – 13rd November 2020

Agenda Item: 	5.4.3

Source: 			Huawei, HiSilicon

Title: 	Summary of offline 013 Rel-15 UE caps III

Document for: 	Discussion and decision

- Introduction

This document summarizes the following offline discussion for Rel-15 UE capability corrections.

[AT112-e][013][NR15] UE caps III (Huawei)

Treat R2-2009480, R2-2008734, R2-2008770, R2-2008771, R2-2010241, R2-2010242, R2-2009392, R2-2009393, R2-2010239, R2-2010240, R2-2010545, R2-2010546, R2-2010561, R2-2010562

Intended outcome: Intermediate: Determine agreeable parts. Final: For agreeable parts, agreed CRs.

Deadline: Intermediate deadline(s) by Rapporteur, Final: Discussion stop at Wed Nov 11, 1200 UTC

- Contact from companies

| Company | Email |
| --- | --- |

- Discussion

  - Part 1 discussion: to achieve agreeable principle

Part 1 discussion is focusing on reaching conclusion whether the proposals/CRs can be agreed in principle, and Part 2 discussion would then focus on detailed changes for those agreeable contributions.

Note: Regarding R2-2010561, R2-2010562, as it was already discussed on Monday GTW, the current discussion did not cover these two contributions.

2.1.1 Clarification on the capability of supportedNumberTAG

Discussion is in [1]. The intention is to indicate the support of 2TAGs in a subset of the band combination,and the following 2 options are proposed:

- Option 1: UE is required to support the different TAGs in the different bands if the TAG number < band entry number;

- Option 2: Introduce the association between the TAG and the band entries, e.g. via the cell grouping;

Proposal 1: Adopt Option 1 or Option 2 for the supportedNumberTAG capability indication in the mix intra/inter-band BC.

Q1-1 Do companies agree with P1?

| Company | Yes/No | Comments |
| --- | --- | --- |

Q1-2 If companies agree with P1, which option is preferred and which release is expected to start the changes?

| Company | Option 1/2 | Release | Comments |
| --- | --- | --- | --- |

2.1.2 Clarification on RAN4 features of NE-DC

The corresponding LS and CRs are in [2]-[6]. The main intention is to clarify the applicability of some features for NE-DC. The two sets of the CRs are quite similar and the major differences are whether to also have changes for some MR-DC parameters. So it could be discussed first and then decide which set of CRs is used as the baseline.

Q2-1 Do companies agree syncIntraBandENDC, intraBandENDC-Support and UL-TimingAlignmentEUTRA-NR are applied to NE-DC as proposed in [3][4]?

| Company | Yes/No | Comments |
| --- | --- | --- |

Q2-2 Do companies agree with other changes for BandCombinationList and CA-ParametersEUTRA listed in [3][4][5][6]?

| Company | Yes/No | Comments |
| --- | --- | --- |

Q2-3 Please indicate which release to start adopting the changes if companies support in general to have the above changes?

| Company | Release | Comments |
| --- | --- | --- |

2.1.3 Correction on PDCP duplication capability for NR-DC

The CRs are in [7][8], and the intention is to add duplication related capabilities specifically for NR-DC.

Q3 Do companies agree with the major principle of the CRs? If yes, please indicate the starting release for the changes.

| Company | Yes/No | Release | Comments |
| --- | --- | --- | --- |

2.1.4 Clarification on handover capability

The CRs are in [9][10], and the main intention is to clarify how to interpret the FDD/TDD and FR1/FR2 differentiation.

Q4 Do companies agree with the major principle of the CRs? If yes, please indicate the starting release for the changes.

| Company | Yes/No | Release | Comments |
| --- | --- | --- | --- |

2.1.5 	Clarification on xDD differentiation

The CRs are in [11][12]. The main intention is to clarify how FDD/TDD differentiation applies to rsrqMeasWidebandEUTRA.

Q5 Do companies agree with the major principle of the CRs? If yes, please indicate the starting release for the changes.

| Company | Yes/No | Release | Comments |
| --- | --- | --- | --- |

  - Part 1 discussion summary

  - Part 2 discussion: TBD

To be updated after Phase I discussion

…

- Reference

R2-2009480	Clarification on the capability of supportedNumberTAG	Apple

R2-2008734	Reply LS on Clarification on RAN4 features of NE-DC (R4-2011688; contact: Samsung)

R2-2008770	Correction for RAN4 features of NE-DC	OPPO, Qualcomm Incorporated	CR

R2-2008771	Correction for RAN4 features of NE-DC	OPPO, Qualcomm Incorporated	CR

R2-2010241	Clarification on NE-DC for bandwidth combination set	Huawei, HiSilicon, Samsung

R2-2010242	Clarification on NE-DC for bandwidth combination set	Huawei, HiSilicon, Samsung

R2-2009392	Corrections on PDCP duplication capability for NR-DC	Huawei, HiSilicon

R2-2009393	Corrections on PDCP duplication capability for NR-DC	Huawei, HiSilicon

R2-2010239	Clarification on the inter-frequency handover capability	Huawei, HiSilicon, Ericsson

R2-2010240	Clarification on the inter-frequency handover capability	Huawei, HiSilicon, Ericsson

R2-2010545	Clarification on UE capabilities with FDD/TDD differentiation	Ericsson, ZTE Corporation, Sanechips

R2-2010546	Clarification on UE capabilities with FDD/TDD differentiation	Ericsson, ZTE Corporation, Sanechips