3GPP TSG-RAN WG2 Meeting #112-e		R2-200xxxx

Electronic, 2nd – 13rd November 2020

Agenda Item: 	5.4.3

Source: 			Huawei, HiSilicon

Title: 	Summary of offline 026 Rel-16 miscellaneous RAN4 issues

Document for: 	Discussion and decision

- Introduction

This document summarizes the following offline discussion for Rel-16 R4 related issues.

[AT112-e][026][R4 NR16] Miscellaneous (Huawei)

Treat R2-2008747, R2-2010598, R2-2010599, R2-2010358, R2-2008741, R2-2009346, R2-2010226, R2-2009245, R2-2009544

Intended outcome: Determine agreeable parts. For agreeable parts, agreed CRs.

Deadline: Intermediate deadline(s) by Rapporteur, Final: Discussion stop at Wed Nov 11, 1200 UTC, If feasible, NR UE caps 38306 38331 deadline Nov 6.

- Contact from companies

| Company | Email |
| --- | --- |
| Qualcomm Incorporated | Masato Kitazoe <mkitazoe [at] qti.qualcomm.com> |
| MediaTek (Felix) | Chun-Fan.Tsai@mediatek.com |
| Samsung (Seungri Jin) | seungri.jin@samsung.com |
| vivo (Chenli) | Chenli5g@vivo.com |
| Huawei, HiSilicon | Yang Zhao <zhaoyang@huawei.com> |

- Discussion

  - Part 1 discussion: to achieve agreeable principle

Part 1 discussion is focusing on reaching conclusion whether the proposals/CRs can be agreed in principle, and Part 2 discussion would then focus on detailed changes for those agreeable contributions.

2.1.1 CGI reading with autonomous gaps

The corresponding LS and CRs are in [1]-[4]. The intention is to capture RAN4’s agreement accordingly.

The CRs in [2][3][4] have common modification to apply 5 seconds to T321 for the case of CGI reading configured by NR towards FR2 cells with autonomous gaps.

The CR in [4] additionally aligned LTE specification to apply the value of 150ms to T321 for the case of CGI reading configured by NR towards EUTRAN cells with autonomous gaps

Q1-1 Do companies agree to apply 5 seconds to T321 for the case of CGI reading configured by NR towards FR2 cells with autonomous gaps?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm Incorporated | Yes |  |
| MediaTek | Yes |  |
| Samsung | Yes |  |
| vivo | Yes |  |
| Huawei, HiSilicon | Yes |  |
| ZTE | Yes | And both 38.331 and 36.331 need update. |

Q1-2 Do companies agree to apply 150 ms to T321 for the case of CGI reading configured by NR towards EUTRAN cells with autonomous gaps?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm Incorporated | No | It is our understanding that RAN4 hasn’t concluded on T321 value. Only CGI identification time is agreed, same as LTE, 150ms. T321 value however needs to take into account other processes that the UE undergoes, e.g. inter-RAT switch back and forth. |
| MediaTek | Yes, but | We are also fine to wait RAN4 to provide the value. |
| Samsung | Yes, but | We are also fine to wait RAN4 to provide the value. |
| vivo |  | We are also fine to wait for further RAN4 inputs. |
| Huawei, HiSilicon | Yes | We understand RAN4 would not have discussion anymore as it is straight forward to reuse LTE value. If companies feel safe to double check with RAN4, we think it would be good to send an LS confirming with RAN4. |
| ZTE | No | Although it is probably ok to reuse LTE value, we are afraid RAN2 cannot make such decision by our own. At least we(ZTE) are planning to bring contribution to next RAN4 meeting to clarify this, so we can wait for further inputs from RAN4. |

2.1.2 Support of HPUE

The corresponding LS and CRs are in [5]-[7]. The main intention is to support capabilities for high power UE.

Q2-1 Do companies agree with the major principle in [6][7]

| Company | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm Incorporated | No | We understand this has been taken care of in the email discussion 900 before the meeting. |
| MediaTek |  | Since this has been discussed in general capability CR, we don’t have to discuss this here. |
| Samsung |  | Agree with QC. |
| vivo |  | Agree with Qualcomm. |
| Huawei, HiSilicon |  | There was some misunderstanding on what was captured in the MEGA CR and now we confirm it was already there and CRs are not needed. |

2.1.3 UL Tx switching clarification

The CR is in [8], and the intention is to clarify the condition of supporting UL Tx switching for CA case.

Q3 Do companies agree with the major principle of the CR?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm Incorporated | Yes | This is acceptable compromise. |
| MediaTek | Yes |  |
| Samsung | Yes |  |
| vivo | Yes |  |
| Huawei, HiSilicon | No | In RAN1 feature list, the prerequisites of UL Tx switching in UL CA case and EN-DC case are clearly indicated as 6-6 normal UL CA and EN-DC respectively. This was explicitly discussed and concluded in RAN1. We think RAN2 should not revert RAN1's conclusion. In addition, we understand the design of UL Tx switching in UL CA and EN-DC case in RAN4 and RAN1 is based on legacy UL CA and EN-DC framework, e.g. power control, regardless of option1 or option2, therefore if RAN2 makes this change, this is not consistent with RAN4 conclusion as well. So we do not agree with the proposal. |
| ZTE | Yes | Proponent As we know, when RAN1 discussing the prerequisite, companies did not touch such detail differentiation of option1/option2. So there is no explicit conclusion saying it applies to both option1/option2 cases. Technically, for “option1 only” capable UEs, they do not support 1T+1T UL transmission in UL Tx switching, then asking them to mandate support 1T+1T UL CA does not make much sense. |

2.1.4 CA additional spectrum emission requirements

The CR is in [9], and the main intention is to clarify the limitation that same emission requirements should be applied for each uplink carrier on same band and configured value is applicable for all uplink carriers of same band.

Q4 Do companies agree with the major principle of the CR?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm Incorporated | Yes |  |
| MediaTek | Yes, but | We would like to check whether this is Rel-16 only issue. Do we need this in Rel-15? In the CR cover page on “reason for change” “In the WI NR_RF_FR1-Core a support for CA is being added.” I assume that NR CA is already supported from Rel-15. Or does it try to say that intra-band CA is introduced in Rel-16 ? |
| Samsung | Yes |  |
| vivo | Yes | We understand this is only for Rel-16. |
| Huawei, HiSilicon | Yes | We also understand this is only for Rel-16. |
| ZTE | Yes |  |

  - Part 1 discussion summary

  - Part 2 discussion: TBD

To be updated after Phase I discussion

…

- Reference

R2-2008747	Reply LS on CGI reading with autonomous gaps (R4-2012156; contact: ZTE)	RAN4

R2-2010598	Correction to 38.331 on T321 for autonomous gap based CGI in FR2	ZTE Corporation, Sanechips

R2-2010599	Correction to 36.331 on T321 for autonomous gap based CGI in FR2	ZTE Corporation, Sanechips

R2-2010358	38331 CR on CGI reading with autonomous gaps	Huawei, HiSilicon

R2-2008741	LS on UE capability for PC2 inter-band EN-DC (LTE FDD+NR TDD) (R4-2011787; contact: China Unicom)	RAN4

R2-2009346	38306 CR for the support of EN-DC FDD+TDD HPUE	China Unicom, Huawei, HiSilicon

R2-2010226	support of EN-DC TDD-FDD HPUE	Huawei, HiSilicon, China Unicom

R2-2009245	CR to add prerequisite of UL Tx switching capability	ZTE Corporation, Sanechips

R2-2009544	NR CA additional spectrum emission requirements	Nokia, Nokia Shanghai Bell