3GPP TSG-RAN WG2 Meeting #112-e	R2-2011065

Online meeting, 2nd - 13th November, 2020

Title:	[DRAFT] Reply LS on questions to RAN WGs on dual Radio UE (2Rx/2Tx or 2Rx/1Tx) support for simultaneous communication with both SNPN and PLMN

Response to:	S2-2007827

Release:	Release 17

Work Item:	FS_eNPN

Source:	Futurewei [TSG RAN WG2]

To:	TSG SA2

Cc:	TSG RAN WG4

Contact Person:

#### Name:	Mazin Al-Shalash

E-mail Address:	mazin.shalash@futurewei.com

Attachments:	n/a

1. Overall Description:

RAN2 would like to thank SA2 for the LS in S2-2007827 (R2-2010691). RAN2 has discussed support for simultaneous communication with both SNPN and PLMN for the following dual Radio UE architectures:

- Dual radio UE using independent Rx/Tx per network (SNPN and PLMN)

- Dual radio UE using independent Rx per network (SNPN and PLMN) and a single Tx for one of the two networks only, e.g. the SNPN (whereby UL user-plane and NAS traffic for the other network is tunnelled via the first network using existing IP-based OTT mechanisms)

RAN2 has the following feedback on SA2’s questions:

Q1: is a) technically feasible without any new Access-Stratum mechanism and standardization?

A1: For scenario a) dual radio UE using independent Rx/Tx per network, RAN2 concluded that it is technically feasible for the UE to simultaneous communicate with both SNPN and PLMN without new AS mechanisms.

This assumes that the UE’s RF frontend is able to operate independently on the carrier frequencies/bands in use in each network, without this operation resulting in significant interference between the two radios.. Minimum performance requirements for the UE’s radios may need to be standardized.

Q2: is b) technically feasible taking into account the uplink Access Stratum activity in each network?

A2: For scenario b) dual radio UE using independent Rx per network (SNPN and PLMN) and a single Tx for one of the two networks only, RAN2 reiterated that if the UE’s RRC state is RRC_CONNECTED in one network (e.g. PLMN) then its RRC state cannot also be RRC_CONNECTED in the other network (e.g. SNPN). The UE may be in RRC_IDLE state with respect to the other network (e.g. SNPN), which would enable it to receive broadcast traffic via the air interface of this second network.

From RAN2 perspective, it is technically feasible for the UE to tunnel traffic (e.g. UP and NAS) for one network (e.g. SNPN) via the other network (e.g. the PLMN), as long as the RRC state of the UE for the first network (e.g. SNPN) is RRC_IDLE.

Q3: whether in case of b) is it feasible to achieve a very low PER for low latency multicast traffic without sending access stratum feedback to the network (e.g. the SNPN RAN)?

A3: RAN working groups are currently considering two delivery modes for MBS: One is used by UEs in RRC_CONNECTED to support high QoS services, and the other to support low QoS services where the UE can also receive data in RRC_INACTIVE/RRC_IDLE.

To receive multicast traffic RAN2 is so far assuming that the UE should use the high QoS mode (reliability, latency), and AS feedback is normally required to support reliable reception of multicast traffic, i.e. the UE needs to be in RRC_CONNECTED for the high QoS mode.

To receive broadcast traffic, RAN2 is so far assuming that the UE can use the low QoS mode, for which AS feedback is not necessary .

2. Actions:

ACTION: 	RAN2 respectfully asks SA2 to take into account RAN2 feedback to the questions raised in LS S2-2007827.

3. Date of Next TSG-RAN WG2 Meetings:

3GPP RAN2#113-e		25th of Jan – 5th of Feb 2021			Electronic Meeting

3GPP RAN2#113-bis-e		12th of April – 20th of April 2021		Electronic Meeting