3GPP TSG-RAN WG2 Meeting #112-e	R2-20xxxxx

Online, 2 – 13 Nov, 2020

Agenda Item:	6.5.3

Source:	Huawei, HiSilicon

Title:	Summary of [AT112-e][041][IIOT] MAC I

Document for: Discussion and Decision

# Introduction

This is the summary of the following email discussion:

[AT112-e][041][IIOT] MAC I (Huawei)

Scope: Treat tdocs R2-2009500, R2-2009373, R2-2009375, R2-2009483 R2-20010054, R2-2009541, R2-2009374

Intended outcome: Intermediate: Determine agreeable parts. Final: For agreeable parts, agreed CRs.

Deadline: Intermediate deadline(s) by Rapporteur, Final: Thu Nov 12, 1200 UTC

R2-2009500 and R2-20010054 are not included in this email discussion as they have been treated online.

Contact from companies

| Company | Email |
| --- | --- |
| LG | SeungJune Yi (seungjune.yi@lge.com) |
| Nokia | Ping-Heng Wallace Kuo (Ping-Heng.Kuo@nokia.com) |
| Samsung | Sangkyu Baek (sangkyu.baek@samsung.com) |
| Ericsson | Zhenhua Zou (Zhenhua.Zou@Ericsson.com) |
| Sharp | Fangying.xiao@cn.sharp-world.com |
| ZTE | Dong.fei@zte.com.cn |
| Huawei | Tao.cai@hauwei.com |

# Discussion

## CR on condition of a de-prioritized grant

R2-2009373	states that, in the current MAC spec, an uplink grant can be considered as a de-prioritized/prioritized uplink grant only if the MAC entity is configured with lch-basedPrioritization. It is suggested as: “For the MAC entity configured with lch-basedPrioritization” is added before the sentence “If the corresponding PUSCH transmission of a configured uplink grant is cancelled by CI-RNTI as specified in clause 11.2A of TS 38.213 [6] or cancelled by a high PHY-priority PUCCH transmission as specified in clause 9 of TS 38.213 [6], this uplink grant is considered as a de-prioritized uplink grant.”, in order to be aligned with other texts in the MAC spec.

Q1 Do companies agree with the CR above?

| Company | Agree? (Yes or No) | Comments |
| --- | --- | --- |
| LG | Yes |  |
| Nokia | Yes |  |
| Samsung | No (no strong view) | When lch-basedPrioritization is not configured, we don’t need to consider it is a de-prioritized uplink grant. But even if we consider it is a de-prioritized uplink grant, this classification is not used anywhere. It does not make anything broken in the current text. So, we think this CR is a cosmetic change with text improvement. Thus we slight prefer not to do this. But no strong view. |
| Ericsson | No | We do not see any issue in the current MAC spec to support autonomousTx without lch-basedPrioritzation. The grant is deprioritized due to PHY-related prioritization and this feature of autonomous transmission is technically not an LCH-based prioritization but rather a recovery mechanism for the deprioritized grant. We don’t think introducing this additional restriction brings any benefits and we prefer having flexibility for network implementation. As a matter of fact, there is no configuration restriction in RRC saying that autonomousTx can only be configured when lch-basedPrioritzation is configured. |
| Sharp | Yes |  |
| ZTE | No | Share the same view with Samsung, if this can be accepted and which measns we shall only use the term of “prioritized” only if the Ich-basedPrioritizatation is configured , we shall change the SR part as well, for example: ===== 38.321 ==================== 1> else, for the SR configuration corresponding to the pending SR: 2> when the MAC entity has an SR transmission occasion on the valid PUCCH resource for SR configured; and 2> if sr-ProhibitTimer is not running at the time of the SR transmission occasion; and 2> if the PUCCH resource for the SR transmission occasion does not overlap with a measurement gap: 3> if the PUCCH resource for the SR transmission occasion overlaps with neither a UL-SCH resource nor an SL-SCH resource; or 3> if the MAC entity is able to perform this SR transmission simultaneously with the transmission of the SL-SCH resource; or 3> if the MAC entity is configured with lch-basedPrioritization, and the PUCCH resource for the SR transmission occasion does not overlap with an uplink grant received in a Random Access Response nor with the PUSCH duration of a MSGA payload, and the PUCCH resource for the SR transmission occasion for the pending SR triggered as specfied in clause 5.4.5 overlaps with any other UL-SCH resource(s), and the physical layer can signal the SR on one valid PUCCH resource for SR, and the priority of the logical channel that triggered SR is higher than the priority of the uplink grant(s) for any UL-SCH resource(s) where the uplink grant was not already de-prioritized, and the priority of the uplink grant is determined as specified in clause 5.4.1; or <omit for short> 3> else: 4> consider the SR transmission as a de-prioritized SR transmission. ========= From 38.321 ===================== |
| Huawei | Yes | According to RRC spec, the need code for autonomousTx is Cond LCH-BasedPrioritization, which means lch-BasedPrioritization is the prerequisite of autonomousTx. autonomousTx-r16 ENUMERATED {enabled} OPTIONAL -- Cond LCH-BasedPrioritization We have used “de-prioritized” almost from the beginning of the prioritization discussion. It would be confusing to say an uplink grant is deprioritized or prioritized when the feature of lch-basedPrioritiztion is not configured for the UE. |

Summary and Proposal:

## On the case when SR and PUSCH conflict

R2-2009483	discusses the case when the SR and PUSCH conflict, it is argued that:

If lch-basedPrioritization is configured, UE behavior on the SR and PUSCH conflict was clearly agreed in RAN2#108 meeting as below:

•	For the SR&PUSCH with different LCH priority, MAC delivers SR or PUSCH to PHY based on the LCH priority;

•	For the SR&PUSCH with equal LCH priority, MAC delivers PUSCH to PHY.

Accordingly, the below proposal is made:

Proposal: UE MAC only provides SR or PUSCH to PHY when the SR and PUSCH resource are overlapped, i.e. no possibility to deliver both SR and PUSCH to PHY in the conflict case.

Q2 Do companies agree with the proposal above?

| Company | Agree? (Yes or No) | Comments |
| --- | --- | --- |
| LG | Yes |  |
| Nokia | Yes | This is also our understanding, but do we need any change in MAC spec. to capture this? It seems quite clear already. |
| Samsung | No | This proposal is related to sequential generation scenario, e.g. - at t1, MAC PDU is generated and deliverd to PHY. But PHY did not transmit PUSCH yet. - at t2, SR with higher priority is triggered. There is sufficient timeline, so MAC instruct to signal the SR. MAC PDU is not transmitted. We think the proposal prohibits this scenario by UE implementation. So we prefer not to agree anything. Regarding LG’s comment, we agree there is no MAC impact. There may be a RAN1 impact of this proposal. |
| Ericsson | No | The agreement is copied below. If PUCCH resource for an SR’s transmission occasion overlaps a UL-SCH resource, SR’s transmission is allowed (prioritized) based on a comparison of priority of the LCH that triggered the SR and a priority value for the UL-SCH resource (where the priority value is determined as in previous agreement), if the priority of the LCH that triggered the SR is higher. This does not preclude the case where a PUSCH has started but there comes a SR in the middle of the PUSCH transmission that can cancel/puncture the PUSCH transmission. It might be worthwhile to confirm w/ Ran1 if this is supported and in which scenario is supported, e.g., might be okay if SR is high-PHY priority and PUSCH is low-PHY priority and might not be okay if both have the same PHY-priority. It is essentially the same exercise we have done for data-data prioritization. |
| Sharp | Yes | There is no MAC impact. |
| ZTE | Yes | No change on MAC is needed |
| Huawei | No | Agree with Samsung. It is possible that MAC PDU is generated for the PUSCH, then SR with higher LCH priority is triggered and the PUCCH for SR is overlapped with the PUSCH. PHY can transmit the SR if it is with higher L1 priority. |

Summary and Proposal:

## On data & SR overlapping with equal L1 priority

R2-2009375	discusses the case when Data and SR overlap with equal L1 priority and SR is prioritized in MAC, it is stated that:

In Rel-16, if the priority of the logical channel that triggered SR is higher than the priority of the uplink grant, and the SR and the UL grant are of the same L1 priority, it is not clear whether PHY layer can signal SR if only SR is instructed to the PHY for transmission. If the PHY layer can signal SR, the MAC layer can instruct SR transmission to the PHY layer, otherwise the MAC layer will only deliver the data.

Accordingly, the below proposal is made:

Proposal 1: RAN2 to confirm the intended UE behaviour for the case that the overlapped data and SR are of equal L1 priority and SR is prioritized in MAC, i.e. whether the MAC can instruct PHY for SR transmission.

Q3: Which below option on the intended UE behavior companies agree with, for the case when SR and data overlap with equal L1 priority and SR is prioritized in MAC?

Option 1: MAC can instruct PHY for SR transmission

Option 2: MAC deliver only the data to PHY for transmission

Other option(s):

| Company | Option | Comments |
| --- | --- | --- |
| LG | 1 | We think there is no problem to send SR in PHY layer if MAC instructs SR transmission without delivering data. |
| Nokia | 1 | Since the SR has higher LCH priority than the UL grant, we should deliver SR only. This way we can have a MAC behavior that is consistent with data v.s. data conflict, i.e. deliver the one with higher LCH priority. |
| Samsung | 1 | LCH based prioritization didn’t assume PHY prioritization. So, we can say that we already considered the equal PHY priority case as a baseline. |
| Ericsson | 2 | The intention is the option 2, with the understanding that equal L1 priority of SR and PUSCH cannot be transmitted together in PHY (pending confirmation by RAN1). The below highlighted part is added in the last meeting to prevent option 1 (i.e., instructs an SR that is not transmitted anyhow by PHY) 3> if the MAC entity is configured with lch-basedPrioritization, and the PUCCH resource for the SR transmission occasion does not overlap with an uplink grant received in a Random Access Response nor with the PUSCH duration of a MSGA payload, and the PUCCH resource for the SR transmission occasion for the pending SR triggered as specfied in clause 5.4.5 overlaps with any other UL-SCH resource(s), and the physical layer can signal the SR on one valid PUCCH resource for SR, and the priority of the logical channel that triggered SR is higher than the priority of the uplink grant(s) for any UL-SCH resource(s) where the uplink grant was not already de-prioritized, and the priority of the uplink grant is determined as specified in clause 5.4.1; or We are open to discuss how to resolve the discrepancy between RAN1 and RAN2 specs so that the intention is captured, with least spec change efforts. |
| Sharp | 1 | This is the intended UE behaviour of the agreement in RAN2#108 meeting: • For the SR&PUSCH with different LCH priority, MAC delivers SR or PUSCH to PHY based on the LCH priority; |
| ZTE | 1 | As quoted by Ericsson, since it is not clear in RAN1 whether the SR is transmitted or not, thus using of “ the PHY layer can signal the SR on one valid PUCCH resource for SR” will cause the mutual ambiguities issue. From implementation point of view, MAC can always generate one of them to instruct PHY to perform transmission based on the LCH based prioritization rule. |
| Huawei | 1 | We agree with Nokia that SR v.s. data with equal L1 priority can have a consistent behavior with data v.s. data with equal L1 priority. MAC shall deliver the SR to PHY for transmission if the SR has higher LCH priority. |

Summary and Proposal:

## On configuring L2 priority and PHY priority

R2-2009541	discusses the configuration of L2 priority and PHY priority based on the below agreement:

- R2 assumes that PHY-based prioritization and LCH-based prioritization are configured independently and one can be configured without the other (assumption may be modified when LS reply from R1 is received)

- Postpone the discussion on additional conditions for Phy Priority and L2 priority feature (assume this can be added later).

It is argued that, there is no need for joint LCH based prioritization and PHY based prioritization, considering:

- The UE capability on LCH based and PHY based prioritization is separate.

- There are potential not complex solutions for only one of the two is configured.

- It is up to gNB implementation to assure LCH based prioritization and PHY based prioritization configured together.

Accordingly, the below proposal is made:

Proposal 1	LCH based prioritization and PHY based prioritization can be configured independently.

Q4 Do companies agree with the proposal above?

| Company | Agree? (Yes or No) | Comments |
| --- | --- | --- |
| LG | No | We don’t see any benefit in configuring only one of them. The purpose of IIOT WI is to ensure prioritized transmission of URLLC data, and if only one of them is configured, the WI purpose is not met. |
| Nokia | Yes | In practice we think it is the best to configure both of them to optimize intra-UE prioritization features. But from spec. point of view we probably don’t need to mandate such joint configuration, so we can have more gNB implementation flexibility. Besides, RAN2 has agreed before that they can be configured independently. |
| Samsung | Yes | Agree with Nokia |
| Ericsson | Yes | It is agreed in this meeting that - No need to introduce additional configuration for Phy Priority and L2 priority feature. It is our understanding that it is not feasible to have a joint configuration. |
| Sharp | Yes | Agree with Nokia. |
| ZTE | Yes | No more discussion |
| Huawei | Yes | Agree with Nokia |

Summary and Proposal:

## On explicit indication of PHY based prioritization

R2-2009374	discusses the configuration for physical layer prioritization and makes the below proposal:

It is stated that, when UE reports the capability parameter ul-IntraUE-Mux-r16 to the network, the network can decide whether PHY based prioritization can be configured for the UE.

It is observed that, if any of the following parameters is configured by the network, PHY based prioritization shall be viewed as configured for the UE.

- priorityIndicatiorDCI-0-1 in PUSCH-Config;

- priorityIndicatiorDCI-0-2 in PUSCH-Config;

- phy-PriorityIndex in ConfiguredGrantConfig;

- phy-PriorityIndex in SchedulingRequestResourceConfig.

Accordingly the below proposal is made:

Proposal 1: No explicit indication of PHY based prioritization is needed.

Q5 Do companies agree with the proposal above?

| Company | Agree? (Yes or No) | Comments |
| --- | --- | --- |
| LG | Yes | We think it is already agreed in the online session. |
| Nokia | Yes | The agreement made online already captures this: - No need to introduce additional configuration for Phy Priority and L2 priority feature. |
| Samsung | Yes | No need to discuss again |
| Ericsson | Yes |  |
| Sharp | Yes |  |
| ZTE | Yes | No more discussion |
| Huawei | Yes |  |

Summary and Proposal:

# Conclusion

Proposals:

# References

- R2-2009373	Clarification on the condition of a de-prioritized grant	Huawei, HiSilicon	CR	Rel-16	38.321	16.2.1	0928	-	F	NR_IIOT-Core

- R2-2009375	Clarification of PHY behaviour for Data & SR overlapping with equal L1 priority	Huawei, HiSilicon	discussion	Rel-16	NR_IIOT-Core

- R2-2009483	Clarification on the SR and PUSCH conflict with equal LCH priority	Apple	discussion	Rel-16	NR_IIOT-Core

- R2-2009541	Consideration on L2 priority and PHY priority feature	OPPO	discussion	Rel-16	NR_IIOT-Core

- R2-2009374	Clarification of configuration for physical layer prioritization	Huawei, HiSilicon	discussion	Rel-16	NR_IIOT-Core.