3GPP TSG-RAN WG2 #112-e	R2-2011039

Electronic, November 2nd – 13th, 2020

Agenda Item:	6.1.2

Source:	Ericsson

Title:	Summary on [AT112-e][046][NR16] Out-of-order CBG-based re-transmission

Document for:	Discussion, Decision

# 1	Introduction

This document summarizes the following at-meeting email discussion after the 1st round online discussion:

[AT112-e][046][NR16] Out-of-order CBG-based re-transmission (Ericsson)

Scope: Treat incoming LS (when it arrives), R2 input (R2-2010049), and make and agree on related Draft CRs.

Intended outcome: Endorsed Draft CRs

Deadline: by the Rapporteur (dep on R1).

R2-2010049	Out-of-order CBG-based re-transmission(s) with cancelled initial PUSCH transmission	Ericsson	discussion	Rel-16

1st DISCUSSION

-	Intel think that once we have info from R1, this will need significant discussion.

-	Chair: We wait for R1. Separate email discussion on this topic (Ericsson), to be kicked-off as soon as LS from R1 is available. Rapporteur creates a draft for how to capture in R2 TSs.

To leave time for further discussion (if needed), the deadline for inputs is Wed Nov 11, 2000 UTC.

Contact

| Company | Name | Email address |
| --- | --- | --- |
| Ericsson | Zhenhua Zou | Zhenhua.Zou@ericsson.com |

# 2	Discussion

RAN1 has concluded the following, see the latest RAN1 chair notes [2]

Agreements: - When UE reports 5-25, the UE supports both in-order and out-of-order CBG-based retransmission(s) (not requiring 11-12 as prerequisite even for Rel-16 UE) - For Rel-16, new FG for UE supporting only in-order CBG-based retransmission(s) (not requiring 5-25 as prerequisite) is introduced - Whether/what TP for TS38.214 in Rel-16 is necessary should be discussed – Klaus (Nokia) - Details of the new FG description should also be discussed

The FG 5-25 [1] is the Rel-15 capability bit cbg-TransIndication-UL, see below in TS 38.306

| cbg-TransIndication-UL Indicates whether the UE supports CBG-based (re)transmission for UL using CBG transmission information (CBGTI) as specified in TS 38.214 [12]. | UE | No | No | No |
| --- | --- | --- | --- | --- |

Subsequently, RAN1 agrees to introduce the new FG 11-12 with details shown below

| 11. NR_L1enh_URLLC | 11-12 | CBG-based re-transmission for UL using CBGTI with only in-order CBG-based re-transmission(s) for cancelled initial PUSCH transmission | 1. Support of CBG-based PUSCH re-transmission(s) of a TB using CGBTI in case the initial PUSCH transmission was not cancelled due to gNB scheduling/indication/configuration. 2. Support of CBG-based PUSCH re-transmission(s) of a TB using CGBTI in case the initial PUSCH transmission was cancelled due to gNB scheduling/indication/configuration and the following condition is satisfied: the UE is scheduled for a re-transmission of a CBG #N in a given TB when CBG #N-1 has been transmitted before or is scheduled in the same UL grant that includes CBG#N. |  | Yes | N/A |  | Per UE | No | No |  |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

The above RAN1 agreement is in align with the principle of the approach 2 in the paper R2-2010049 (copied in Annex 5 for easy reference), in which a new “slimmed-down” version of the legacy bit is introduced. The RAN1 FG 11-12 is more precise in the sense that it also includes the case when there is no cancellation from the gNB. Per Ran1 feedback, due to NBC concern, Rel-15 cbg-TransIndication-UL cannot have new FG 11-12 as prerequisite and it is unnecessary to do something different in Rel-16. Additionally, with this approach, the only change is to add a new capability in Rel-16 38.306 which also simplifies a lot the RAN2 signalling discussion. From the rapporteur point of view, this seems to be one way forward.

Question 1: Do you support adding a new capability bit for FG 11-12 and no other changes are needed?

| Company | Support (y/n) | Additional comments |
| --- | --- | --- |

The TPs can be as below

#### 4.2.7.10	Phy-Parameters

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| cbg-TransCancelledPUSCH-UL-r16 Indicates whether the UE supports CBG-based re-transmission(s) of a TB using CBG transmission information (CBGTI) as specified in TS 38.214 [12] in the following two cases: - if the initial PUSCH transmission was not cancelled due to gNB scheduling/indication/configuration; and - if the initial PUSCH transmission was cancelled due to gNB scheduling/indication/configuration and the following condition is satisfied: the UE is scheduled for a re-transmission of a CBG #N in a given TB when CBG #N-1 has been transmitted before or is scheduled in the same UL grant that includes CBG#N. | UE | No | No | No |
| cbg-TransIndication-UL Indicates whether the UE supports CBG-based (re)transmission for UL using CBG transmission information (CBGTI) as specified in TS 38.214 [12]. | UE | No | No | No |

The parameter is added in the RRC IE Phy-ParametersCommon

Phy-ParametersCommon ::=            SEQUENCE {

[[

cbg-TransCancelledPUSCH-UL-r16          ENUMERATED {supported}              OPTIONAL

]]

}

Question 2: Do you agree with the above TP to capture the FG 11-12?

| Company | Support (y/n) | Additional comments |
| --- | --- | --- |

Lastly, please fill in the below table if there are any other issues that need to be discussed in the email.

| Company | Additional issues |
| --- | --- |

# 3	Conclusion

TBC

# 4. References

TR 38.822, User Equipment (UE) feature list, Rel-15

Session Notes for NR UE Features, AI 7.2.11, 3GPP TSG RAN WG1 #103-e, October 26th – November 13th, 2020, NTT DoCoMo, Inc.

- https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_103-e/Inbox/Hiroki_sessions/Session%20Notes_Hiroki_NR_UEFeatures_v009.zip

# 5. Annex - Approach 2 of paper R2-2010049

Approach 2 for Rel-15:

No change to Rel-15. Rel-15 FG would hence only have the legacy bit with the legacy meaning, i.e. it would indicate that the UE supports both in-order and out-of-order CBG retransmissions.

| cbg-TransIndication-UL Indicates whether the UE supports CBG-based (re)transmission for UL using CBG transmission information (CBGTI) as specified in TS 38.214 [12]. | UE | No | No | No |
| --- | --- | --- | --- | --- |

Approach 2 for Rel-16:

Add a new bit for the in-order scheduling alternative. Basically a slimmed down version of the legacy bit.

| cbg-TransCancelledPUSCH-UL-r16 Indicates whether the UE supports CBG-based retransmission for UL using CBG transmission information (CBGTI) in case the initial PUSCH transmission was cancelled and the following condition is satisfied: the UE is scheduled for a re-transmission of a CBG #N in a given TB when CBG #(N-1) has been scheduled before or CBG #(N-1) is scheduled in the same UL grant that includes CBG#N. The UE indicating support of cbg-TransIndication-UL also supports this feature. | UE | No | No | No |
| --- | --- | --- | --- | --- |
| cbg-TransIndication-UL Indicates whether the UE supports CBG-based (re)transmission for UL using CBG transmission information (CBGTI) as specified in TS 38.214 [12]. | UE | No | No | No |

UE signalling:

- A Rel-15 UE can indicate that it supports CBG retransmission by indicating that it supports cbg-TransIndication-UL. Both in-order and out-of-order CBG retransmission are supported.

- A Rel-16 can indicate that

  - it supports in-order CBG retransmission, and does not support out-of-order CBG retransmission, when initial PUSCH transmission is cancelled, by indicating that it does not support cbg-TransIndication-UL and support cbg-TransCancelledPUSCH-UL.

  - it supports both in-order and out-of-order CBG retransmission regardless of cancellation of initial PUSCH transmission, by indicating that it supports cbg-TransIndication-UL. In this case, the UE has to indicate support of cbg-TransCancelledPUSCH-UL.

This approach has the advantage that it is backwards compatible.

A drawback is a Rel-15 UE cannot indicate that it supports only in-order CBG retransmission. Since the understanding is that out-of-order is more difficult to implement, the entire feature of CBG-based retransmissions has a risk of not being implementable as there is no Rel-15 UE reporting the support of both out-of-order and in-order.