3GPP TSG RAN WG2 #112-e		R2-20xxxxx

Online, 2nd – 13th November, 2020

Title:	LS on Paging Enhancement

Response to:

Release:	Release 17

Work Item:	NR_UE_pow_sav_enh-Core

Source:	MediaTek, RAN2

To:	RAN1

Cc:

Contact Person:

Name: Li-Chuan TSENG

E-mail Address:	Li-Chuan.Tseng@mediatek.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

In RAN2#112-e, RAN2 discussed UE grouping for paging enhancement in Rel-17 UE power saving WI. RAN2 confirmed that UE grouping is considered as a candidate of paging enhancements for UE power saving. Regarding paging for UE subgroups, RAN2 has discussed and considered the following methods:

- Paging indication for UE subgroups using paging DCI

- Paging early indication (PEI) / wake-up signal (WUS) for UE subgroups

- Cross-slot Scheduling for UE subgroups

Notice that these methods are not mutually exclusive.

From RAN2 perspective, the following methods are de-prioritized:

- P-RNTI based group discrimination is deprioritized from RAN2 perspective

- Paging for UE subgroups using different time/frequency resources

2. Actions:

To RAN1:

RAN2 respectfully asks RAN1 to provide their recommended solution for paging indication/scheduling, and information on the feasibility and limitations of carrying subgroup information with the recommended solution.

3. Date of Next RAN1 Meetings:

TSG-RAN WG2 Meeting 113-e 	25 January – 05 February 2021		Online

TSG-RAN WG1 Meeting 113bis-e	12 – 20 April 2021 	 		Online