3GPP TSG-RAN WG2 #112-e	R2-20xxxxx

Electronic Meeting, 2 – 13 Nov 2020

Agenda Item:	7.4.2

Source:	ZTE Corporation, Sanechips

Title:	[AT112-e][214][NR][MOB] Avoiding DAPS with multi-TRP/CA/DC (ZTE)

Document for:	Discussion, Decision

1	Introduction

This document is to collect companies comment in the following email discussion:

[AT112-e][214][NR][MOB] Avoiding DAPS with multi-TRP/CA/DC (ZTE)

Scope:

    - Discuss the CRs under AI 4.5, 7.1.X and 7.5 marked for this email discussion

Intended outcome:

    - Agreeable Stage-2 CRs in R2-2010748 (38.300, revision of R2-2009384) and R2-2010747 (36.300, revision of R2-2009382),

    - Agreeable Stage-3 CRs in R2-2010749 (36.331, revision of R2-2009769) and R2-2010750 (38.331, revision of R2-2009383)

Deadline for providing comments and for rapporteur inputs:

    - Initial deadline (for companies' feedback):  2nd week Thu, UTC 1000

    - Deadline for CR finalization: 2nd week Thu, UTC 1700

2	Discussion

To make it easier to find the correct contact delegate in each company for potential follow-up questions, the rapporteur encourages the delegates who provide input to provide their contact information in this table:

| Company | Delegate contact |
| --- | --- |

| ZTE | zhang.mengjie@zte.com.cn |
| --- | --- |
| OPPO | lihaitao@oppo.com |
| Ericsson | Mattias.a.bergstrom@ericsson.com |
| Intel | Yi.guo@intel.com |
| Huawei, HiSilicon | tangxun@huawei.com |
| LG | geumsan.jo@lge.com |
| Sharp | ningjuan.chang@cn.sharp-world.com |

At RAN2#112e meeting, it was discussed how to avoid simultaneous operation of CA, DC, or multi-TRP with DAPS and the followings were noted:

Network ensures that SCG and/or SCells are not configured when UE receives DAPS HO. This will typically require network to do RRC reconfiguration before sending DAPS HO command.

Network ensures that multi-TRP does not operate simultaneously with DAPS HO. This will typically require network to do RRC reconfiguration before sending DAPS HO command.

FFS how to capture this in Stage-2 and Stage-3, handled in Offline 214

Based on agreements above, the rapporteur has updated Stage-2 and Stage-3 CRs and uploaded them in the draft.

Companies are requested to add their comments in the boxes below.

2.1	Stage-2 CRs

R2-2010747	Clarification on no support of CA or DC with DAPS		ZTE Corporation	CR	Rel-16	36.300	16.3.0	1320	1	F	LTE_feMob-Core	  R2-2009382

R2-2010748	Clarification on no support of CA, DC or multi-TRP with DAPS	ZTE Corporation	CR	Rel-16	38.300	16.3.0	0307	1	F	NR_Mob_enh-Core	R2-2009384

NR:

NOTE 3:	Only PCell is kept during DAPS handover. All other serving cells and multi-DCI/single-DCI based multi-TRP are released by the network before the handover command is sent to the UE.

LTE:

During DAPS handover, UE maintains only PCell connection with both source and target cells and any other configured serving cells are released by network before the handover command is sent to the UE. When DAPS handover is configured, PDCP duplication is not allowed.

Question 1: Do companies agree the changes proposed in the drafts R2-2010748 (NR) and R2-2010747(LTE)? And if any additional correction is needed for the CRs?

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes | The stage-2 CR captured that the NW should release CA, DC or multi-TRP before sending DAPS HO command. |
| OPPO | Yes |  |
| Ericsson | Yes |  |
| Intel | Yes | The changes are ok. But the coversheet should be improved. „If the UE implements the CR and the network does not, the NW may configure DAPS HO with CA or DC simultaneously, and the UE behavior is unpredictable which violates agreements on no support for simultaneous operation of DAPS with CA or DC.“. „The UE can operate simultaneously with DAPS and CA or DC. If the NW configures DAPS HO with CA or DC simultaneously, the UE behavior is unpredictable “ |
| Huawei, HiSilicon | Yes |  |
| LG | Yes |  |
| Sharp | Yes |  |

2.2	Stage-3 CRs

R2-2010750	Clarification on no support of CA, DC or multi-TRP with DAPS	ZTE Corporation	CR	Rel-16	38.331	16.2.0	2061	1	F	NR_Mob_enh-Core	R2-2009383

R2-2010749	Clarification on no support of CA or DC with DAPS		ZTE Corporation	CR	Rel-16	36.331	16.2.1	4486	1	F	LTE_feMob-Core	  R2-2009769

NR:

daps-Config

Indicates that the bearer is configured as DAPS bearer.This field is optional present, need N, in case masterCellGroup includes ReconfigurationWithSync, MR-DC and CA are not configured, multi-DCI/single-DCI based multi-TRP are not configured in any DL BWP and ethernetHeaderCompression is not configured for the DRB. Otherwise the field is absent.

LTE:

daps-HO

This field indicates that the handover, triggered in the same RRCConnectionReconfiguration message, shall be performed as a DAPS HO for the DRB. DAPS HO is not configured when the rach-Skip is included or if uplinkDataCompression or ethernetHeaderCompression is configured for the DRB. daps-HO is not configured in the RRCConnectionReconfiguration message included in a conditionalReconfiguration. daps-HO is not configured if DC or CA is configured.

Question 2: Do companies agree the changes proposed in the drafts R2-2010750 (NR) and R2-2010749(LTE)? And if any additional correction is needed for the CRs?

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes | The stage-3 CR captured that the NW should not configure CA, DC or multi-TRP in the DAPS HO command. For the terminology of multi-TRP, although there is no clear definition of “multi-DCI/single_DCI based multi-TRP” in the current spec, we found that the similar terminology has been used in TS 38.306 to describe muti-TRP related capabilities. So we assume the meaning of “multi-DCI/single_DCI based multi-TRP” are clear to all companies. If this is not the case, we are also fine to add more details to elaborate the meaning of “multi-DCI/single-DCI based multi-TRP”, and we may also need to update the description in TS 38.306 accordingly. Besides, there is a stage-2 CR (R2-2009170) on introducing stage-2 description of multi-TRP is discussed in the eMIMO WI. And one email discussion is allocated for this issue (i.e. [AT112-e][107][eMIMO] Stage 2 CRs (Nokia)). If the CR is agreed, we can add a stage-2 reference for multi-TRP in the stage-3 CR. |
| OPPO | See comments | For NR, we wonder for the multi-TRP part, we should say “configured in any DL BWP“ or only in the active DL BWP? For LTE, should DC be MR-DC? |
| Ericsson | Yes | Agree with ZTE that perhaps the simplest approach would be if we in Stage-2 add a description for mTRP is added, and then we can use these terms freely in Stage-3. Our interpretation of the current status is that there cannot be "any" DL BWP with mTRP configured during DAPS. |
| Intel | Yes | Same comments as above on the coversheet. |
| Huawei, HiSilicon | Yes |  |
| LG | Yes |  |
| Sharp | Yes |  |

2.2	Clarification on “other configuration”, “SpCell Configuration” in DAPS handover command

R2-2009272	Release SCells/SCG configuration during DAPS HO	Intel Corporation	discussion	Rel-16	NR_Mob_enh-Core, LTE_feMob-Core

Proposal 2:To add the RRC specification, “other configuration”, “SpCell Configuration” in DAPS handover command is applied for target side;

This contribution was also discussed during RAN2#112e meeting and the following was noted:

Offline 214 to discuss if we add to the RRC specification that “other configuration”, “SpCell Configuration” in DAPS handover command is applied for target side

Question 3: Do companies agree with proposal 2 above?

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes | It’s fine to clarify that “other configuration”, “SpCell Configuration” in DAPS handover command is applied for target side to keep the consistent description for reconfiguration handling. |
| OPPO | Yes |  |
| Ericsson | No? | We have not seen an explicit example TP so far. It may be OK to do the change if it does not become messy in the specification. We do not see that there is a big room for misinterpretation. |
| Intel | Yes | It would be good to align the handling across the spec. |
| Huawei, HiSilicon |  | We would like to suggest to implement this P2 in draft CRs, in this way we can see the spec impact clearly. |
| LG | Yes |  |
| Sharp | No strong opinion | We understand the intention of the proposal, but agree with Ericsson that there seems be not much misinterpretation here. |

3	Conclusion

In the previous sections we made the following observations:

Based on the discussion in the previous sections we propose the following: