3GPP TSG-RAN WG2 #112e	R2-20xxxxx

Electronic meeting, November 2nd – 13th 2020

Agenda Item:	6.4.2

Source:	Ericsson

Title:	Summary of [AT112-e][708][V2X] SL related RRC procedure

Document for:	Discussion, Decision

1	Introduction

This document is to kick off the following email discussion:

[AT112-e][708][V2X] SL related RRC procedure (Ericsson)

Discuss CRs (including need of changes) in the above list (in Recommendation2) and prepare the agreeable CR in R2-2010937 (discussion summary in R2-2010936 if needed). CR will be agreed by email. Deadline is 12:00pm 11/12/2020 (UTC).

Deadline Phase 1: Collect companies’ views and formulate proposals, by Friday November 6th 12:00 UTC

Deadline Phase 2: Further review proposals and related CRs, by Thursday November 12nd 1200 UTC

2	Contact Information

| Company (Name) | Email |
| --- | --- |
| Ericsson (Tony) | antonino.orsino@ericsson.com |
| OPPO (Qianxi) | qianxi.lu@oppo.com |
| CATT (Da) | wangda@catt.cn |
| Huawei (Xiao) | xiaoxiao6@huawei.com |
| Samsung (Hyunjeong) | hyunjeong.kang@samsung.com |

3		Discussion

3.1	SDAP entity reconfiguration

R2-2009406	Correction on SDAP related procedures and configurations in TS 38.331	Huawei, HiSilicon	CR	Rel-16	38.331	16.2.0	2070	-	F	5G_V2X_NRSL-Core

| Company | Agree (Y/N)? | Comments |
| --- | --- | --- |
| OPPO | Yes |  |

| LG | Yes |  |
| --- | --- | --- |

| Ericsson | No | We are not sure the change is correct. The proposed change said that the SDPA entity should be (re)configured, but the section in which is added is that one fort he sidelink DRB addition. This sounds a bit strange. If the SDAP entity already exist, shouldn’t the reconfiguration oft he SDAP entity part of the sidelink DRB modification? |
| --- | --- | --- |
| Interdigital | Yes |  |
| CATT | Yes |  |
| Huawei | Yes | Without this change, the SDAP entity cannot be reconfigured when the mapping of PC5 QoS flow to SLRB changes. |
| Samsung | Yes |  |

3.2	SL related reset operation

R2-2009713	Correction on sidelink reset configuration	Ericsson	CR	Rel-16	38.331	16.2.0	2118	-	F	5G_V2X_NRSL-Core

| Company | Agree (Y/N)? | Comments |
| --- | --- | --- |
| OPPO | See comment | Intention agreeable, suggest rewording as follows: Since SRB of specified configuration does not need to be mentioned at all, and it is a bit contradictory to say the configuration in RRCRecofnigurationSidelink is included but later say DRB configuration is not included. And we wonder what is the reason for the „may“ here, can it be removed? |

| LG |  | Ok with OPPO’s suggested text. |
| --- | --- | --- |

| Ericsson | Yes with comment | We are fine with OPPO suggestion. The reason of why there is “may” here is because some radio configuration are optional and thus some of them may not be present. This is also the same formulation that we have for Uu. |
| --- | --- | --- |
| Interdigital | Yes | Ok with the suggested rewording. However, we think may can be kept. |
| CATT | Yes with comment | We are also fine with OPPO suggestion wording. We also think “may” is correct. |
| Huawei | Yes | No strong view. Since it is to change informative texts, we can follow majority’s view whether to make it clearer. |
| Samsung | Yes with comment | We are fine with OPPO’s rewording. |

R2-2010302	Correction on trigger of SL specific MAC reset in TS 38.331	Huawei, Hisilicon	CR	Rel-16	38.331	16.2.0	2205	-	F	5G_V2X_NRSL-Core

| Company | Agree (Y/N)? | Comments |
| --- | --- | --- |
| OPPO | Yes with comment | Intention agreeable, i.e., the processing for connection release requested by upper layers is missing. On the other hand, this CR put the steps into the section of „SRB release“, which is not very decent – one alternative is similar to section 5.8.9.3 handling RLF case, to have a separate section for the parrellel case, i.e., connection release requested by upper layers. |

| LG | Yes |  |
| --- | --- | --- |

| Ericsson | Yes with comment | Agree with OPPO. |
| --- | --- | --- |
| Interdigital | Yes |  |
| CATT | Yes | Agree with OPPO. |
| Huawei | Yes | This is to add a missing case for SL MAC reset, as agreed before. |
| Samsung | Yes |  |

3.3	Operations related to SL-RSRP configuration

R2-2009520	Correction on Stored Sidelink Measurement Configuration	Apple, Ericsson	CR	Rel-16	38.331	16.2.0	2085	-	F	5G_V2X_NRSL-Core

| Company | Agree (Y/N)? | Comments |
| --- | --- | --- |
| OPPO | See comment | Change-2 is OK for us. For change-1, although we understand the intention, yet the initiation condition for as-configuration is missing in the whole spec, not just for measurement, because we rely on the section 5.8.9.1.1 for that There is no need to go into details, so we do not think change- 1 is needed. |

| LG | Yes |  |
| --- | --- | --- |

| Ericsson | Yes |  |
| --- | --- | --- |
| Interdigital | Yes |  |
| CATT | Yes |  |
| Huawei | See comments | For Change 1, i.e. adding one more trigger for RRCRecofnigurationSidelink, we, as pointed out in the summary, would like to add it into 5.8.9.1.1, instead of 5.2.2.4.13 as currently proposed by the CR. The reason is that the triggers of RRCReconfigurationSidelink are now all specified and gathered in subclause 5.8.9.1.1, and this framework should not be broken. Otherwise, we may start to distribute the triggers everywhere in the Spec. A very simple change, if made in 5.8.9.1.1, is as follows: - the (re)configuration of the peer UE to perform NR sidelink measurement and report. For the need of Change 2, we have no strong view and are fine to follow majority’s view. However, if companies want to change, the current change in the CR is not accurate, because for NR SL communication procedures in TS 38.331, we don’t use “Out-of-coverage” as the RRC state that allows the UE to use preconfiguration. Reason is that the preconfiguration usage is not limited to out of coverage case (i.e. no NR coverage on Uu), but also applies to the case that the UE has NW coverage in Uu but cannot get RAN-provided SL configurations for the SL carrier — This is how the current SL procedures (e.g. NR SL communication reception/transmission, CBR configurations, etc.) work, and should be followed. |
| Samsung | See comment | The second change is fine. Regarding 1st change, we have similar view with OPPO and HW and the change is not needed. |

3.5	SL synchronization related

R2-2008942	Discussion on synchronization procedure under in-device coexistence operation	LG Electronics France	discussion	Rel-16	38.331	5G_V2X_NRSL-Core

| Company | Agree (Y/N)? | Comments |
| --- | --- | --- |
| OPPO | See reply to the following question |  |
| Ericsson | No | We prefer the solution in R2-2010442 |
| Interdigital | No | Also prefer a note |
| CATT | No for Opt 1‎ | For Opt 2, i.e., to add a NOTE, we slight prefer the NOTE added in R2-2010442. |
| Huawei | No | No normative texts are needed as what we analyzed in our paper below, R2-2010442, because the issue is already handled in RAN1 Spec. |
| LG | Yes | We believe a clarification is needed to avoid a possible ambiguity in understanding the operations specified in 38.213 and 38.331 spec, under in-device coexistence. Otherwise, some vendors may make UE to transmit/receive subframe boundary aligned signals while others may not, which causes an inter-vendor interoperability issue for IDC operation. Based on discussions with some companies, if most of companies have common understanding that the normative text in 38.331 does not need to be changed, we agree to add a NOTE in Section 5.8.6.2 of TS 38.331 as follows. NOTE: When NR SL and LTE SL are time-division multiplexed and UE knows the frame indexes for the two SL RAT, the UE transmits and receives with subframe boundary alignment between NR/LTE SL under in-device coexistence operation, as specified in Section 16.7 of TS 38.213. |
| Samsung | No | Prefer R2-2010442 |

R2-2010442	On synchronization alignment between V2X SL and NR SL in the in-device coexistence environment	Huawei, HiSilicon	discussion

| Company | Agree (Y/N)? | Comments |
| --- | --- | --- |
| OPPO | Yes |  |
| Ericsson | Yes |  |
| Interdigital | Yes |  |
| CATT | Yes |  |
| Huawei | Yes, with comment | Our first preference is to do nothing. If companies want some forms of clarification, a NOTE referencing TS 38.213 is more than enough. |
| LG | Yes | Similar to LGE response to R2-2008942 above, we believe a clarification is needed to avoid a possible ambiguity in understanding the operations specified in 38.213 and 38.331 spec, under in-device coexistence. As a result, we support adding a note in TS38.331. Regarding the NOTE that was proposed in R2-2010442, we suggest to use the same terminology as in TS38.213 as follows: NOTE X: How the UE achieves subframe boundary alignment between V2X sidelink communication and NR sidelink communication under in-device coexistence (if both are performed by the UE) is as specified in TS 38.213, subcases 16.7. We’re ok with either this NOTE or the one suggested in the response to R2-2008942 above. |
| Samsung | Yes |  |

3.6	SL CG handling during the RLF case

R2-2010060	Correction on SL configured grant type 1 validity under RLF	Nokia, Nokia Shanghai Bell	CR	Rel-16	38.331	16.2.0	2171	-	F	5G_V2X_NRSL-Core

| Company | Agree (Y/N)? | Comments |
| --- | --- | --- |
| OPPO | No | The related text in 331 is to allow the usage of exceptional pool when there is no configured grant provided. For the case when both exceptional pool and configured grant are provided during T310, the following note in MAC spec can solve the issue, i.e., relying on CG resource. NOTE 1: If the MAC entity is configured with Sidelink resource allocation mode 2 to transmit using a pool of resources in a carrier as indicated in TS 38.331 [5] or TS 36.331 [21], the MAC entity can create a selected sidelink grant on the pool of resources based on random selection or sensing only after releasing configured sidelink grant(s), if any. |

| LG | Yes | The proposal text describes the RAN2 agreement even more clearly. Although the MAC specification explains that selected sidelink grant can be created after releasing configured sidelink grant, it is necessary to have more clear text in the RRC specification that the exceptional pool can be used when the T310 timer expires if the configured sidelink grant is configured. |
| --- | --- | --- |

| Ericsson | Yes | We think the clarification makes sense and it is, indeed, needed in RRC. We support this. |
| --- | --- | --- |
| Interdigital | No | The change results in unitended behavior when CG is not configured. We think this agreement should be handled in the MAC (and the note mentioned by OPPO seems sufficient). |
| CATT | Yes | We prefer to have such clarification in RRC spec and the proposal text in this CR makes sense. |
| Huawei | No | Share OPPO’s view, basically. The enhancement of resuming the SL CG usage had actually been discussed quite a lot during previous meetings, and finally it was agreed that the prohibition of using exceptional pool, before the SL CG is released, is specified in the MAC, with the below note in TS 38.321. NOTE 1: If the MAC entity is configured with Sidelink resource allocation mode 2 to transmit using a pool of resources in a carrier as indicated in TS 38.331 [5] or TS 36.331 [21], the MAC entity can create a selected sidelink grant on the pool of resources based on random selection or sensing only after releasing configured sidelink grant(s), if any. This note means that when SL CG type 1 is configured, then even if an exceptional pool is configured, it cannot be used until the SL CG type 1 is released at the moment when T311 starts as specified in TS 38.331. Therefore, the previous RAN2 agreements referenced by the above CR is already realized by the current Spec. |
| Samsung | No | Agree with OPPO |

3.7	Correction related to SL RLF procedure

R2-2009711	Correction on setting of sl-FailureList in SidelinkUEInformation	Ericsson	CR	Rel-16	38.331	16.2.0	2116	-	F	5G_V2X_NRSL-Core

| Company | Agree (Y/N)? | Comments |
| --- | --- | --- |
| OPPO | Yes |  |

| LG | Yes |  |
| --- | --- | --- |

| Ericsson | Yes |  |
| --- | --- | --- |
| Interdigital | Yes |  |
| CATT | Yes |  |
| Huawei | Yes, with comments | We share the intention. The specific change can be further reviewed during the CR reiewing phase. |
| Samsung | Yes |  |

4	Conclusion

According to the previous sections the following proposals are made: