3GPP TSG-RAN WG2 Meeting #113bis-e						R2-2102814

E-Meeting, 12th - 20th April 2020

Title:	 [draft] LS to RAN1 on the minimum gap ensuring issue

Response to:

Release:	Rel-16

Work Item:	5G_V2X_NRSL

Source:	vivo [RAN2]

To:	RAN1

Cc:

Contact Person:

Name:	Jing Liang

E-mail Address:	<liangjing@vivo.com>

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

1. Overall Description:

In 38.321, the minimum time gap between any two selected resources of the selected sidelink grant is ensured in case that PSFCH is configured for the pool when a UE performs resource (re-)selection in it. The following text is an example:

5.22.1.2 TX resource (re-)selection check <text omitted…> 1> if retransmission of a MAC PDU on the selected sidelink grant has been dropped by either sidelink congestion control as specified in clause 8.1.6 of TS 38.214 or de-prioritization as specified in clause 16.2.4 of TS 38.213 [6], clause 5.4.2.2 of TS 36.321 [22] and clause 5.4.2.2: 2> remove the resource(s) from the selected sidelink grant associated to the Sidelink process, if the resource(s) of the selected sidelink grant is indicated for re-evaluation or pre-emption by the physical layer; 2> randomly select the time and frequency resource from the resources indicated by the physical layer as specified in clause 8.1.4 of TS 38.214 [7] for either the removed resource or the dropped resource, according to the amount of selected frequency resources, the selected number of HARQ retransmissions and the remaining PDB of either SL data available in the logical channel(s) by ensuring the minimum time gap between any two selected resources of the selected sidelink grant in case that PSFCH is configured for this pool of resources, and that a resource can be indicated by the time resource assignment of a SCI for a retransmission according to clause 8.3.1.1 of TS 38.212 [9];

RAN2 discussed the current text of condition to ensure minimum time gap and understand it is not aligned with RAN1 agreement made in RAN1 #100e meeting. However, RAN2 has no consensus to change MAC specification on this in mode-2 resource (re-)selection to align with RAN1 agreement because of technical concern i.e., if minimum time gap should be ensured when HARQ is enabled, TB may be dropped for which HARQ FB is enabled but resource without ensuring minimum time gap is selected.

RAN2 would like to request feedback from RAN1 on the following question:

Q1: Is RAN2’s concern valid that if the minimum time gap is ensured when HARQ is enabled, TB may be dropped for which HARQ FB is enabled but resource without ensuring minimum time gap is selected? If so, whether it needs to be solved?

Q2: Does RAN1 has any concern if RAN2 keep the current MAC specification to define the condition to ensure minimum time gap as ‘when PSFCH is configured for this pool of resources’?

2. Actions:

To RAN WG1: RAN2 respectfully requests RAN1 to feedback on Q1 and Q2 as above.

3. Dates of Next TSG-RAN WG2 Meetings:

TSG RAN WG2 Meeting #114-e	19 May – 27 May 2021	eMeeting