3GPP TSG-RAN WG2 Meeting #113bis electronic	R2-2103494

Online, April 12 – 20, 2021

Agenda Item:	8.7.4.2

Source: 	Huawei, HiSilicon

Title: 	Adaptation layer functionalities for L2 U2N relay

Document for:	Discussion and Decision

- Introduction

According to the newly approved WID for NR Sidelink Relay [1], one important objective specific to L2 relaying is to specify the mechanisms for U2N Adaptation layer design.

Work Item objectives specific to Layer-2 (L2) relaying: - Specify mechanisms for U2N Adaptation layer design [RAN2] - For bearer mapping and Remote UE identification, incl. RAN related security aspects if any

Based on the discussion during SI stage, there are still some leftovers we should resolve in the WI stage. In this contribution, we would like to have detailed analysis on the remaining issues of the adaptation layer functionality for L2 U2N relay and provide corresponding observations and proposals.

- Discussion

  - The necessity of Adaptation layer at PC5

In the SI stage, one of the controversial issues was whether to support Adaptation layer at PC5 between Remote UE and Relay UE. There were two main motivations raised by the proponent companies. First, was that it can be used to enable flexible mapping, e.g., more than one Uu radio bearers with similar QoS requirement can share one PC5 RLC bearer and such functionality is beneficial to reduce LCID space. Second, one was that it can provide forward compatibility to multi-hop case in the further release.

Some other companies had opposite opinion that the LCID space of the Relay UE is not the bottleneck.

In NR sidelink the LCID space is maintained per PC5 RRC connection for unicast, instead of per UE and as specified in MAC specification, the LCID value ranges for Uu and PC5 are the same. Considering that both Uu radio bearers and PC5 RLC bearers are configured by RAN in UE-to-Network relay scenarios, it is not clear in which case RAN needs to configure more Uu radio bearers than PC5 RLC bearers if these Uu radio bearers can be mapped to one PC5 RLC bearer.

Furthermore, multi-hop relay is not in the scope of current release [1]. To minimize specification impact, we propose not to support the adaptation layer at PC5. That is, the proposed protocol stack of L2 U2N relay should be as shown in Fig. 1.

Observation 1: Without the need of multi-hop functionality, the only benefit of PC5 Adaptation layer is to support N:1 mapping at Remote UE to save PC5 RLC space, which should not be a critical issue.

Proposal 1: No Adaptation layer is needed at PC5 for L2 UE-to-Network relay. [Cross-WG related: At least SA2 should be aware of the final protocol stack ASAP]

- User plane

- Control plane

Figure 1: Protocol Stack for L2 UE-to-Network Relay

  - Uu Adaptation layer functionality

2.2.1 Bearer mappings at Relay UE

We discuss the detailed functionality of the Relay UE’s Uu Adaptation layer to enable successful uplink and downlink data transmission when N:1 bearer mapping is supported, i.e., different end-to-end RBs of the same Remote UE and/or different Remote UEs can be multiplexed over one Uu RLC channel.

We consider the uplink data transmission and downlink data transmission respectively, and discuss how Relay UE’s Uu Adaptation layer supports the bearer mapping.

- UL data transmission

In the uplink, PDCP PDUs from Remote UE are relayed and transferred to the corresponding E2E PDCP entity of the gNB. The whole process can be illustrated in 3 steps.

Step 1. Remote UE transfers PDCP PDUs from its Uu PDCP entities to the associated PC5 RLC bearers.

Step 2. Relay UE receives the PDCP PDUs from the PC5 RLC bearers, and adds the Remote UE’s E2E bearer IDs to the Adaptation layer header (see Table 2.1), such that RLC SDUs can be delivered correctly to the E2E PDCP entities in gNB (see step 3). Therefore, Relay UE should be configured with the mapping table between PC5 RLC IDs and Remote UE E2E bearer IDs, to determine the adaptation header to be added.

Table 2.1: operation for adding adaptation header at Relay UE for UL

For UL, Ingress PC5 RLC ID => Remote UE E2E bearer ID to be added in the header

Next, Relay UE transfers the RLC PDUs to the Uu RLC bearers. To achieve this, Relay UE should also be configured with the mapping table between Remote UE bearer IDs and Uu RLC bearer IDs (see Table 2.2).

Table 2.2: bearer mapping operation at Relay UE

For UL, Remote UE E2E bearer ID in the header => egress Uu RLC ID;

For DL, Remote UE E2E bearer ID in the header => egress PC5 RLC ID;

Step 3. The gNB receives the RLC SDUs from the Uu RLC bearers, and delivers them to the correct E2E PDCP entity according to the E2E bearer ID in Adaptation layer header.

- DL data transmission

In the downlink, the operations for transferring the gNB’s PDCP PDUs to the Remote UE’s E2E PDCP entities are similar.

Step 1. gNB’s Uu adaptation layer adds the Remote UE’s E2E bearer ID to the Adaptation layer header, and deliver the PDCP PDUs to Uu RLC channels.

Step 2. Relay UE receives RLC SDUs from the Uu RLC Channels. Then, it reads the Remote UE E2E bearer ID in the adaptation header, and transfers them to the PC5 RLC channels according to the mapping table between PC5 RLC IDs and Remote UE E2E bearer IDs (see Table 2.2).

Step 3. Remote UE transfer the RLC SDUs from PC5 RLC channels to E2E PDCP entities.

Based on the above procedures and operations, followings should be configured to the relay UE.

Proposal 2: Relay UE is configured with the mapping table between PC5 RLC IDs and Remote UE E2E bearer IDs, to determine the adaptation header to be added.

Proposal 3: Relay UE is configured with the mapping table between Remote UE E2E bearer IDs and Uu RLC bearer IDs.

Proposal 4: The Remote UE Uu Radio Bearer identification is the Remote UE’s E2E DRB ID.

- 2.2.2 Remote UE ID allocation

In addition to different bearers, different Remote UEs may be mapped into the same Uu RLC bearer. To distinguish the Remote UEs, additional information should be included in Adaptation layer header which is added to PDCP PDU.

Reusing the method in LTE, the Remote UE can be identified in the Adaptation layer header by local identifier (i.e. an index). The local identifier can be allocated by the Relay UE, which is able to uniquely identify one Remote UE under the Relay UE. As the allocated ID is in the scope of the Relay UE, it is also named the local identifier.

Proposal 5: The Remote UE identification is Remote UE’s local ID allocated by the Relay UE, as in LTE.

Proposal 6: The Remote UE’s local ID is allocated by Relay UE and uniquely identify one Remote UE in the scope of the Relay UE.

As we will see in chapter 2.2.3, the local identifier can be used as a reference to the specific Remote UE. The gNB should know the association between “Relay UE identification + local ID” and the specific Remote UE to: 1) determine which PDCP entity an RLC SDU from Uu RLC channel belongs to in the uplink data transmission and 2) add local identifier in the PDCP PDU from an Remote UE’s PDCP entity in the downlink. This is necessary to enable correct data routing when multiple Remote UEs share the same relay.

Observation 2: gNB needs to be aware of the association between “Relay UE identification + local ID” and the specific Remote UE.

One good way to achieve the purpose in Observation 2 is as follows. Relay UE allocates a unique identity to each Remote UE when it receives the Msg3 from the Remote UE. Then, the Relay UE adds the allocated ID into the adaption layer header of Msg3 data and delivers it to the gNB, so that the gNB can associate the remote UE ID with the local ID.

Proposal 7: The local ID is allocated during Msg3 transmission, and included in the adaption layer header of Msg3 data.

- 2.2.3 Remote UE identification for Data routing

As mentioned above, multiple Remote UEs can access to the gNB via the same Relay UE. As stated in section 2.2.2, a local identifier is needed between Relay UE and gNB to distinguish the Remote UEs. Here we would like to illustrate the routing mechanism based on the local identifier via the example shown in Figure 2.

Figure 2: Illustration for data routing between Remote UE and gNB via Relay UE

For the uplink, the routing processes can be divided into two steps.

Step 1. The Relay UE knows which UE (i.e., Remote UE1 or Remote UE2) the PC5 connection is associated with.

The Relay UE receives uplink RLC SDUs via the PC5 connections from the Remote UEs. Based on the mapping relation between PC5 connection and Remote UE local IDs, the Relay UE adds Adaptation layer headers including associated local identifiers to the received RLC SDUs, as shown in Table 2.3.

Step 2. When gNB receives the data, it determines which Remote UE the data belongs to.

The Relay UE transmits the RLC SDUs together with Adaptation layer header to Uu RLC channels. As the gNB knows the association between “Relay UE identification + local ID” and the specific Remote UE, it delivers the RLC SDUs to the destination Remote UE’s higher L2 entity according to the local identifier in the Adaptation layer header.

For the downlink, the whole process is similar.

Step1: The gNB knows which UE (i.e., Remote UE1 or Remote UE2) the DL data belongs to.

The gNB establishes higher L2 entities (i.e., Uu SDAP and Uu PDCP) for the DRBs of each UE sharing the same lower L2 entities (i.e., RLC and MAC) and gNB maintains the UE context including the local identifier of each UE.

When DL data arrives from one PDCP entity, the gNB knows which UE the PDCP entity belongs to. Correspondingly, the gNB is able to determine the local identifier to be included in the Adaptation layer header. Then gNB sends the PDCP PDU together with the Adaptation layer header to Relay UE.

Setp2: When Relay UE receives the data, it determines which Remote UE the data belongs to.

Upon receiving the data from gNB, the Relay UE is able to interpret the Adaptation layer header and get the included information, i.e., local ID. Based on the local ID, the Relay UE is able to know the associated Remote UE, and transfer it to the corresponding PC5 connection, as shown in Table 2.3.

Table 2.3: operation for data routing at Relay UE

For UL, ingress PC5 connection=>Remote UE local ID to be added in the header

For DL, Remote UE local ID in the header=> egress PC5 connection

Proposal 8: Relay UE maintains the mapping between PC5 connection and Remote UE local ID.

- Conclusion

In this contribution, we discussed considerations on Adaptation layer functionalities for L2 U2N relay and have the following observation and proposal:

Observation 1: Without the need of multi-hop functionality, the only benefit of PC5 Adaptation layer is to support N:1 mapping at Remote UE to save PC5 RLC space, which should not be a critical issue.

Proposal 1: No Adaptation layer is needed at PC5 for L2 UE-to-Network relay. [Cross-WG related: At least SA2 should be aware of the final protocol stack ASAP]

Proposal 2: Relay UE is configured with the mapping table between PC5 RLC IDs and Remote UE E2E bearer IDs, to determine the adaptation header to be added.

Proposal 3: Relay UE is configured with the mapping table between Remote UE E2E bearer IDs and Uu RLC bearer IDs.

Proposal 4: The Remote UE Uu Radio Bearer identification is the Remote UE’s E2E DRB ID.

Proposal 5: The Remote UE identification is Remote UE’s local ID allocated by the Relay UE, as in LTE.

Proposal 6: The Remote UE’s local ID is allocated by Relay UE and uniquely identify one Remote UE in the scope of the Relay UE.

Observation 2: gNB needs to be aware of the association between “Relay UE identification + local ID” and the specific Remote UE.

Proposal 7: The local ID is allocated during Msg3 transmission, and included in the adaption layer header of Msg3 data.

Proposal 8: Relay UE maintains the mapping between PC5 connection and Remote UE local ID.

- Reference

- RP-210904, “New WID on NR Sidelink Relay”, Ericsson (Moderator), OPPO (rapporteur), March 16-26, 2021.