3GPP TSG RAN WG2 Meeting# 113bis electronic   	                            R2-2103619

Online, April 12 – April 20, 2021

Agenda Item:	8.16.3

Source: 	Sony

Title:	UE on-boarding cell reselection

Document for:	Discussion

# Introduction

In this contribution we look into the SA2 LS response and propose that UE shall include the new indication only if the cell broadcast support for on-boarding.

# Discussion

SA2 response:

Question 3: Can RAN2 assume uniform support of onboarding in all cells in an O-SNPN? (I.e. can RAN2 assume that all cells of an O-SNPN broadcasts the support for onboarding or can some cells not set the ”onboardingEnabled” bit to e.g. control RAN congestion?) [SA2 answer] The ”onboardingEnabled” bit can be set/enabled per cell e.g. when onboarding is enabled in only part of the SNPN network and can also be used to avoid the load from onboarding UEs. The parameter is used to assist the UE in network selection. Even if there is no uniform support and a UE moves to a cell in an O-SNPN not supporting onboarding, SA2 foresees no impact to mobility procedures as remote provisioning can continue in the target cell. Once the PDU session for remote provisioning has been activated existing 5GS functionality applies for mobility.

SA2 response mentions that there is no impact on UE mobility i.e. if a cell does not broadcast on-boarding indication then UE shall not search for another cell. Based on SA2 response there are three scenarios:

- Cell does not support the capability of UE on-boarding

- Cell supports on-boarding capability and currently loaded/overloaded

- Cell supports on-boarding capability and currently not-loaded

We think there is no point in distinguishing #1 and #2 as the UE behaviour won’t be different except for the consequence that UE should include on-boarding request in RRC connection establishment only if the cell broadcast 1-bit on-boarding indication.

RAN2#113e agreements

- Broadcast a 1-bit indication for onboarding per O-SNPN. - R2 assumes that the 1-bit indication for onboarding is in SIB1. - The UE sends an indication for onboarding to the gNB at RRC Connection Establishment (intention to support AMF selection). - Focus on the O-SNPN scenario. Wait for SA2 further conclusion on how a PLMN can be used as onboarding network.

Proposal: UE includes on-boarding request in RRC connection establishment only if the cell broadcast 1-bit on-boarding indication.

In general, we think the work completion depends on the progress in other WGs as Credential details is FFS including the details of the interface between AUSF and AAA server. Interface may be realized by implementation specific solutions like e.g. a special PLMN UE physically attached to the AAA server and utilizing the UE’s access of PLMN as a secure tunnel for transferring the RADIUS signaling between AAA server and AUSF or alternatively allowing AUSF access from fixed IP addresses based on operator policy and probably both options can be left to operator implementation. However, details need to be discussed in SA2/3. RAN2 should discuss the need for LS to other WGs.

# Conclusion

We propose RAN2 to discuss and agree on following proposals:

Proposal: UE includes on-boarding request in RRC connection establishment only if the cell broadcast 1-bit on-boarding indication.

RAN2 to discuss coordination with SA2/3 regarding the progress of this feature.

# References

[1]	RP-202363: New WID: NR Non-Public Network enhancements

[2]	R2-210xxxx: Reply LS on clarification request for eNPN features, SA2