3GPP TSG-RAN WG2 Meeting #113bis-e                                                           R2-2103672

Online, April 12 – 20, 2021

Agenda Item	:	8.6.2

Title	:	Discussion on small data transmission

Source	:	Google Inc.

Work Item	:	NR_SmallData_INACTIVE-Core

Document for	:	Discussion and Decision

# 1		Introduction

In this tdoc, we discuss buffer status report for non-SDT DRBs and calculation of data volume for deciding SDT or non-SDT. We also propose to use logical channel priority to decide SDT or non-SDT procedure.

# 2		Discussion

In the post-113e email discussion, it was discussed whether MAC should consider suspended non-SDT DRBs in buffer status report. It was pointed out that in LTE NB-IoT, MAC may consider suspended DRBs in buffer status report. For SDT, we think it is beneficial to reuse the same approach to non-SDT DRBs because if UE can report buffer status for both DRBs, once gNB has the buffer status information, gNB can decide whether to resume RRC connection or not after receiving resume request form UE. It is also simpler because no new mechanism is needed to indicate data arrival for non-SDT DRBs.

Proposal 1: MAC should consider suspended non-SDT DRBs in buffer status report.

RAN2 agreed that a data volume threshold is used to decide SDT or non-SDT procedure. However, it is FFS how to calculate the data volume. From our understanding, the intention of data volume threshold is if UE has lot of data to transmit, it should transmit the data with link adaption. Under this assumption, the data volume could be the sum of data from both SDT and non-SDT DRBs.

Proposal 2: The data volume for deciding SDT or non-SDT procedure should the sum of data volume from both SDT and non-SDT DRBs.

RAN2 also agreed that small data transmission is configured on a per DRB basis. If UE is configured with more than one DRB, some of the DRBs may be configured with SDT and the other are not. If STD DRBs and non-SDT DRBs have data to transmit, the question is which procedure UE should perform. We think that in addition to data volume, UE should also consider logical channel priority when deciding to SDT or non-SDT procedure.

Proposal 3: Logical channel priority should be used to decide SDT or non-SDT procedure.

RAN2 agreed to support subsequent data transmission and for CG-SDT and the subsequent data transmission can use dynamic grant resource or configured grant resource. Since dynamic grants can be used in subsequent transmission, UE may need a RNTI to receive dynamic grants.

In LTE PUR, eNB configures UE with a pre-configured uplink resource and a PUR-RNTI. After transmission using PUR, UE uses the PUR-RTNI to monitor PDCCH in order to receive an uplink grant or a response from gNB. For CG-SDT, the network could configure UE with a new RNTI or UE could reuse previous C-RNTI. To avoid complexity, it may be simper to configure UE with a new RNTI.

In LTE PUR, if MAC receives L1 ACK for PUR transmission or successfully decodes a MAC PDU with the PUR-RNTI, MAC will not use PUR-RNTI to monitor PDCCH. Similarly, UE may not need to use the new RNTI to monitor PDCCH after subsequent transmission because if UE has lot of data in buffer during subsequent transmission, gNB can request the UE to enter RRC_CONNECTED.

Proposal 4: For CG-SDT, a new RNTI should be used in subsequent transmission period.

In the last meeting, a question is raised whether to ensure SDT data is included in the first message. With the support of subsequent data transmission and RLC segmentation, if the SDT data is not included in the first message, gNB still can schedule another transmission for UE to transmit the SDT data so it may not be needed to ensure SDT data is included in the first message. Instead, if UE has data pending for transmission, it should include a BSR in the first message.

Proposal 5: No need to ensure that SDT data is included in the first message.

# 3		Conclusions

In conclusion, we have the following proposals:

Proposal 1: MAC should consider suspended non-SDT DRBs in buffer status report.

Proposal 2: MAC should use the sum of data volume from both SDT and non-SDT DRBs to decide SDT or non-SDT procedure.

Proposal 3: Logical channel priority should be used to decide SDT or non-SDT procedure.

Proposal 4: For CG-SDT, a new RNTI should be used in subsequent transmission period.

Proposal 5: No need to ensure that SDT data is included in the first message.