3GPP TSG-RAN WG2 Meeting #113 bis electronic	                                     R2-2103704

Online, April 12 – April 20, 2021

Agenda Item:	8.1.3

Source:	CMCC

Title:	Discussion on delivery mode 2 remaining issues

Document for:	Discussion

Introduction

In RAN2#112e, support of MBS idle mode or delivery mode 2 was discussed, and the following was agreed [1]:

UE receives the MBS configuration (for broadcast/delivery mode 2) by BCCH and/or MCCH (TBD), and this can be received in Idle / Inactive mode. Connected mode FFS (dep on UE cap and where service is provided etc). A notification mechanism is used to announce the change of MBS Control information.

In RAN2#113, following further agreements related to delivery mode 2 were made based on the post email discussion between the two meetings.  But it’s more like a conclusive description, without detail design. And there are still some FFS related to service continuity [2]:

The two-step based approach (i.e. BCCH and MCCH) as adopted by LTE SC-PTM is reused for the transmission of PTM configuration for NR MBS delivery mode 2.

Assume it is possible to reuse LTE SC-PTM mechanism for the CONNECTED UEs to receive the PTM configuration for NR MBS delivery mode 2, i.e. broadcast based manner.

Assume that MCCH change notification mechanism is used to notify the changes of MCCH configuration due to session start for delivery mode 2 of NR MBS (other cases FFS, if any).

Assume that MBS Interest Indication is supported for UEs in connected mode for Broadcast service (assume that as usual there is no mandatory network requirement, network action is up to network).

MBS Interest Indication is NOT supported for UEs in idle/inactive mode for NR MBS delivery mode 2.

Assume that some information for purpose of service continuity can be provided for NR MBS delivery mode 2. (FFS what - need to be revisited, e.g. based on progress in other groups, e.g. USD, SAI/TMGI etc)

FFS whether support UE awareness of MBS services on frequency basis for service continuity for NR MBS delivery mode 2 (i.e. Reuse LTE SC-PTM mechanism).

FFS Support frequency prioritization during cell reselection for service continuity for NR MBS delivery mode 2 (i.e. Reuse LTE SC-PTM mechanism).

P2: Whether UEs that receive Multicast can be released to RRC Inactive / Idle and continue receiving Multicast is Postponed. Should limit to RRC inactive in future discussions.

Also, an email discussion [Post113-e][053][MBS17] was proposed, which mainly focus on the for further discussion about MCCH transmission and notification between the two meetings. In this contribution we provide our view on MCCH design and service continuity related issues.

Discussion

MCCH transmission and scheduling

In LTE SC-PTM, both RRC_Idle and RRC_Connected UEs could receive MBMS service via SC-PTM, the control information associated transmission of MBMS using SC-PTM (SC-MCCH) is provided by SIB 20 via the following parameters [3].

SystemInformationBlockType20 information element

-- ASN1START

SystemInformationBlockType20-r13 ::=	SEQUENCE {

sc-mcch-RepetitionPeriod-r13		ENUMERATED {rf2, rf4, rf8, rf16, rf32, rf64, rf128, rf256},

sc-mcch-Offset-r13				INTEGER (0..10),

sc-mcch-FirstSubframe -r13		INTEGER (0..9),

sc-mcch-duration-r13			INTEGER (2..9)	OPTIONAL,

sc-mcch-ModificationPeriod-r13	ENUMERATED {rf2, rf4, rf8, rf16, rf32, rf64, rf128, rf256,

rf512, rf1024, r2048, rf4096, rf8192, rf16384, rf32768,												rf65536},

In the previous meetings, it was agreed that the UE can receive the MBS configuration for broadcast session in RRC_IDLE and , RRC_INACTIVE and RRC_CONNECTED state. And the two-step based notification mechanism (i.e. BCCH and MCCH) is used for the transmission of PTM configuration for NR MBS in Broadcast based manner, i.e., MCCH is delivered via broadcast signalling.

As in LTE SC-PTM, the following principles govern the MCCH structure can be reused:

-	the MCCH is sent on DL-SCH;

-	the MCCH provides the list of all MBMS services with ongoing sessions transmitted on MTCH(s), (detail information is FFS)

-	MCCH is transmitted by RRC every MCCH repetition period;

-	MCCH uses a modification period;

-	MCCH notification mechanism is used to announce changes of MCCH due to Session Start.

The following principles apply to MBS signalling on BCCH:

-	BCCH points to the resources where the MCCH can be found;

-	BCCH indicates the MCCH modification period, MCCH repetition period and MCCH offset.

Proposal 1:

The following principles govern the MCCH structure, similar as in LTE SC-PTM is proposed to be agreed:

-	the MCCH is sent on DL-SCH;

-	the MCCH provides the list of all MBMS services with ongoing sessions transmitted on MTCH(s) , (detail information is FFS)

-	MCCH is transmitted by RRC every MCCH repetition period;

-	MCCH uses a modification period;

-	MCCH notification mechanism is used to announce changes of MCCH due to Session Start.

Proposal 2: The following principles apply to MBS signalling on BCCH, similar as in LTE SC-PTM is proposed to be agreed:

-	BCCH points to the resources where the MCCH can be found;

-	BCCH indicates the MCCH modification period, MCCH repetition period and MCCH offset.

Besides, in LTE, SIB 20 is new defined for SC-PTM related information, RAN2 should decide whether a new SIB is needed for delivery mode 2 PTM control information transmission.

Proposal 3: It is proposed that RAN2 need consider whether a new SIB is needed for delivery mode 2 PTM control information transmission.

In LTE SC-PTM, there was the restriction that one cell has only one SC-MCCH. And SC-RNTI with fixed value is used for the transmission of SC-MCCH message scheduling. The same principle could be reused in delivery mode 2 that a special RNTI needs to be defined for a MCCH, and considering that there could be diverse MBS services with different requirements, so there could be beneficial for UE power saving if multiple MCCH is supported, to avoid the UE monitoring and decoding the MCCH information which it is not interested.

Based on the analysis, it is proposed for RAN2 to confirm the principle that a new RNTI is defined for a MCCH, but whether the value is fixed could be further discussed based on the discussion of multiple MCCH supporting.

Proposal 4: The principle that a special RNTI is required to designated to a MCCH could be confirmed by RAN2.

Proposal 5: Whether multiple MCCH are allowed for different MTCH groups with different QoS requirement could be considered.

The most significant difference of LTE and NR is the introduction of beam-sweeping, which may have great impact on MCCH and MTCH design, for example search space, association between PDCCH occasions in MCCH search space and SSBs, etc., which are related to RAN1’s work.  In our opinion, RAN2 should provide more information for RAN1’s better understanding of MCCH, MTCH. And we provide detail discussion in the companion contribution [4].

MCCH notification

As we know, in SC-PTM, for the regular UE (Except for NB-IoT UEs, BL UEs or UEs in enhanced coverage), the change notification of the MBMS control information due to Session Start is sent in the first subframe in a Repetition Period where the SC-MCCH can be scheduled. The notification is sent using the DCI format 1C with SC-N-RNTI. When the UE receives the notification, it will acquire the updated SC-MCCH. And during email discussion, there is concern from companies on the flexibility and efficiency of such SC-PTM mechanism. For example, using MCCH increases the UE requirements to regularly check for MCCH for changes, but UEs not interested in MBS are impacted by frequent SI change notifications. Furthermore, current SC-PTM mechanism cannot avoid the UE will need to wake up and receive the updated SC-MCCH control information which it is not interested. And in the conclusion of email discussion of [Post112-e][069][MBS] Delivery mode 2, it is noted that:

- Mark the enhancement for PTM change notification as an open issue for delivery mode 2 of NR MBS.

Therefore, we tend to prefer some enhancement in the change notification of the MBMS control information, as follows:

Firstly, as mentioned above, different services with different latency requirements may require different SC-MCCH repetition period and modification period respectively. In case of such different MBS services are to be transmitted in the same cell, gNB should configure the SC-MCCH repetition period and modification period according to the service with the most stringent latency requirement. Moreover, 5G network is demanded to supply more diverse service type than LTE. Hence, in some cases, MCCH modification period is possible to be very short and update frequency will be very high resulting in extremely high overhead, if only one SC-MCCH per cell.

Based on this, we propose to introduce multiple SC-MCCHs per cell, where the short message mechanism for paging message can be adopted into the change notification of the MBMS control information, that is, utilizing the indications in DCI format with SC- RNTI to indicate some information (merging the DCI with SC-N-RNTI and the DCI with SC-RNTI into one DCI with SC-RNTI), e.g. whether the MBMS control information is changed, whether the DCI already conveyed the changed information and the resource to acquire the specific changed MCCH.

Furthermore, the DCI can indicate which MBS service group (s)’ MBMS control information is changing as well. This can be combined with the possible enhancement mentioned in email discussion that the MBS services could be grouped and the change is only notified to the involved UEs which have interest.

Proposal 6: It is proposed to merge the DCI with SC-N-RNTI and the DCI with SC-RNTI into one DCI with SC-RNTI to indicate required information, e.g., whether the MBMS control information is changed, whether the DCI already conveyed the changed information and the resource to acquire the specific changed MCCH.

Service continuity

In LTE MBMS, several ways were designed for UE’s service continuity, including:

- MBMS assistance information in USD and system information (SIB15), and the assistance information in SIB 15 is classified by the current and/ or neighbouring carrier frequencies.

- Cell list supporting one MBMS service information in SCPTMconfiguration carried by SC-MCCH.

- Frequency based prioritization during cell reselection.

For the first approach, SIB 15 could provide a holistic view of the service supporting situation on current and/or neighbouring frequency for UEs, therefore, we think it should be supported in delivery mode 2. However, to use the information carried by SIB 15, USD is necessary, since the MBMS services are indicated by SAI in SIB 15, while SAI and USD SAI defining is out of RAN2 scope, decided by SA4. Meanwhile, delivery mode 2 aims at RRC_Idle and Inactive UEs, some services information is needed as a kind of guidance anyhow. Therefore, it’s better for RAN2 to send LS to SA4 to consult the SAI and USD issues since we have already agreed in last meeting that some information for purpose of service continuity can be provided.

Proposal 7: UE is aware of MBS services on frequency basis for NR MBS delivery mode 2 like SC-PTM does for service continuity.

Proposal 8: RAN2 is kindly asked to send LS to SA4 to consult the SAI and USD related issues, so as to progress the service continuity enhancement.

What’s more, if proposal 5 is supported, the third approach frequency-based prioritization during cell reselection also be supported.

Proposal 9: Frequency based prioritization during cell reselection also be supported if UE is aware of MBS services on frequency basis.

Besides, if the MBS SIB and PTM configuration/MCCH are area specific, the UE may not need to read the MBS SIB after cell reselection and then may help to ensure better service continuity. And it may not introduce too much workload, since NR already supports area-specific SIB.

Proposal 10: Area-specific MBS SIB and MCCH could be supported.

Conclusions

In this paper, we discussed MCCH related issues including transmission, scheduling and notification and service continuity issues. Our proposals are listed below:

Proposal 1:

The following principles govern the MCCH structure, similar as in LTE SC-PTM is proposed to be agreed:

-	the MCCH is sent on DL-SCH;

-	the MCCH provides the list of all MBMS services with ongoing sessions transmitted on MTCH(s) , (detail information is FFS)

-	MCCH is transmitted by RRC every MCCH repetition period;

-	MCCH uses a modification period;

-	MCCH notification mechanism is used to announce changes of MCCH due to Session Start.

Proposal 2: The following principles apply to MBS signalling on BCCH, similar as in LTE SC-PTM is proposed to be agreed:

-	BCCH points to the resources where the MCCH can be found;

-	BCCH indicates the MCCH modification period, MCCH repetition period and MCCH offset.

Proposal 3: It is proposed that RAN2 need consider whether a new SIB is needed for delivery mode 2 PTM control information transmission.

Proposal 4: The principle that a special RNTI is required to designated to a MCCH could be confirmed by RAN2.

Proposal 5: Whether multiple MCCH are allowed for different MTCH groups with different QoS requirement could be considered.

Proposal 6: It is proposed to merge the DCI with SC-N-RNTI and the DCI with SC-RNTI into one DCI with SC-RNTI to indicate required information, e.g., whether the MBMS control information is changed, whether the DCI already conveyed the changed information and the resource to acquire the specific changed MCCH.

Proposal 7: UE is aware of MBS services on frequency basis for NR MBS delivery mode 2 like SC-PTM does for service continuity.

Proposal 8: RAN2 is kindly asked to send LS to SA4 to consult the SAI and USD related issues, so as to progress the service continuity enhancement.

Proposal 9: Frequency based prioritization during cell reselection also be supported if UE is aware of MBS services on frequency basis.

Proposal 10: Area-specific MBS SIB and MCCH could be supported.

References

- RAN2 112-e Chairman Notes 2020-11-15 EOM

- RAN2 113-e Chairman Notes 2021-02-05 EOM

- 3GPP TS 36.331 V15.8.0 (2019-12) Radio Resource Control (RRC) (Release 15)

- R2-2103705 Discussion on delivery mode 2 discussion, CMCC