3GPP TSG-RAN2 Meeting #113bis-e	R2-2103769

eMeeting, 12th – 20th April, 2021

Agenda Item:	5.4.3 UE capabilities and Capability Coordination

Source: 	Ericsson

Title:  	Open issues K0 configuration and use

Document for:	Discussion and Decision

# Introduction

The DL scheduling slot offset capabilities were discussed in email discussion #051 [1], but some topics did not conclude in that email discussion, which are discussed further in this contribution:

- K0 configuration in PDCCH-Config

- K0 use in PDCCH scheduling

# Discussion

K0 configuration in PDCCH-Config

Email discussion #051 [1] proposes to clarify that a UE that does not support dl-SchedulingOffset-PDSCH-TypeA or dl-SchedulingOffset-PDSCH-TypeB capability does support pdsch-TimeDomainAllocationList configuration in PDSCH-ConfigCommon in SIB1 including K0 values larger than 0, but the network does not use K0 > 0 for that UE.

During the email discussion it was questioned whether the configuration of K0 > 0 in PDSCH-ConfigCommon in dedicated signalling in connected mode should take into account the UE capability, i.e. network should not configure K0 > 0 for a UE that indicates to not have IOT-tested K0 > 0 (i.e. dl-SchedulingOffset-PDSCH-TypeA or dl-SchedulingOffset-PDSCH-TypeB capability set to FALSE).

First we would like to clarify that we only discuss the configuration not the use, i.e. when the UE did not IOT-test K0 > 0, then the network should not use K0 > 0 to schedule that UE:

Observation 1: The network cannot use K0 > 0 for PDCCH/PDSCH scheduling without possible IOT issues when the network does not know if the UE has IOT-tested K0 > 0.

In case the UE in idle mode supports K0 > 0 configuration independent of the UE capability, then we suspect that the UE also supports such configuration in dedicated signalling in connected mode:

Observation 2: If UEs support K0 > 0 configuration in Idle mode without having IOT-tested K0 > 0, then the UEs also support K0 > 0 configuration in connected mode without having IOT-tested K0 > 0.

However the network in connected mode typically uses the UE capabilities to configure the UE, and the network is assumed to follow the same procedure in this case:

Proposal 1: Clarify in chairman notes that network configures K0 in dedicated signalling according to the UE capabilities.

K0 use in PDCCH scheduling

During email discussion #051 [1] it was discussed that the network cannot use K0 > 0 in PDCCH scheduling when the network does not know if the UE has IOT-tested K0 > 0 (i.e. the network does not know the UEs dl-SchedulingOffset-PDSCH-TypeA and dl-SchedulingOffset-PDSCH-TypeB capabilities).

Observation 3: The network cannot use K0 > 0 for System Information and Contention Based Random Access (CBRA) for UEs in Idle and Inactive mode.

The network could use K0 > 0 during Contention Free Random Access (CFRA) and for DL data in connected mode for UEs that have IOT-tested the feature and set the capability to TRUE.

Proposal 2: Clarify in chairman notes that the network cannot use K0>0 for PDCCH/PDSCH scheduling without possible IOT issues when the network does not know if the UE has IOT-tested K0>0.

# Summary

RAN2 is kindly asked to discuss K0 configuration:

Observation 1: The network cannot use K0 > 0 for PDCCH/PDSCH scheduling without possible IOT issues when the network does not know if the UE has IOT-tested K0 > 0.

Observation 2: If UEs support K0 > 0 configuration in Idle mode without having IOT-tested K0 > 0, then the UEs also support K0 > 0 configuration in connected mode without having IOT-tested K0 > 0.

Observation 3: The network cannot use K0 > 0 for System Information and Contention Based Random Access (CBRA) for UEs in Idle and Inactive mode.

Proposal 1: Clarify in chairman notes that network may configure K0 in both idle and connected mode, independent of the UE capability.

Proposal 1: Clarify in chairman notes that network configures K0 in dedicated signalling according to the UE capabilities.

# References

- R2-2103769, Summary of [Post113-e][051][NR15] DL scheduling slot offset, Ericsson, Report, RAN2#113bis-e