3GPP RAN WG2 Meeting #113bis-e	R2-2103968

eMeeting April 12th – April 20th, 2021

Agenda Item:	8.10.2

Source:	InterDigital

Title:	MAC open issues

Document for:	Discussion, Decision

# Introduction

The following RAN2 user plane agreements have been capture in NTN so far [1] [2] [5]:

## RAN2#113-e Agreements

Both Type 1 and Type 2 configured grant are feasible in NTN. From RAN2’s perspective, no need to modify parameter periodicity of IE ConfiguredGrantConfig to support NTN. No need to modify maxNrofConfiguredGrantConfig-r16 and maxNrofConfiguredGrantConfigMAC-r16 to support NTN. UE in NTN can have both 2-step RACH and configured grant configurations at the same time. For HARQ processes with DL HARQ feedback disabled, drx-HARQ-RTT-TimerDL is not started. FFS: method(s) to support blind retransmission for HARQ processes with HARQ feedback disabled. From RAN2 perspective, for HARQ processes where gNB can sends UL grant without waiting for decoding result of previous PUSCH transmission, no new network scheduling restrictions are introduced to schedule subsequent grants (i.e. up to network implementation. (Can come back if we don't find an agreement on p8) For HARQ processes with DL HARQ feedback enabled, drx-HARQ-RTT-TimerDL length is increased by offset (i.e. existing values within value range increased by offset). RAN2 working assumption: offset is equal to UE-gNB RTT (if RAN1 decides something that requires to change this we can revisit it)

## RAN2#112-e Agreements

RAN2 working assumption (for RRC idle. FFS for Inactive/Connected): Rel-17 UE with pre-compensation capability obtains UE specific UE-gNB RTT based on its GNSS in LEO/GEO. FFS how this is calculated and what/if anything needs to be broadcasted for the different pre-compensation methods (e.g. common TA) to help the UE to obtain the full UE-gNB RTT. If the UE-gNB RTT is pre-compensated, preamble ambiguity is not an issue in Rel-17 NTN (i.e. no enhancements are necessary). FFS how and by whom the possibly multiple components of UE-gNB RTT are pre-compensated From RAN2 perspective, for UE with UE-specific pre-compensation as a baseline it is up to gNB implementation to ensure sufficient time on UE side for the Msg3 transmission. For UE with pre-compensation capability (at least for the HARQ-feedback enabled case. FFS for HARQ-feedback disabled, if supported), drx-HARQ-RTT-TimerDL is offset by UE-specific RTT (UE-gNB delay) in LEO/GEO. FFS if offset is applied to: 1) the start of the timers or 2) the timer value range (i.e. existing values within value range increased by offset) From RAN2 perspective, for dynamic grant, one possibility for "enabling"/"disabling" HARQ uplink retransmission at UE transmitter is without introducing an additional mechanism (i.e. gNB can send grant with NDI not toggled/toggled without waiting for decoding result of previous PUSCH transmission). FFS on the handling of RTT timers. Other solutions for enabling/disabling HARQ UL reTX are not precluded If the start of the ra-ResponseWindow and msgB-ResponseWindow is accurately compensated by UE-gNB RTT, ra-ResponseWindow and msgB-ResponseWindow are not extended in LEO/GEO. At least the following are FFS in Rel-17 NTN: - Report UE-calculated TA in e.g. msg3/msg5/msgA - Enhancements to RSRP-based selection mechanism of 2-step vs. 4-step RACH - LCP impact caused by disabling HARQ UL retransmission RAN2 decision on starting ra-ContentionResolutionTimer, ra-ResponseWindow and msgB-ResponseWindow is postponed until further progress in RAN1 regarding UE pre-compensation method and TA estimation accuracy.

## RAN2#111-e Agreements

From RAN2 perspective, an offset is applied to the start of ra-ResponseWindow in NTN for both LEO and GEO scenarios. An offset to the start of the ra-ContentionResolutionTimer is introduced for both LEO and GEO scenarios. Modification of drx-LongCycleStartOffset, drx-StartOffset, drx-ShortCycle, drx-ShortCycleTimer, drx-onDurationTimer, drx-SlotOffset and drx-InactivityTimer is not needed in Rel-17 NTN. From a RAN2 perspective, for DL, HARQ feedback can be enabled/disabled in Rel-17 NTN, but HARQ processes remain configured. The criteria and decision to enable/disable HARQ feedback is under network control and is signalled to the UE via RRC in a semi-static manner. FFS for UL At least the following methods to enhance UL scheduling are further studied in NTN: configured grant and BSR over 2-step RACH. (other solutions to enhance UL scheduling are not precluded) Both 2-step and 4-step RACH are supported in Rel-17 NTN. FFS enhancements to RACH to accommodate the NTN environment.

This contribution lists remaining issues identified by RAN2 and the NTN MAC CR rapporteur.

# Open Issues

## Pre-compensation

Open Issue 1: Method of TA calculation and TA value

RAN1 is currently discussing the method of time/frequency compensation in NTN. In RAN1#103e, [3] it was agreed:

An NTN UE in RRC_IDLE and RRC_INACTIVE states is required to at least support UE specific TA calculation based at least on its GNSS-acquired position and the serving satellite ephemeris.

However, it was further noted at this stage that:

Note: UE will not assume that the RTT between UE and gNB is equal to the calculated TA for Msg1/Msg A.

Definition of the TA calculation methodology and what this value represents must be completed prior to confirmation of RAN2 working assumption “Rel-17 UE with pre-compensation capability obtains UE specific UE-gNB RTT based on its GNSS in LEO/GEO” and specification of RAN2 timer offsets (e.g. open issues 4 and 6)

Open Issue 2: Additional signalling to obtain UE-gNB RTT

In RAN1#103e [3], it was agreed that prior to Msg1/MsgA, the UE calculates its TA via the UE-specific TA and at least a common timing offset X value if broadcast by network. However, the granularity of X, and whether X is indicated as a Timing Advance or as a Timing Offset value [unit] are FFS.

Definition of the common timing offset must be completed prior to specification of RAN2 timer offsets (e.g. open issues 4 and 6) to ensure UE can determine UE-gNB RTT.

Open Issue 3: Accuracy of timing offset

Given the possible sources of error in offset delay estimation (e.g. atmospheric drag), it was agreed in RAN1#102e [4]:

FFS: The TA margin, if needed and indicated by the network (in order to account for the TA estimation uncertainty)

Analysis of offset estimation accuracy and definition of TA margin (if necessary) must be completed prior to specification of RAN2 timer offsets (e.g. open issues 4 and 6).

## RACH aspects

Open Issue 4: RA timers offset and extension

In RAN2#111e [2] it was agreed that an offset be applied to the start of the ra-ResponseWindow (msgB-ReponseWindow) and ra-ContentionResolutionTimer. It was further agreed in RAN2#112e [1] that if ra-ResponseWindow and msgB-ReponseWindow are accurately compensated by UE-gNB RTT, an extension of the timer duration is not necessary.

However as described above, definition and analysis of TA estimation method, accuracy, and additional signalling must be completed by RAN1 (i.e. a resolution to open issues 1-3) prior to specification of start offset to ra-ContentionResolutionTimer, ra-ResponseWindow and msgB-ResponseWindow, and confirmation that an extension to ra-ResponseWindow and msgB-ResponseWindow is not required.

Open issue 5: FFS: RACH enhancements

The following have been captured as possible areas for enhancement for RACH in NTN:

- Report of UE-specific TA in e.g. msg3/msg5/msgA;

- Enhancements to RSRP-based selection mechanism of 2-step vs. 4-step RACH.

- Enhancements to RACH to accommodate the NTN environment.

The above aspects are being addressed in ongoing email discussion [Post113-e][106][NTN] MAC aspects [6]

## HARQ

### DL HARQ feedback

Open Issue 6: Offset value of drx-HARQ-RTT-TimerDL (when DL HARQ feedback is enabled)

From RAN2#113e, RAN2 agreed to the following working regarding offset to drx-HARQ-RTT-TimerDL for UE with pre-compensation capability and for HARQ processes where DL HARQ feedback is enabled [5]:

RAN2 working assumption: offset is equal to UE-gNB RTT (if RAN1 decides something that requires to change this we can revisit it)

Like RA timers, definition and analysis of TA estimation method, accuracy, and additional signalling must be completed by RAN1 (i.e. a resolution to open issues 1-3) prior to confirmation of working assumption and spec implementation.

Open Issue 7: Support for blind retransmission when HARQ feedback disabled

The following has been captured as FFS in RAN2#113e:

- FFS: method(s) to support blind retransmission for HARQ processes with HARQ feedback disabled

Open Issue 8: How to capture which HARQ processes are disabled.

Based on comments received in [Post113-e][105][NTN] [7], how to indicate which HARQ processes are disabled could either be via a bitmap, or list.

Current implementation in specification is captured as list, however RAN2 to confirm.

### UL HARQ retransmission

Open Issue 9: drx-HARQ-RTT-TimerUL if UL HARQ retransmission is “enabled”

From RAN2#113e [5], it was agreed that for HARQ processes with DL HARQ feedback enabled, drx-HARQ-RTT-TimerDL length is increased by offset (i.e. existing values within value range increased by offset). Though discussed, no agreement has been made regarding behaviour of drx-HARQ-RTT-TimerUL when UL HARQ retransmission is “enabled”.

RAN2 to confirm if UE-gNB RTT offset is also applicable to drx-HARQ-RTT-TimerUL when HARQ UL retransmission is enabled as in the case for DL or define alternative behaviour.

Open Issue 10: Operation of drx-HARQ-RTT-TimerUL if UL HARQ retransmission is “disabled”

In RAN2#112e [1], it was agreed that for dynamic grant, HARQ uplink retransmission may be “disabled” by gNB can send grant with NDI not toggled/toggled without waiting for decoding result of previous PUSCH transmission. However, handling of RTT timers (specifically drx-HARQ-RTT-TimerUL) is listed as FFS, and discussion during RAN2#113e was inconclusive.

RAN2 to confirm behaviour of drx-HARQ-RTT-TimerUL when HARQ UL retransmission is “disabled”

Open Issue 11: FFS: LCP impact caused by disabling HARQ UL retransmission

The following have been captured as FFS:

- LCP impact caused by disabling HARQ UL retransmission

# Conclusions

Based on the above open issues, MAC CR rapporteur proposes the following way forward on open issues:

Observation	RAN2 cannot progress on open issues 1-4 and 6 (i.e. issues regarding TA pre-compensation method, value, and accuracy) until further progress in RAN1 on pre-compensation.

Proposal 1	RAN2 to prioritize progress on open issues 8 to 10.

Proposal 2 	RAN2 sends an LS to RAN1 to kindly ask them to prioritize the topic of pre-compensation, also indicating RAN2's dependency on RAN1's progress.

Proposal 3	RAN2 can progress on issues 5, 7, and 11 as a second priority.

# References

R2-2100001 Report of 3GPP TSG RAN2#112-e meeting – ETSI MCC

R2-2008701 Report of 3GPP TSG RAN2#111-e meeting – ETSI MCC

Draft_Minutes_report_RAN1#103-e_v020

Final_Minutes_report_RAN1#102-e_v100

Draft_RAN2_113-e_Meeting_Report_v1

[Post113-e][106][NTN] MAC aspects (Huawei)

[Post113-e][105][NTN] MAC Running CR (InterDigital)