3GPP RAN WG2 Meeting #113bis-e	R2-2103975

eMeeting April 12th – April 20th, 2021

Agenda Item:	8.9.2

Source:	InterDigital

Title:	UE grouping-based paging enhancement

Document for:	Discussion, Decision

# Introduction

In RP#88e a new WI was approved on Rel-17 UE power saving enhancements. The following objectives have been identified [1]:

- Specify enhancements for idle/inactive-mode UE power saving, considering system performance aspects [RAN2, RAN1]

  - Study and specify paging enhancement(s) to reduce unnecessary UE paging receptions, subject to no impact to legacy UEs [RAN2, RAN1]

Addressing the above objective, in RAN2#112e it was confirmed that UE grouping is considered as a candidate paging enhancement [2]:

=> Confirm that UE grouping is considered a candidate of paging enhancement for UE power saving

Candidate options for UE grouping were subsequently discussed in email discussion [Post112-e][064][Pow17] Group Determination [3]. Among candidate options UE ID-based grouping obtained near consensus support, however, there was significant interest in additional enhancements incorporating paging probability and CN/RAN paging differentiation.

Although heavily discussed in RAN2#113e, no agreement was reached on UE grouping and final decision is expected in RAN2#113bis-e. This contribution discusses views on UE grouping methods under consideration.

# Discussion

As summarized in [3], UE ID-based subgrouping consists of sub-dividing UEs within the same paging occasion (PO) into different groups based on the UE ID. By randomly distributing UEs into subgroups, the number of false alarms can be reduced, especially for POs with many UEs. The key benefits of this solution are that it is simple, would fit into the existing paging framework, and impact to legacy UEs could be minimized as paging group can be identified e.g. using reserve bits in DCI. Considering minimal specification impact and overwhelming support in [3], UE-ID based subgrouping should be adopted as a Rel-17 power saving enhancement.

Proposal 1:	UE ID-based subgrouping is supported as paging enhancement for Rel-17.

One potential drawback of UE ID-based subgrouping is that the grouping is performed randomly and could possibly be enhanced by additional information such as paging probability. However, properly allocating UEs to subgroups requires accurate and up-to-date knowledge regarding paging probability. Given the diversity of traffic profiles and device types in NR as compared to NB-IoT/eMTC this may not be trivial, and frequent variation of these probabilities would result in additional UE overhead.

An alternative solution is to indicate whether paging message originates from CN/RAN. Considering only UEs in RRC Inactive state receive RAN paging, if a PO or UE ID-based subgroup contains both RRC Inactive and Idle UEs, and paging message indicates it only contains RAN paging, RRC Idle UEs could skip the paging message. Legacy UEs would also not be impacted if reserve bits in DCI are used for indication. However, considering this paging enhancement would only provide benefit a subset of UEs, unless the gains over UE-ID based subgrouping are significant it seems unnecessary to include at this time.

Proposal 2:	Other UE subgrouping methods are not supported as paging enhancement in Rel-17.

# Conclusion

In this contribution the following proposals were made concerning UE subgrouping method for Rel-17 UE power saving paging enhancement:

Proposal 1:	UE ID-based subgrouping is supported as paging enhancement for Rel-17.

Proposal 2:	Other UE subgrouping methods are not supported as paging enhancement in Rel-17.

# References

RP-200938 - 3GPP Work Item Description: UE Power Saving Enhancements

R2-2100001 – Report of 3GPP TSD RAN2#112-e meeting – ETSI MCC

R2-2100389 – Report of [Post112-e][064][Pow17] Group Determination – Intel Corporation