3GPP TSG-RAN WG2 Meeting #113-bis-e		R2-2104129

Online, April 12th – April 20th 2021

Source:	vivo

Title:          UL and DL+UL NR positioning in RRC_INACTIVE state

Agenda Item:	8.11.3

Document for:	Discussion and Decision

- Introduction

During RAN2#113-e[1], the following agreement has been made regarding positioning in IDLE/INACTIVE:

Agreements: Proposal 1a: RAN2 confirms on the following recommendation of TSG RAN (17/17)  Positioning in RRC_INACTIVE  DL, UL and DL+UL positioning methods  UE-based and UE-assisted positioning solutions  Support of UE positioning measurements for UEs in RRC_INACTIVE state  Options that can be considered include DL-PRS or DL-PRS and SSB  Support of gNB positioning measurements for UEs in RRC_INACTIVE state Proposal 1b: RAN2 confirms on the following (17/17)  Positioning in RRC_IDLE  It is feasible for a UE to perform DL positioning measurement in RRC_IDLE state  It is up to RAN2 to decide whether to support the enhancements of NR positioning reporting of DL positioning measurements and/or positioning estimates for RRC_IDLE UEs. Proposal2: RAN2 recommends the following for normative work for DL positioning  The report of PRS measurement performed in RRC_IDLE/INACTIVE when the UE is in RRC_INACTIVE is supported (10/12)  PRS measurement report and/or location estimate are sent from the UE to the gNB in RRC_INACTIVE. RAN2 generally agree to do this by enhancing small data transmission in RRC_INACTIVE (details of the use of SDT to be studied in the WI phase) (15/16) Proposal4: For DL positioning in IDLE/INACTIVE, the followings are already supported for the current spec and can be reused:  Current stage3 spec has already supported assistance data delivery for DL positioning during RRC_CONNECTED and on-demand SI request in RRC_IDLE/ INACITVE for IDLE/INACTIVE positioning. (14/14)  Current stage3 spec already supports the transfer of RequestLocationInformation in RRC_CONNECTED for PRS measurement in IDLE/INACTIVE. (14/14) Proposal5: Support RAT-Independent positioning in RRC_IDLE/INACTIVE. FFS the procedures that can be supported. (13/14)

At the RAN#91e meeting, the study item on NR Positioning Enhancements RP-210903[2] was approved. From RAN2’s perspective, the SI includes the following objective:

- Specify methods, measurements, signalling and procedures to support positioning for UEs in RRC_ INACTIVE state, for UE-based and UE-assisted positioning solutions, including [RAN2, RAN1, RAN3,RAN4]: - DL NR positioning methods and RAT-independent positioning methods - Support of UE positioning measurements for UEs in RRC_INACTIVE state - Reporting of positioning measurement or location estimate performed in RRC_INACTIVE when the UE is in RRC_INACTIVE state Note: this work will be coordinated with the SDT WI. - As 2nd priority: - UL and DL+UL NR positioning methods - Support of gNB positioning measurements for UEs in RRC_INACTIVE state

In this contribution, we will discuss some issues on inactive positioning from the perspective of RAN2.

- UL Positioning in RRC_INACITVE State

  - UL positioning configuration

UL positioning in inactive states should also be supported. In order to reduce unnecessary workload, we propose to study the uplink positioning in the inactive state based on the uplink positioning framework in the connected state. Therefore, in the inactive state, we should prioritize research on SRS for positioning. So, we propose

Proposal 1: Support SRS for positioning as UL positioning RS in inactive state.

According to the NR design, dedicated SRS is only allocated to UE in connected state. Hence, in current specification, UL positioning is only applicable to UE in connected state. In inactive state, UL positioning can be applied to UE if the UE’s dedicated SRS configuration is not released after the UE leaving connected mode but still in an area that is valid predefined by the network. With this enhancement, an inactive state UE could transmit dedicated SRS, and then the network could measure SRS and obtain the position of the UE. The dedicated SRS configuration can be transmitted to the UE via RRC release with suspend config when UE is in connected state. The predefined area can be the cell in which RRC Release is received or a cell list, where the SRS configuration is relatively stable without large update and does not need to be updated frequently. When the UE leaves this area, the UE should initiate random access to enter the connected state. Correspondingly, the UE and gNB need to release the previously configured SRS applied in inactive state.

We think there may be some concerns about this enhancement. For each inactive UE requiring positioning, SRS configuration needs to be reserved in both the camping and neighbouring cells. This may cause a lot of resource occupation, especially when there are many UEs performing UL positioning in the inactive state. Will this enhancement lead to a very limited capability for positioning? In our understanding, the number of positioning UE can be supported is related to the period of SRS allocated to each UE. The longer the period is, the more UEs can be supported. In some popular applications, the interval between consecutive positioning requests is very long, e.g. 1 minute. Accordingly, the period of positioning SRS resource for each UE can be very long. Therefore, the number of positioning UEs can be supported seems not a big issue. We believe these applications are the main potential motivation for UL positioning inactive state. So, long-period SRS can be applied for uplink positioning in inactive state, e.g. period>=160ms.

Proposal 2: Support to transmit SRS configuration via RRC release when UE is in connected state for UL positioning in inactive states.

- SRS configuration is valid in a predefined area, where the SRS configuration is relatively stable without large update and does not need to be updated frequently.

- The predefined area can be the cell in which RRC release is received or a cell list.

Proposal 3: Support to configure long-period SRS for UL positioning in inactive state.

Due to the mobility of the UE in inactive state, the part or all of the SRS configuration may become invalid. In addition to the predefined area, the other validity criteria should also be considered for SRS configuration in inactive state. For example, the RSRP of the pathloss RS or the spatial relation RS measured by the UE has changed a lot, causing the pathloss RS or the spatial relation RS previously configured to be no longer available; or, the timing of UE transmitting SRS changes a lot, e.g., due to camping cell change or TA command update; or, some other event causing the pre-configured SRS configuration is no longer valid. Therefore, we think RAN2 should further study the validity criteria for SRS configuration in inactive state.

Proposal 4: The validity criteria for SRS configuration in inactive state should be considered by RAN2.

In addition, in order to reduce UE power consumption, we hope that the UE enters the connected state as little as possible during the UL positioning procedure. For example, when a small part of the SRS configuration becomes invalid or a small part of the SRS configuration needs to be updated, the UE may not enter the connected state to receive the updated SRS configuration. A simple way to update small SRS configuration in inactive state is through the paging message,. Thus, we think transmission of the SRS configuration while retaining the UE in inactive state should be supported.

Proposal 5: Transmission of the SRS configuration while retaining the UE in inactive state should be supported.

Moreover, the fallback behavior should also be considered when the validity criteria for SRS configuration in inactive state is not met. One way is to enable the UE to enter the connected state and perform positioning or update the SRS configuration in the connected state. Moreover, considering that positioning report in the inactive state has been supported, when the validity criteria for SRS configuration is not met, the UE can remain in the inactive state and use the similar method as the positioning report (e.g., via SDT) to indicate the event causing validity criteria not met and require updated SRS configuration; and then the gNB can deliver the updated SRS configuration to the UE without entering connected state.

Proposal 6: The fallback behavior should be considered when the validity criteria for SRS configuration in inactive state is not met, e.g., including:

- Entering connected state to perform UL positioning or update the SRS configuration

- Remaining in inactive state to perform UL positioning and receive updated SRS configuration

  - Transmission procedure for UL positioning RS

In this section, we focus on transmission procedure of SRS in inactive state, including: SRS spatial relation, SRS power control and SRS transmission timing.

Regarding SRS spatial relation in inactive state, there may be two ways that can be applied to SRS transmission. One way is to pre-configure SRS spatial relation with another RS, such as SSB or PRS. However, this way needs to consider the problem of spatial relationship failure caused by UE movement. Another way is to enable SRS beam sweeping in inactive state. In this way, there is no need to consider maintaining the spatial relationship between the UE and gNB. In addition, considering that in UL positioning, all gNBs around the UE need to measure SRS, SRS beam sweeping is a suitable way for SRS transmission. At the same time, in order to improve coverage, SRS repetition during beam sweeping may also be considered.

For SRS power control, the UE can be configured with power control related parameters when it leaving connected state. For example, for one SRS resource set, the UE can be indicated with a set of SSB, and then it selects a certain SSB whose measurement result is larger than the threshold as the pathloss RS. If the UE determines that the UE is not able to accurately measure the pre-configured pathloss RS, the UE calculates pathloss using a RS resource obtained from the SS/PBCH block of the cell that the UE uses to obtain MIB, e.g. the camping cell.

For UL positioning in inactive state, the SRS transmission timing can be changed depending on the change of DL timing of the camping cell and related TA adjustment. However, in inactive state, SRS transmission timing cannot be adjusted in time as the connected state. This makes the inactive UE not synchronized in uplink, causing interference to other UEs which are synchronized in connected mode. In our understanding, the positioning SRS is monitored by more than one gNB. However, one UE can at most maintain uplink synchronization with one of the gNBs (i.e. the serving gNB of a connected UE). It implies even connected UE maybe unsynchronized with some gNBs monitoring its positioning SRS according the current UL positioning design. Hence, uplink un-synchronization seems not a new issue introduced by allowing inactive mode UE to send dedicated positioning SRS.

In short, there is still a difference between the design of the transmission procedure of SRS in inactive state and the connection state. We think the related work does not belong to the scope of RAN2. So, we propose

Proposal 7: The details of SRS spatial relation, SRS power control, and SRS transmission timing for SRS transmission in inactive state should be further studied by RAN1.

- DL and UL positioning in RRC_INACITVE state

For DL and UL positioning method, the UE not only needs to measure downlink signals from TRPs and report Rx-Tx measurements, but also transmits an uplink signal to the same TRPs. It can be considered as a combination of downlink only positioning method and uplink only positioning method. So that if DL only positioning method and UL only positioning method are supported, DL and UL positioning method is naturally supported. Therefore, we propose to give priority to the work on separate DL positioning and separate UL positioning, and then work on the DL and UL positioning. prioritize

Proposal 8: We should prioritize the work on separate DL positioning and separate UL positioning in inactive state, and then work on the DL and UL positioning in inactive state.

- Conclusion

Based on the discussion, we propose the following:

Proposal 1: Support SRS for positioning as UL positioning RS in inactive state.

Proposal 2: Support to transmit SRS configuration via RRC release when UE is in connected state for UL positioning in inactive states.

- SRS configuration is valid in a predefined area, where the SRS configuration is relatively stable without large update and does not need to be updated frequently.

- The predefined area can be the cell in which RRC release is received or a cell list.

Proposal 3: Support to configure long-period SRS for UL positioning in inactive state.

Proposal 4: The validity criteria for SRS configuration in inactive state should be considered by RAN2.

Proposal 5: Transmission of the SRS configuration while retaining the UE in inactive state should be supported.

Proposal 6: The fallback behavior should be considered when the validity criteria for SRS configuration in inactive state is not met, e.g., including:

- Entering connected state to perform UL positioning or update the SRS configuration

Remaining in inactive state to perform UL positioning and receive updated SRS configuration

Proposal 7: The details of SRS spatial relation, SRS power control, and SRS transmission timing for SRS transmission in inactive state should be further studied by RAN1.

Proposal 8: We should prioritize the work on separate DL positioning and separate UL positioning in inactive state, and then work on the DL and UL positioning in inactive state.

- Reference

- RAN2-113-e-Positioning-Relay-2021-02-04-0630

- RP-210903-(rev.fr.RP-210897)-clean

- RP-201305, Work Item on NR smalldata transmissions in INACTIVE state