3GPP TSG-RAN WG2 Meeting #113bis-e 	R2-2104133

Online, April 12 – April 20 2021

Agenda Item:		8.17

Source:				Huawei, HiSilicon

Title:		Discussion on RAN4 LS on handover with PSCell

Document for:	Discussion and Decision

- Introduction

From RAN2 perspective, the following HO scenarios are supported since Rel-15/Rel-16 as indicated in annex B in TS37.340. Currently the RRM requirements for the cases MR-DC configured by target in HO command are discussed in RAN4, and they asked some questions related to RAN2 defined signalling/procedure in LS R4-2103674.

| To (column) HO from (row) | E-UTRA with EPC | E-UTRA with 5GC | NR | GERAN or UTRAN | EN-DC | NGEN-DC | NE-DC | NR-DC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA with EPC | YES | YES | YES | YES | YES | NO | NO | NO |
| E-UTRA with 5GC | YES | YES | YES | NO | NO | YES | NO | NO |
| NR | YES | YES | YES | NOTE 1 | YES | NO | YES | YES |
| GERAN or UTRAN | YES | NO | NO | YES | NO | NO | NO | NO |
| EN-DC | YES | YES | YES | YES | YES | NO | NO | NO |
| NGEN-DC | YES | YES | YES | NO | NO | YES | NO | NO |
| NE-DC | YES | YES | YES | NOTE 1 | NO | NO | YES | NO |
| NR-DC | YES | YES | YES | NOTE 1 | NO | NO | NO | YES |

In this contribution, we provide our views on the RAN4 questions.

- Discussion

2.1 Q1 on RRC processing delay

In the LS, RAN4 is asking what is the RRC processing delay for following cases of handover with PSCell configured in target side?

| Scenario | Source PCell | Target PCell | Target PSCell | RRC procedure delay for HO with PSCell |
| --- | --- | --- | --- | --- |
| NR SA to EN-DC | NR (incl. FR1 and FR2) | LTE | NR (incl. FR1 and FR2) | ? |
| EN-DC to EN-DC | LTE | LTE | NR (incl. FR1 and FR2) | ? |
| NE-DC to NE-DC | NR FR1 | NR FR1 | LTE | ? |
| NR-DC to NR-DC | NR FR1 | NR FR1 | NR FR2 | ? |

Our interpretation of the RAN4 question is from two aspects: RRC message processing time and the whole procedure delay/performance.

RRC message processing time:

- In LTE and NR RRC specification, the processing delay requirement for each DL RRC message is specified. For instance, for intra-LTE mobility with NR SCG establishment/ /modification/release the processing delay of LTE RRC connection re-configuration message (e.g. the one RRC reconfiguration message indicating both of E-UTRA PCell HO and NR PSCell addition/change) is 20ms. In TS 38.331 the RRC processing delay requirement for RRC reconfiguration with SCG establishment/ modification/ release is 16ms. Our understanding this is only for SN mobility but not cover the cases of intra-NR HO with SN change wherein the UE will require more delay time than 16ms, e.g. 20ms. So we would like RAN2 to first specify the RRC processing delay requirement for HO to NR-DC/NE-DC, and then inform RAN4. And for inter-RAT HO to EN-DC the performance procedure should be specified in RAN4 spec, including the RRC processing delay.

Procedure delay/performance:

- During the intra-RAT of inter-RAT HO when target configuring MR-DC operation directly in HO command, the whole procedure delay includes HO command processing time after reception of the HO command in source side, and the latency due to RACH procedures towards PCell and PSCell, and may also involve RF/baseband processing time and so on. The procedure performance including each delay component should be specified by RAN4.

Proposal 1: RAN2 to specify the processing delay requirement of RRC reconfiguration message for intra-NR mobility with NR/E-UTRA SCG establishment/modification/release (i.e. HO to NR-DC/NE-DC) as 20ms.

Proposal 2: RAN2 to reply RAN4 on Q1 that for HO with PSCell configured in target side,

- For intra-RAT HO to EN-DC, the RRC processing delay specified in TS 36.331 is 20ms, and for the intra-RAT HO to NR-DC or NE-DC listed in annex B in TS37.340, the RRC processing delay specified in TS 38.331 is 20ms (if P1 is agreed);

- For inter-RAT HO to EN-DC, the RRC processing delay could be defined by RAN4;

- For the delay components other than RRC processing delay within the whole HO procedure could be defined by RAN4 as well.

2.2 Q2 on RACH procedures toward to PCell and PSCell

As we discussed above, if target configure MR-DC operation directly in HO command, then the UE needs to perform two separate RACH procedure on PSCell and PSCell in the target side. On this issue, RAN4 listed the following 4 options to ask RAN2 to clarify.

Question 2: Regarding HO with PSCell triggered by single RRC HO command, which of following options is in line with RAN2 definition when UE fails to synchronize to the expected PSCell? - Option 1: UE performs conventional Rel-15 HO procedure and PSCell addition separately, i.e., UE can handover to the new PCell without PSCell addition - Option 2: UE tries to synchronize another SCG which is the most likely to connect successfully (assumes that the target PCell configures multiple SCGs), i.e., UE can handover to the new PCell with a different PSCell addition - Option 3: UE won’t handover to new PCell upon PSCell addition failure, i.e., UE will treat it as conventional Rel-15 HO failure - Option 4: RAN2 is welcomed to share additional failure cases if any.

In our understanding, according to RAN2 specifications there is no explicit requirement on which RACH procedure should be performed first or two at parallel which may depend on UE capability. For instance, the following cases are allowed/possible:

- In case the UE accesses PCell and PSCell successfully, the UE will response a HO complete message (containing SCG RRC connection reconfiguration complete message) to the target MN, then MN will forward the SCG RRC connection reconfiguration complete message to SN.

- Otherwise, if the UE experiences an access failure on PCell, it would be a HO failure as defined in Rel-15/16. In this case RRC reestablishment procedure would be initiated by the UE, which means no need to access PSCell anymore.

- If the UE accesses PCell successfully but fails to access PSCell, then the UE can report SCG failure information to MN as defined in Rel-15. In this case, whether the UE needs to wait for the outcome if accessing PSCell succeed or not, our impression is no. The UE can send the HO complete message (contained SCG RRC connection reconfiguration complete message) to MN first, and after that if there is a PSCell access failure, the SCG failure information could be reported as usual.

If above is RAN2’s common understanding, the option1 from RAN4 is feasible, while option 3 is not aligned with RAN2 defined procedure. For option 2, it seems implying the UE can chose a different PSCell without network configuration, which is not supported by RAN2 clearly.

Proposal 3: RAN2 to reply RAN4 on Q2 that option1 is possible, while option2 and option3 are not aligned with RAN2 specifications.

- Conclusions

In this contribution, we discuss RAN4 questions on HO with PSCell in LS R4-2103674, and give our proposals on how to reply RAN4 LS as below:

Proposal 1: RAN2 to specify the processing delay requirement of RRC reconfiguration message for intra-NR mobility with NR/E-UTRA SCG establishment/modification/release (i.e. HO to NR-DC/NE-DC) as 20ms.

Proposal 2: RAN2 to reply RAN4 on Q1 that for HO with PSCell configured in target side,

- For intra-RAT HO to EN-DC, the RRC processing delay specified in TS 36.331 is 20ms, and for the intra-RAT HO to NR-DC or NE-DC listed in annex B in TS37.340, the RRC processing delay specified in TS 38.331 is 20ms (if P1 is agreed);

- For inter-RAT HO to EN-DC, the RRC processing delay could be defined by RAN4;

- For the delay components other than RRC processing delay within the whole HO procedure could be defined by RAN4 as well.

Proposal 3: RAN2 to reply RAN4 on Q2 that option1 is possible, while option2 and option3 are not aligned with RAN2 specifications.

- Reference