3GPP TSG-RAN WG2 #113-e	R2-2104141

E-meeting, Apr 12 – Apr 20, 2021

Agenda Item	: 8.10.2.1 (NR_NTN_solutions-Core)

Source	: LG Electronics Inc.

Title	: Discussion on RA type selection and TA report

Document for	: Discussion and Decision

Introduction

In POST113e#106 email discussion, the MAC aspects were discussed and the following proposals are made.

RA type selection: Proposal 1: RAN2 to further discuss whether and how to introduce new criteria for RA type selection: Option 1: based on “UE specific UE-satellite RTT” or “distance between UE and satellite”; Option 2: based on delay QoS requirement of logic channel; Option 3: only use RSRP as in legacy. Proposal 2: if new criteria is agreed, it works in combination with legacy RSRP threshold. Proposal 3: if new criteria based on delay QoS requirement of logic channel is agreed, further discuss how to implement it, e.g. a new configuration to allow or prevent LCH to use 2-step RA. Proposal 4: not to introduce additional RA type switching mechanism. TA report: Proposal 5: RAN2 to discuss which way to go: Option 1: UE reports User specific TA (NTA as defined by RAN1) to network. RAN2 can revisit it if RAN1 agrees to assistance information other than User specific TA; Option 2: postpone the reporting of TA or position until RAN1 have concluded. Proposal 6: RAN2 to further discuss how to trigger TA report: Option 1: TA report can be triggered when RACH is initiated, and whether TA report is included in MSG3/MSG5 or MSGA/next UL Grant following MSGB depends on existing LCP procedure; Option 2: TA report can be triggered by some event or rule, e.g. If the difference between the current TA used by the UE and the TA value known to gNB (=the value last reported by the UE) exceeds a threshold; or by a threshold/hysteresis in the UE (the threshold can be wrt the last reported TA + common drift rate); Option 3: Whether UE reports UE-calculated TA to NW and in which message the report should be included should only be controlled by NW. Proposal 7: the exact reported User specific TA value is derived by updating the initial User specific TA by received TA command. Proposal 8: MAC CE is used to send TA report. Proposal 9: network can request UE to report User specific TA, and configure UE to perform periodic TA reporting. sr-ProhibitTimer: Proposal 10: Extend the timer length of sr-ProhibitTimer by adding the UE specific RTD to the configured sr-ProhibitTimer length.

However, the RA type selection and the TA report highlighted in yellow are not concluded yet. In this contribution, we show our views on the RA type selection and TA report.

Discussion

RA type selection

For Rel-16, a UE initiates 2-step RACH based on an RSRP threshold. This is because the radio quality should be good enough to ensure reliable transmission of the MsgA.

However, in NTN, cell coverage is very large, and there may be no big difference in RSRP between cell center and cell edge. Therefore, it is likely that most UEs in a cell perform the same type of RACH procedure, either 2-step RACH or 4-step RACH, at the same time. If all UEs trigger the same type of RACH procedure, the collision probability would be increased because the RACH resource, i.e., RAP, is separate for 2-step RACH and 4-step RACH. For this reason, the following options were proposed in the email discussion

- Option 1: based on “UE specific UE-satellite RTT” or “distance between UE and satellite”;

- Option 2: based on QoS requirement of the logical channel;

- Option 3: only use RSRP as in legacy.

For Option 1, the UE triggers the 2-step RACH based on the UE location, e.g., GNSS information. It means that the multiple UEs in an area would trigger the same type of RACH procedure. However, if the UE density in the area is very high, the collision probability would be increased.

For Option 3, as mentioned above, since it is likely that most UEs in a cell perform the same type of RACH procedure, most UEs in the cell triggers the same type of RACH procedure. It means the collision probability would be increased.

For Option 2, the network configures whether to allow 2-step RACH or not for each logical channel, and the UE selects 2-step RACH if the logical channel that triggers the RACH procedure is allowed to use 2-step RACH. With this, since only UEs having the data allowing 2-step RACH triggers the 2-step RACH, the collision probability would be decreased. In addition, considering that the main motivation of the 2-step RACH is latency reduction, Option 2 would be aligned with the motivation of the 2-step RACH.

Based on the above, we prefer Option 2 in order to decrease the collision probability for 2-step and 4-step RACH.

Proposal 1. Whether 2-step RACH is allowed is configured per logical channel.

TA reporting

In POST103e#106 email discussion, it was discussed that how the UE reports the UE-calculated TA value and the following proposal was made.

Proposal 5: RAN2 to discuss which way to go:

- Option 1: UE reports User specific TA (NTA as defined by RAN1) to network. RAN2 can revisit it if RAN1 agrees to assistance information other than User specific TA;

- Option 2: postpone the reporting of TA or position until RAN1 have concluded.

As looking at the above proposal, it is anyway needed to wait for RAN1 decision to report the UE-calculated TA. This is because how to report the UE-calculated TA value is under discussion in RAN1. Note that RAN1 considers various ways to report the UE-calculated TA using distance and UE’s position as well as Msg1/A, Msg3/5/B. In other words, even if RAN2 decides how to report the UE-calculated TA, additional discussion in RAN2 may be required if there is a collided conclusion between RAN1 and RAN2. Thus, it would be good to wait for RAN1 decision to avoid collided conclusion from different WGs.

Proposal 2. How to report the UE-calculated TA is postponed until RAN1 make a conclusion.

Conclusion

In this contribution, we show our views on RACH and TA reporting, and the following proposals are made.

Proposal 1. Whether 2-step RACH is allowed is configured per logical channel.

Proposal 2. How to report the UE-calculated TA is postponed until RAN1 make a conclusion.