3GPP TSG-RAN WG2 Meeting #113 bis electronic	R2-2104189

Online, April 12 – April 20, 2021

Agenda item:	8.11.5 (GNSS positioning integrity)

Source:	Samsung

Title:	Consideration on the signalling design for Positioning Integrity

Document for:	Discussion & Decision

- Introduction

Updated WID (RP-210903) indicates the following objective regarding positioning integrity:

- Specify the signalling, and procedures to support GNSS positioning integrity determination, including [RAN2, RAN3]:

  - The assistance information that will be used to support integrity determination

  - The information that will be used to provide the positioning integrity KPIs and integrity results

  - Support of integrity for UE-based and UE-assisted A-GNSS positioning.

Note: This objective is applicable to NR and E-UTRA.

In this paper, we discuss the support of the positioning integrity for A-GNSS.

- Baseline of integrity result signaling

Even though there is still unclear points on the exact concept on this positioning integrity, we think that TIR, AL, TTA, PL is the main parameters to be considered for positioning integrity, and GNSS related feared events which could be carried in assistance information are also important.

In computing entity, PL needs to be calculated based on other remaining three parameters KPIs (e.g., TIR, AL, TTA), and the history of measurement and its error in real time. Since PL is used for determining system availability, this value needs to be reported from computing entity to the positioning integrity decision (controlling) entity e.g., LCS entity.

Based on the endorsed TR 38.857, there are possible integrity modes and its signaling considerations as below:

Table 9.4.1.1.1: Summary of network-assisted (UE-Based) and UE-assisted (LMF-Based) positioning integrity mode considerations.

NOTE: The table provides a summary of considerations and the final details and specification impacts are FFS in the WI.

*NOTE: Examples of KPIs are the TIR, AL, TTA. Examples of Integrity results are the PL and Integrity Availability.

**NOTE: From LMF to UE does not mean the integrity assistance information is generated by the LMF.

| Positioning Integrity Mode | Location service type | Source of KPIs* | Source of Integrity results* | Positioning Integrity assistance information** | Specification impact |
| --- | --- | --- | --- | --- | --- |
| Network assisted (UE-based): Positioning integrity result is derived by the UE | MO-LR | UE internal implementation | UE internal implementation | From LMF to UE: - Feared events in the GNSS Assistance Data - Feared events in transmitting the data to the UE - GNSS feared events | Procedure to transfer Integrity assistance information from LMF to UE (case A) |
|  | MT-LR | From LMF | From UE | From LMF to UE: - Feared events in the GNSS Assistance Data - Feared events in transmitting the data to the UE - GNSS feared events | Procedure to transfer Integrity assistance information and KPIs from LMF to UE Procedure to transfer Integrity results from UE to LMF (case B) |
| UE assisted (LMF-based): Positioning integrity result is derived by the LMF | MO-LR | From UE | From LMF | From GNSS corrections provider (external source) to LMF: - Feared events in the GNSS Assistance Data - Feared events in transmitting the data to the UE - GNSS feared events From UE to LMF: - UE feared events - GNSS feared events | Procedure to transfer Integrity assistance information and KPIs from UE to LMF Procedure to transfer Integrity results from LMF to UE (case C) |
|  | MT-LR | LMF implementation | LMF internal implementation | From GNSS corrections provider (external source) to LMF: - Feared events in the GNSS Assistance Data - Feared events in transmitting the data to the UE - GNSS feared events From UE to LMF: - UE feared events - GNSS feared events | Procedure to transfer Integrity assistance information from UE to LMF (case D) |

In case B and C there should be the signaling to transfer integrity result from computing entity to LCS entity. Since measurement from GNSS signal and any feared event occurrence is time-varying, PL calculating entity calculates in real time, and report this in real time, if possible, to the integrity decision entity. There is consideration point to design the corresponding signaling above. Since PL computation can be done based on the given KPI related values and feared event information from KPI source, providing the positioning integrity assistance information and returning its resulting PL or system availability would be paired, as described in above table.

Observation 1. Given KPI parameters and GNSS feared event information, PL and/or system availability information needs to be reported to the LCS entity (or positioning integrity decision entity).

Based on the real time requirement on PL and system availability, there must be continuous and frequent transfer of the computing result according to the time duration for one PL result computed. However it is hard to guarantee all the required characteristics on contiguous/frequent/fast transaction of the positioning integrity result reporting because the physical radio resource is definitely limited to be used. Therefore, the signaling design need to consider the optimal point to balance the required characteristics of positioning integrity computing result signaling and available radio resource management.

Observation 2. There should be an optimal point in frequent reporting of positioning integrity result and radio resource budget available.

Therefore, we would like to suggest to consider those aspects to design the signaling for providing assistance information for positioinig integrity and its result reporting.

Proposal 1: RAN2 agree to design the signaling for positioning integrity assistance data providing and the positioning integrity result reporting on the given KPI and feared event information by optimizing the reporting frequency and the overhead of signaling/ radio resource used.

- Conclusion

In this contribution, we discussed on the signaling design direction for providing positioning integrity assistance data and its result reporting. As a result, we have the following conclusion:

Proposal 1: RAN2 agree to design the signaling for positioning integrity assistance data providing and reporting the positioning integrity result on the given KPI and feared event information by optimizing the reporting frequency and the overhead of signaling/ radio resource used.