3GPP TSG-RAN WG2 Meeting #113bis electronic						    	R2-2104225

E-Meeting, 12th April – 20th April, 2021

Source: 	Xiaomi Communications

Title:  	Clarification on the survival time

Agenda Item:	8.5.4

Document for:	Discussion and Decision

# Introduction

According to the RAN2#113-e meeting discussion [1], RAN2 made the following agreements on survival time requirement:

- RAN2 confirms that specification enhancement for survival time support may only needed for uplink. Downlink is addressed by implementation and no specification impacts. - Support for survival time in UCE is up to network configuration. - Continue discussing whether burst spread and burst ending time is beneficial from RAN2 perspective, but trigger the discussion after SA2 progress in February. - Only periodic traffic is considered for survival time work in Rel-17. - RAN2 assumes one application message is conveyed by one PDCP SDU, and may further consider the cases where one application message is conveyed by varying number of PDCP SDUs depending on the progress

And RAN2 also made the following agreement [2] while selecting the survival time requirement provided by the SA2:

- Time period during which “message loss” can be tolerated is adopted as the preferred format for Survival time.

According to the survival time definition provided by SA2 [3], the survival time is considered as “as a maximum time in units of “time” where each unit corresponds to the data burst periodicity defined in TSCAI in Rel-16”. In this contribution, we provide our understandings on the survival time requirement.

# Discussion

## Survival time requirement

According to the offline discussion summary [4] in the RAN2#113e meeting, there are two steps to fulfil the survival time requirement:

- Step 1 (including 11 solutions): Monitoring of survival time state

- Step 2 (including 4 solution categories): Action after entering survival time state

The intention of step 1 is to detect/count the packet loss, and the intention of step 2 is to avoid consecutive packet loss. After the UE enters the survival time state, the UE can take certain actions (e.g. PDCP duplication, change L1/L2 parameters and report the failure to the gNB as given in [4]) to avoid the intolerable consecutive errors. However it seems that some solutions for monitoring the survival time state cannot fulfil all cases of survival time. Thus we consider that RAN2 should firstly decide how the UE monitors the survival time state (i.e. Step 1) before discussion how to avoid intolerable consecutive errors (Step 2), as the Step 2 would rely on the monitoring result of Step 1.

Proposal 1: RAN2 is kindly requested to firstly select a solution of monitoring the survival time state before selecting the solution of avoiding intolerable consecutive error.

According to the Table quoted from TS 22.104 [5] and the survival timer definition given by SA2 [3], the survival time could be more than one period of the traffic. This means that if there is only one packet loss, the UE should not enter the survival time state to take excessive scheduling strategies, as the excessive scheduling strategy as given in Step 2 would consume more network resources or take over the resource allocated for other services. As such the solution in Step 1 should be able to count consecutive packet loss.

Observation: The UE can tolerate more than one consecutive packet loss before entering the survival time state.

Proposal 2: The UE should be able to count the number of consecutive packet loss.

As the consecutive packet loss is to count the number of lost PDCP SDUs (i.e. IP packets), the transmitting PDCP entity should be responsible to count the number of consecutive packet loss.

Proposal 3: The transmitting PDCP entity of the UE counts the number of consecutive packet loss.

Then the UE should enter the survival time state only when the number of the consecutive packet loss exceeds a configured threshold, so as to avoid the too early action of the Step 2.

Proposal 4: The UE enters the survival time state only when the number of the consecutive packet loss exceeds a configured threshold.

The details on how the UE determines a packet as lost (e.g. via HARQ/RLC ACK or timer) can be discussed further.

# Conclusions

According to the analysis given above, we have the following observations and proposals:

Observation: The UE can tolerate more than one consecutive packet loss before entering the survival time state.

Proposal 1: RAN2 is kindly requested to firstly select a solution of monitoring the survival time state before selecting the solution of avoiding intolerable consecutive error.

Proposal 2: The UE should be able to count the number of consecutive packet loss.

Proposal 3: The transmitting PDCP entity of the UE counts the number of consecutive packet loss.

Proposal 4: The UE enters the survival time state only when the number of the consecutive packet loss exceeds a configured threshold.

# Reference

[1] RAN2#113-e meeting minutes.

[2] RAN2#112-e meeting minutes.

[3] R2-2010692, LSin from SA2 to RAN2, “LS on Use of Survival Time for Deterministic Applications in 5GS”

[4] R2-2102074	Offline on RAN enhancements QoS [AT113-e][506] 	Nokia

[5] 3GPP TS 22.104, “Service requirements for cyber-physical control applications in vertical domains”.