3GPP TSG-RAN WG2 Meeting #113-bis-e	R2-2104256

Electronics Meeting, April 12 – 20, 2021

Agenda Item	: 8.15.2 (NR_SL_enh-Core)

Source	: LG Electronics Inc.

Title	: Consideration on sidelink DRX determination

Document for	: Discussion and Decision

1.	Introduction

RAN2 had an email discussion in [1] to figure out who decide sidelink DRX configurations is configured in NR SL unicast. There is not clear how to determine SL DRX configuration while UE is in RRC_CONNECTD. This contribution provides discussions on this issue.

2.	Discussion

2.1 Determination of SL DRX

First of all, we would like to investigate which entity will decide and provide aligned SL DRX configuration. We can assume that UE is TX UE and Peer UE is RX UE. Both TX UE and RX UE can establish with its serving network.

Figure 1 Scenario assumption

2.1.1. TX centric

Table 1. TX-centric: possible entity for RX UE’s SL DRX determination

|  | RRC state |  | RX UE’s SL DRX determination (TX UE -> RX UE) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | TX UE | RX UE | TX UE | RX UE | TX UE’s network | RX UE’s network |
| Case 1 | CONNECTED | CONNECTED | O | X | X | X |
| Case 2 |  |  | X | X | O | X |
| Case 3 | CONNECTED | IDLE | O | X | X | X |
| Case 4 |  |  | X | X | O | X |
| Case 5 | IDLE | CONNECTED | O | X | X | X |
| Case 6 | IDLE | IDLE | O | X | X | X |

Table 2. TX-centric: possible entity for TX UE’s SL DRX determination

|  | RRC state |  | TX UE’s SL DRX determination |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | TX UE | RX UE | TX UE | RX UE | TX UE’s network | RX UE’s network |
| Case 7 | CONNECTED | CONNECTED | X | X | O | X |
| Case 8 | CONNECTED | IDLE | X | X | O | X |

Given that case 1 from 4, When a TX UE is in RRC_CONNECTED, either TX UE’s network or TX UE can determine SL DRX configuration of the RX UE (i.e. TX UE-> RX UE). If RAN2 decides to determine SL DRX configuration of the RX UE with network, essential information (i.e. traffic pattern of TX UE) would be reported. After deciding the SL DRX configuration, then TX UE will deliver to the RX UE the SL DRX configuration. However, so far we are not clear why network determines the SL DRX configuration of the peer UE.

Observation 1: There is no clear technical motivation that a network configure the SL DRX configuration for peer UE.

Additionally, considering the case 7 and 8 based on table 2, network of TX UE also can determine its connected TX UE’s DRX configuration (i.e., RX UE -> TX UE). We can prioritize network’s decision for SL DRX configuration. However, when we follow this principle, additional issues are raised. It is not clear which network (e.g. TX side or RX side) will decide SL DRX configuration (e.g., TX UE -> RX UE or RX UE -> TX UE) while both UE and peer UE are in RRC_CONNECTED. Since one network (e.g., TX side or RX side) can determine SL DRX configuration for TX UE and/or RX UE, we have to specify another rule for handling this case. Moreover, while TX UE in RRC_IDLE, but RX UE is in _RRC_CONNECTED, the network of the RX UE will determine SL DRX configuration which seems to RX centric approach is applied.

Observation 2: If a network determines SL DRX configuration of its connected UE, TX-centric approach does not work well.

In contrast, if we decide to the SL DRX configuration via TX UE itself, recommend DRX patterns (via assistance information) are helpful to determine the optimized SL DRX. After determining the SL DRX configuration of the RX UE, the determined/configured SL DRX configuration for each unicast needs to be reported to the network. This case is needed when a UE operating mode 1 based scheduling. In a network perspective, configured wake-up time based on SL DRX is required that SL data is scheduled appropriately within wake-up time of the RX UE. Moreover for achieving the alignment of Uu link, the TX UE can report its received DRX (for its own) from RX UE as well as determined DRX (for RX UE).

Observation 3: When a CONNECTED TX UE configures SL DRX pattern for itself, the determined SL DRX pattern needs to be reported to the network.

1.2. RX centric-oriented

Table 3. RX-centric: possible entity for TX UE’s SL DRX determination

|  | RRC state |  | TX UE’s SL DRX determination (RX UE -> TX UE) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | TX UE | RX UE | TX UE | RX UE | TX UE’s network | RX UE’s network |
| Case A | CONNECTED | CONNECTED | X | O | X | X |
| Case B |  |  | X | X | X | O |
| Case C | CONNECTED | IDLE | X | O | X | X |
| Case D |  |  | X | X | X | O |
| Case E | IDLE | CONNECTED | X | O | X | X |
| Case F | IDLE | IDLE | X | O | X | X |

Given that case from A and D, When a RX UE is in RRC_CONNECTED, either RX UE’s network or RX UE can SL DRX configuration of the RX UE (i.e. TX UE-> RX UE). If RAN2 decides to determine SL DRX configuration of the RX UE (TX UE -> RX UE) with network, essential information (i.e. traffic pattern of TX UE) would be reported.

The network of RX UE also can determine its connected RX UE’s DRX configuration (i.e., TX UE -> RX UE). When the RX-centric is applied, the network can configure its SL DRX configuration. However, so far, technical benefit is not clear since the RX UE can determine its optimized SL DRX configuration. For the purpose of alignment, the RX UE can report to the network after determining its own SL DRX configuration. Based on the reported information, network can perform the alignment procedure.

Observation 4: RAN2 needs to consider when the network determines SL DRX configuration for the RX UE.

In contrast, if we decide to the SL DRX configuration via RX UE itself, traffic pattern is needed form TX UE. After determining the SL DRX configuration for the TX UE, the determined SL DRX configuration needs to be reported to the network for the purpose of alignment with Uu link. Based on the information, network can perform alignment with Uu link. Based on the comparison above considering TX centric and RX centric, when a UE is in RRC_CONNECTED, it is better that SL DRX determination is determined by UE itself.

Proposal 1. While a TX UE or RX UE in RRC_CONNECTED, SL DRX configuration is determined by UE itself.

Proposal 2. While a UE in RRC_CONNECTED, UE reports its own DRX configuration and peer UE’s DRX configuration to the network.

3.	Conclusion

Observation 1: There is no clear technical motivation that a network configure the SL DRX configuration for peer UE.

Observation 2: If a network determines SL DRX configuration of its connected UE, TX-centric approach is not applied.

Observation 3: When a CONNECTED TX UE configures SL DRX pattern for itself, the determined SL DRX pattern needs to be reported to the network.

Observation 4: RAN2 needs to consider when the network determines SL DRX configuration for the RX UE.

Proposal 1. While a TX UE or RX UE in RRC_CONNECTED, SL DRX configuration is determined by UE itself.

Proposal 2. While a UE in RRC_CONNECTED, UE reports its own DRX configuration and peer UE’s DRX configuration to the network.

4.	Reference

[1] [POST113-e][704][V2X/SL] TX UE centric or RX UE centric DRX configuration determination (OPPO)