3GPP TSG RAN WG2 Meeting #113bis-e                                                    R2-2104360

# Electronic meeting, 12th-20th April 2021

Agenda item:		8.12.3.1

Source:			Intel Corporation

Title:		Summary of offline 101 - [REDCAP] eDRX cycles - first round

Document for:	 	Discussion and decision

# Introduction

The intention is to discuss the following topics as part of the offline [AT113-e][101][RedCap] eDRX cycles (Intel) taken into consideration the related proposals on submitted TDocs [1]-[14]:

- Which node decides the eDRX cycle for RRC inactive (RAN vs CN)

- Whether eDRX cycles for idle and inactive use different or same configuration

- List of issues to be included in a LS to SA2/CT1

Initial intended outcome: Summary of the offline discussion with e.g.:

- List of proposals for agreement (if any)

- List of proposals that require online discussions

The deadlines for this offline are:

- Initial deadline (for companies' feedback): Tuesday 2021-04-13 16:00 UTC

- Initial deadline (for rapporteur's summary in R2-2104360): Tuesday 2021-04-13 20:00 UTC

# Discussion

## Control Node of the eDRX for RRC_INACTIVE

- Which node (i.e. RAN or CN) should control the extended DRX in RRC_INACTIVE?

| Company’s name | RAN / CN | Comments, if any |
| --- | --- | --- |
| Nokia, Nokia Shanghai Bell | RAN | Paging is initiated by RAN in INACTIVE. For RRC_INACTIVE, the UE specific DRX is configured by RAN in the suspendConfig. From NW perspective, keeping the principle that RAN configures the DRX and eDRX cycles for the UE in RRC_INACTIVE, is beneficial and provides flexibility in handling all the UEs in RRC_INACTIVE mode from which CN does not have too much visibility (other than the anchor point of the RAN-CN interface). RAN can then provide assistance information towards CN about DL data forwarding for UEs in RRC_INACTIVE. |
| OPPO | RAN | We think RAN should be the node who is responsible for eDRX configuration in RRC INACTIVE, based on the eDRX configuration in RRC IDLE configured by CN, and some RAN side consideration. |
| Ericsson | RAN | Agree with Nokia and OPPO views, RAN shouls be in control of configuring eDRX for RRC_INACTIVE. The DP should be clarified, i.e. what does “control the extended DRX” refer to? We assume this refers to configuration of the relevant parameters and paging the UE in extended DRX. |
| Apple | Why not CN? | We see that some companies want RAN to configure. We would like to discuss as RAN config is useful for having the RAN page the UE in a shorter lead time in case RAN wants to resume/inform the UE. - With RedCap (esp with longer DRX where power-saving is important, and if the ETWS/CMAS or SI update is not guaranteed), there is no compelling reason for RAN to have to ping the UE earlier when UE anyway monitors at CN paging cycle - With PTW/PH framework even for INACTIVE, RAN may have to ensure that CN PTW and RAN PTW are aligned to prevent the UE from monitoring separate discrete times (for power saving). This means the RAN has to understand the CN config and re-config the RAN PTW in a simiar way, - RAN anyway has info on the CN config, and CN informs RAN about the CN DRX config for the UE (including RedCap). No additional signaling is needed - With RAN config of DRX in INACTIVE, the RAN nodes need to exchange the config so that the UE follows the same config in every cell the UE re-selects. With CN config, the RAN does not have to worry about this, as the CN provides the DRX info at paging and RAN can just follow it - This may also avoid the buffering issue at RAN! We understand that RAN config of INACTIVE is useful for shorter DRX cycles (like in legacy), but with RedCap this is not always the case. We think it might simplify the design to just allow CN to configure for both IDLE and INACTIVE, as CN anyway knows that the UE is INACTIVE using the existing signaling. |
| Qualcomm | RAN | We have the same comment as Nokia |
| Intel | RAN | We support having RAN control as CN is not aware when a UE is in RRC_INACTIVE. If CN has the control of eDRX configuration / operation, additional signaling would be required between gNB and AMF every time that UE enters or exits from RRC_INACTIVE. In our understanding, this breaks the main principle and motivation of RRC_INACTIVE (which supposes to be a transparent RRC state from AMF point of view). |
| Samsung | RAN | RAN is responsible, while CN may need to provide any useful information to RAN, e.g. eDRX configuration for RRC_IDLE. |
| Xiaomi | RAN | RAN configures the UE with an e-DRX cycle in RRC-INACTIVE and this should be up to the value used for the UE's idle mode e-DRX cycle as provided by the AMF, or up to the maximum value allowed based on the NAS retransmission timers. It should be noticed that UE in RRC inactive can be configured with idle mode e-DRX cycle by AMF but without e-DRX cycle in RRC-INACTIVE given by RAN. |
| Sharp | RAN | RAN controls the eDRX in RRC_INACTIVE. |
| NEC | RAN | CN can only provide eDRX configuration for RRC_IDLE, while for INACTIVE UE, RAN should fully in control since CN is not aware of the RRC_INACTIVE UE. |
| Futurewei | RAN | Agree with Nokia and OPPO that RAN should be responsible for configuring eDRX for RRC_INACTIVE. |
| MediaTek | RAN | RAN could control the eDRX configuration in Inactive, based on the coordination with the CN. RAN should ensure that the CN requirements are met (for example with respect to NAS timers) when configuring eDRX for Inactive. |
| CATT | CN | CN has better insight on the UE traffic profile and is already in charge of the eDRX cycle for Idle. Thus it provides the smoothest migration from legacy, also considering that eDRX cycles are intended for long periods of sleep with little influence from RAN. This approach is also best suited when common PTW and eDRX cycle configuration is used for Idle and Inactive. |
| vivo | RAN | For DRX, UE specific DRX cycle for inactive (i.e. ran-PagingCycle) is provided by RAN. Following the same principle, we think RAN is responsible to provide UE specific eDRX configuration for inactive state, based on information provided by CN, e.g. eDRX configuration for RRC_IDLE. |
| DENSO | RAN | We think that RAN should control the eDRX for INACTIVE because RAN controls the transition between Connected and INACTIVE of UE. |
| ZTE | prefer CN, but can accept RAN | CN has better insight on UE traffic profile. Considering the main intention to have eDRX cycle larger than 10.24s for RRC inactive is to enable small data transmission in RRC INACTIVE, the traffic profile should be the main factor in eDRX cycle length determination. As the traffic profile is not expected to be changed frequently, CN can configure eDRX for RRC INACTIVE before RAN transits RedCap UE to RRC INACTIVE. Thus there is no additional signaling exchange between RAN and CN. And anyway, CN can still configure a shorter eDRX cycle for RRC INACITVE. However, we are ok to follow majority. |
| Huawei, HiSilicon | RAN | We agree with Nokia’s comments. |
| CMCC | RAN | We think RAN should be responsible for the eDRX cycle configuration in INACTIVE, based on the information about eDRX for IDLE provided by CN, since paging in INACTIVE is initiated by RAN. |
| Sequans | RAN | Agree with Nokia and others |
| LGE | RAN | It is reasonable that RAN controls eDRX in RRC_INACTIVE. Information change for eDRX configuration would be needed between RAN and CN. How to exchange the information is up to RAN3 discussion. |
| Convida | RAN | Both options are captured in the TR 38.875 (section 8.3.4) and possible, but our preference is for RAN. It is worth to noting that in the case of WB-E-UTRA and LTE-M connected to 5GC, NG-RAN configures the eDRX cycle as specified in TS 23.501 section 5.31.7.2.1. “If the UE supports eDRX in RRC inactive, based on its UE radio capabilities, NG-RAN configures the UE with an eDRX cycle in RRC-INACTIVE ...” |

## Configuration of the eDRX for RRC_IDLE / RRC_INACTIVE

- For configuration of the eDRX for RRC_IDLE / RRC_INACTIVE, do you prefer (a) using the same configuration [4][7][8][11], or (b) allowing the network to choose different configurations [1][2][3][6][9]? Note that the following Discussion point 3) and Discussion point 4) addressed detailed approaches proposed by the different supportive companies.

| Company’s name | Option | Comments, if any |
| --- | --- | --- |
| Nokia, Nokia Shanghai Bell | b) | DL Data buffering in the NW is different in IDLE and INACTIVE and it would be beneficial to have flexibility for the NW to configure different configuration including eDRX cycle for IDLE and INACTIVE. |
| OPPO | Option b | We think RAN should be allowed to configure a different eDRX cycle for RAN paging, which could provide more flexible configuration compared with a common eDRX cycle configuration. Otherwise, RAN could only page the UE based on eDRX cycle configured by CN, but has no ways to page the UE with a shorter latency (if it wants). |
| Ericsson | b) | This depends, at least partly, on the outcome of the discussion on which node configures the RRC_INACTIVE parameters. If RAN does not have choice to configure other than what CN configures, then it is functionally CN which configures. Agree with Nokia and OPPO comments, but on top of OPPO view we would also let RAN configure PTW. |
| Apple | We prefer a | (b) would be avoided by CN config for INACTIVE. |
| Qualcomm | b) | We do not think always forcing common eDRX configurations between RRC Idle and Inactive are fully justified, as argued in our paper [1]. There are use cases in which it is beneficial to have different eDRX configurations (at least different eDRX cycles) between the two RRC states. If in some cases network does want to have common eDRX configurations, it can use the existing signaling to do so without the need for new enhancements. |
| Intel | b) |  |
| Samsung | b) | It seems beneficial to have a flexibility. |
| Xiaomi | b) | As the RAN buffers DL packets for the UE in RRC_INACTIVE, it is natural that RAN decides the e-DRX parameters for RAN paging. To guarantee that a UE in RRC_INACTIVE can monitor both RAN paging and CN paging, it is better that the RAN paging e-DRX configuration can be decided using some assistance information from the CN, e.g. a shorter e-DRX cycle. |
| Sharp | b) | Different eDRX cycle can be configured for RRC_INACTIVE from for RRC_IDLE. |
| NEC | b) | RAN should decide the paging for INACTIVE UE. CN takes INACTIVE UE CMM_CONNECTED but takes IDLE UE CMM IDLE, so CN may send data for INACTIVE UE, in this case, RAN should trigger the paging for INACTIVE UE. So INACTIVE/IDLE UE should have different eDRX. |
| Futurewei | b) |  |
| MediaTek | b) | Because the scenarios and the requirements for Idle and Inactive modes can be different, it may be beneficial to use different configurations. |
| CATT | (a) | Simplest as it does not require any complex “merge” procedure of different configurations when they have to be supported simultaneously to cope with states mismatch. |
| vivo | b) | For DRX, UE specific DRX cycle for inactive and idle can be different. For eDRX, there is no motivation to have common eDRX configuration for idle and inactive. We prefer to have similar flexibility for eDRX configuration. |
| DENSO | a&b | Basically, eDRX is used in the same configuration for IDLE and INACTIVE, but if different configurations are set for IDLE and INACTIVE, the eDRX settings corresponding to the different configurations should be reflected in the system. |
| ZTE | b) | We prefer to support different configurations for flexibility, e.g. a shorter eDRX cycle for RRC INACTIVE. In our paper[7], we propose to configure overlapped PTW for RRC idle and RRC inactive as much as possible, i.e. eDRX cycle for RRC IDLE and INACTIVE can be different, but the PTWs are partial overlapped. Following figure show an example of our preference. For IDLE and INACTIVE eDRX cycle equal to or shorter than 10.24s, we are fine to support different eDRX configuration for RRC IDLE and INACTIVE. |
| Huawei, HiSilicon | b) | We should not limit the flexibility of the RAN. RAN can configure same or different eDRX configuration for RRC_INACTIVE based on its implementation. |
| CMCC | a) & b) | In our opinion, common eDRX configuration could be a start point, and we also see the benefit of different eDRX cycle for IDLE and INACTIVE, as analysed in our contribution. And considering UE power consumption, common PTW could be considered. Besides, we have the similar concern with OPPO, some approaches for Option a) seem like approaches for Option b). |
| Sequans | b | This flexibility seems beneficial as mentioned above |
| LGE | b) |  |
| Convida | Option b | We prefer that the eDRX cycle for the RRC_IDLE and RRC_INACTIVE can have different configurations for flexibility. It is worth noting that in the case of WB-E-UTRA and LTE-M connected to 5GC, NR-configure the eDRX cycle for RRC_INACTIVE that may be different from eDRX cycle for RRC_IDLE as specified in TS 23.501 section 5.31.7.2.1. “If the UE supports eDRX in RRC inactive, based on its UE radio capabilities, NG-RAN configures the UE with an eDRX cycle in RRC-INACTIVE up to the value for the UE's idle mode eDRX cycle as provided by the AMF in "RRC Inactive Assistance Information" or up to 10.24 seconds (whichever is lower).” |

- If option a) same configuration of the eDRX for RRC_IDLE / RRC_INACTIVE were supported, please indicate any of your preferred approach(es):

- Different eDRX cycle, but a common PTW length and the PTW starting point [2].

- Common eDRX cycle defined as min{ eDRX cycle given by AMF, eDRX cycle given by RAN }and common PTW (i.e. PTW start/ PTW length) [4].

- For eDRX > 10.24sec, common PTW but with different eDRX cycle [7].

- For eDRX 10.24s, set the common eDRX cycle as T in PF/PO determination [7]

- Common eDRX configuration, i.e. eDRX cycle and when applicable, PTW [8] [11].

- Other approaches to consider.

| Company’s name | Approach(es) [y/n] |  |  |  |  |  | Comments, if any |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | a.1 | a.2 | a.3 | a.4 | a.5 | a.6 |  |
| Apple |  |  |  |  |  | Y | Much simpler to let CN configure the DRX for INACTIVE (in other words have same config for IDLE/INACTIVE, assuming that CN does not distinguish between IDLE/INACTIVE). |
| CATT | n | n | n | n | y | n | Other approaches but a.5 are more in the option (b) in our view. |
| DENSO | y |  | y |  |  |  | From a UE perspective, we think that if UE is used differently in IDLE and INACTIVE, it will require different eDRX settings. Since different eDRX are set for some reason, the setting should be reflected properly as a system. |
| CMCC | Y |  | Y |  | Y |  | Common eDRX configuration could be a start point. Considering the difference between IDLE and INACTIVE, a different eDRX cycle could be considered, and this will provide more flexibility. Besides, from the UE power consumption point, a common PTW is preferred. |

- If option b) different configuration of the eDRX for RRC_IDLE / RRC_INACTIVE were supported, please indicate any of your preferred approach(es):

- Any coordination between eDRX configurations for RRC_IDLE / RRC_INACTIVE is left up to network implementation [1] [3] [6].

- RAN guarantees that guarantees that the configuration provided for eDRX in RRC_INACTIVE is a sub-set of one provided for eDRX in RRC_IDLE [3]. E.g. eDRX cycle in RRC_INACTIVE may be shorter than the eDRX cycle in RRC_IDLE, and the PTW should be either common or overlapping in the case either window is shorter than the other [6].

- Different eDRX configuration, i.e. eDRX cycle and when applicable, PTW [1][2][3][6][9].

- Different eDRX cycle, but a common PTW length and the PTW starting point [2].

- For eDRX > 10.24sec, common PTW but with different eDRX cycle [7].

- Other approaches to consider.

| Company’s name | Approach(es) [y/n] |  |  |  |  | Comments, if any |
| --- | --- | --- | --- | --- | --- | --- |
|  | b.1 | b.2 | b.3 | b.4 | b.5 |  |
| Nokia, Nokia Shanghai |  |  | X |  |  |  |
| OPPO | N | N | N | Y |  | We prefer (b.4), i.e. different eDRX cycle and common PTW length for RRC IDLE and RRC INACTIVE. We think RAN should be allowed to configure a different eDRX cycle for RAN paging, which could provide more flexible configuration compared with a common eDRX cycle configuration. Otherwise, RAN could only page the UE based on eDRX cycle configured by CN, but has no ways to page the UE with a shorter latency (if it wants). Regarding PTW, from UE power saving point of view, the PTWs for CN paging and RAN paging should be overlapped as much as possible, so a common PTW length can be used for both CN paging and RAN paging. |
| Ericsson | N | N | Y | N |  | If RAN configures the parameters for inactive eDRX, we don’t think there should be restrictions on the configuration. However, we do agree that it would be beneficial to have a common, or overlapping, PTW. We think this should be left up to NW / RAN implementation to configure. We also prefer to specify coordination so that operation is possible e.g. in multi-vendor situations, however details of such signaling are up to other WGs. |
| Qualcomm | Y | N | N | N | Y | If majority of companies prefer to have some constraints on the two eDRX configurations, it is fine with us if that constraint is for ensuring there are some overlapping POs between two PTWs (e.g. have a common PTW starting point). But that is necessary only when RRC Inactive has an eDRX cycle > 10.24 sec (whose feasibility is pending approval by SA2/CT1). |
| Intel | y | y | y | n |  | In our understanding, the behaviour described by points (b.2) and (b.3) would be handled by network implementation (as explained in point (b.1)). |
| Samsung | Y | Y | Y | N |  | We have assumed any coordination is required to maximize UE power saving gain while guaranteeing the flexibility. |
| Xiaomi |  | Y |  |  |  | Is b.2 included in b.4? We support that eDRX in RRC_INACTIVE is a sub-set of one provided for eDRX in RRC_IDLE, e.g., a shorter e-DRX cycle. We also prefer a common PTW. |
| Sharp | N | Y | N | Y |  | If the eDRX cycle values defined in RAN spec is sub-set of the eDRX cycle values defined in CN spec, RAN doesn’t need to do extra effort to guarantee the configuration and coordination between RAN and CN is not needed. Although CN and RAN configure different eDRX cycles for UE, the UE can monitor the minimum one to reduce complexity. Considering eDRX cycle for INACTIVE is sub-set of eDRX cycle for IDLE, UE will not miss any paging. |
| NEC | Y | Y | Y | N | N | We prefer option b.1 and we think b.2/b.3 can be covered by network implementation. |
| Futurewei |  |  | Y |  |  |  |
| MediaTek | Y | Y |  | Y | Y | The network (CN and RAN) should coordinate and make sure that the eDRX configurations satisfy the requirements for Idle and Inactive. For Inactive, RAN should provide the eDRX to the UE, while making sure that the CN requirements are not violated. To simplify the eDRX configurations for Idle and Inactive and to achieve some commonality between the two, Approach b.4 can be useful (i.e. with different eDRX cycles, and common PTW configuration). As it is already agreed that PTW will not be used for eDRX cycle<=10.24s, b.4 and b.5 look similar to us. |
| vivo | Y | Y | N | N | Y | Firstly, we think it is up to network implementation whether to have coordination for eDRX configuration between idle and inactive. As UEs in inactive mode will monitor both RAN paging and CN paging, it is better to guarantee that the configuration for eDRX in inactive is the sub-set of eDRX configuration in idle. Such coordination is required to maximize UE power saving gain. |
| DENSO | y |  | y |  |  | If there are use cases where it is more effective for the NW to adjust the eDRX settings, it is desirable to adjust the eDRX settings. However, we think that NW should avoid blindly adjusting. |
| ZTE | Y |  |  |  | Y | First of all, network (CN or RAN) can decide whether to configure same different eDRX configurations for IDLE and INACTIVE. Further, when both eDRX cycles are longer than 10.24s, we prefer b.5: IDLE PTW and INACTIVE PTW can overlap as much possible, i.e. an IDLE PTW always overlaps an INACTIVE PTW (share same PH and PTW starting point). We think this is beneficial for UE power saving. For PTW length, we think the same PTW length is enough and simpler for paging monitoring. Considering the intention of introducing PTW is to improve paging robustness with multiple paging chances during long eDRX cycle, there is no difference for IDLE PTW and INACTIVE PTW on this aspect. Following figure shows an example of our preference |
| Huawei, HiSilicon | N | Y | Y | N |  | In our view, (b.3) should be the baseline, i.e. no need to put restrictions on the RAN. We agree that, from the UE point of view, it would be beneficial to have common/overlapping PTW and we think it can be achieved by NW implementation. However, if there is strong support to have some restrictions on the configuration, we are open to discuss further on the realisation of (b.2). |
| CMCC |  |  |  | Y |  |  |
| Sequans | Y | Y | Y |  |  | What is the difference between b.1 and b.3? they both give full NW control. If we decide to limit NW control, then it makes sense to us to maximize the UE power saving, as per option b.2 We are also fine with the other options. |
| LGE |  | Y |  |  | Y | No strong view on b.1). We are thinking the network side behavior may need to be specified, but we are also fine to leave it to implementation. |
| Convida | N | N | Y | N |  | The latency/power savings requirements in RRC_IDLE and RRC_INACTIVE may vary, suggesting a flexible configuration. Although we think it will be beneficial to have a common or overlapping PTW, we do not see a compelling reason to allow for different eDRX cycles but mandate a common PTW. A smart NW/RAN implementation can always configure a common PTW where/when appropriate. |

## LS to SA2/CT1: list of issues and topics to include

- Please indicate which of the following point(s) you like to include on the LS to SA2/CT1. Note that some points may overlap a little bit aiming to capture all proposals from different companies e.g. point (A) partially overlaps with (B), (C) and (G).

- Include RAN2 agreements from this meeting, if any, related to NR eDRX design (e.g. node to control INACTIVE eDRX, whether eDRX config. for IDLE and INACTIVE are same or different), and ask them for any feedback, if any [3][14][4][2].

- Ask them about feasibility of having a maximum eDRX cycle up set to 10485.76s for both RRC_IDLE and RRC_INACTIVE (e.g. considering NAS re-transmission timer)[3][6][7][14][2].

- Ask them about the responsible node to decide eDRX parameters for RRC_INACTIVE [5].

- Ask them about feasibility of buffering data in CN when the UE is unreachable from CN perspective, i.e. when the UE is in eDRX in RRC_INACTIVE. If so, RAN2 assumes that CN provides an indication to RAN in case the mobile terminating traffic is pending towards the UE [6].

- RAN2 assumes that RAN provides necessary information to CN so that CN can estimate when the UE is unreachable, e.g. by providing the RAN paging configuration [6].

- Include WI objective and ask them whether there is any concern [6][15].

“Specify support for the following Extended DRX enhancements for RedCap UEs [RAN2, RAN3, RAN4]:

  - Extended DRX for RRC Inactive and Idle with eDRX cycles up to 10.24 s, without using PTW and PH, and with common design (e.g. common set of eDRX values) between RRC Inactive and Idle

  - Extended DRX for RRC Inactive and Idle with eDRX cycles up to 10485.76 s; the details of mechanisms and feasibility regarding maximum length of the extended DRX cycles for RRC Inactive and Idle need to be checked by SA2, CT1 and/or RAN4.

  - RAN2 to decide which Node(s) configure eDRX in RRC_Idle and RRC_Inactive.”

- Include the conclusions on eDRX in TR 38.875, v2.0.0 [6][16]:

“The study of UE power saving on extended DRX in RRC_INACTIVE and/or RRC_IDLE can be summarized as follows:

  - Extended DRX for RedCap UEs for RRC_IDLE and RRC_INACTIVE have been studied. The study includes analysis of UE power saving, possible upper and lower bounds for eDRX cycles and study of possible mechanisms for eDRX for RedCap UEs in clauses 8.3.1-8.3.4.

    - The upper bound for DRX cycles and shorter eDRX values than 5.12 seconds, i.e. 2.56 seconds have been studied and options are discussed in clause 8.3.3.

    - Solutions for PTW and eDRX cycle configuration and which node should configure the eDRX cycle for RRC_INACTIVE have been studied and solutions are captured in clause 8.3.4.

Based on the study of UE power saving on extended DRX, the following are recommended from RAN2 perspective, where feasibility is to be confirmed with SA2 and/or CT1:

  - The applicable parts of eDRX mechanisms for LTE, including use of H-SFN, PH and PTW are expected to be re-used for RedCap UEs.

  - It is recommended that for eDRX cycles below and equal to 10.24 seconds PTW and PH is not used and that common design for handling eDRX cycle equal to 10.24 seconds in RRC_IDLE and RRC_INACTIVE is specified.

  - It is recommended eDRX cycles in RRC_IDLE are extended up to 10485.76 seconds, unless RAN4 indicates such eDRX value requires UE to perform RRM on serving cell outside PTW.

  - It is recommended eDRX cycles in RRC_INACTIVE are extended > 10.24 seconds

- Other aspects

| Company’s name | Points [y/n] |  |  |  |  |  |  |  | Comments, if any |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | A | B | C | D | E | F | G | H |  |
| Nokia, Nokia Shanghai Bell | X |  |  |  |  |  |  |  |  |
| OPPO | Y | Y |  |  |  |  |  |  | We think the main concern to support a maximum eDRX cycle up to 10485.76s for RRC_INACTIVE is the NAS re-transmission timer impact. We also think if RAN2 make some progress on the NR eDRX design, the related agreements should be informed to SA2/CT1. |
| Ericsson | Y | Y | N | Y | Y | Y | Y | N | We think the most important points are to 1) check the feasibility 2) inform about RAN2 intention so that it is clear what RAN2 preferences are, and 3) inform about decision on which node configures RAN paging eDRX. |
| Apple | Y | Y | Y | Y | Y | N | Y | N | We do not think F is needed. |
| Qualcomm | ? | Y | N | N | N | N | Y | N | Whether to include RAN2 agreements from this meeting depends on whether they require confirmation from SA2/CT1. We think the most important points ask SA2/CT1 about the feasibility of extending eDRX cycle for RRC Inactive beyond 10.24 sec. The conclusion in the TR can be provided in the LS as a courtesy. |
| Intel | y | y | n | n | y | n | n |  | We are open to add TDoc reference to the agreed WID (point (F)) and the TR (point (G)) even though it is not essential. Note that the key questions agreed during the SI phase are already addressed within points (A) and (B) which in our understanding is the key information to include the LS. |
| Samsung | Y | Y | N | Y | Y | N | Y | N | Regarding E, in the opposite direction, CN may provide necessary information to RAN. In coordination aspect, RAN2 could ask if RAN and CN need to share necessary information? |
| Xiaomi | X | X | X |  |  | X |  |  | A, C: yes. B:yes, since the upper boundary will have an impact on NAS re-transmission timer D: I checked the [6], the case is based on the path switch between the CN and the receiving gNB fails. Is this a corner case? I do not think this question is specified for Redcap. It exists for eMBB. E: not convinced. If RAN configures the UE with an e-DRX cycle in RRC-INACTIVE up to the value used for the UE's idle mode e-DRX cycle as provided by the AMF, or up to the maximum value allowed based on the NAS retransmission timers (whichever is lower). Does the problem solved? F: yes, we need to ask them 10.24 s, without using PTW and PH is feasible because it is diverting from LTE. G:overlapped with the previous points |
| Sharp | Y | Y |  |  |  |  |  |  | C seems included in A. |
| NEC | y | y | n | n | y | n | y |  | For C&D: RAN should make the decision. It is unnecessary to ask CN regarding the INACTGIVE UE eDRX. For G, the TR includes the latest progress of RAN2 which is clear enough, thus F may not be necessary. |
| Futurewei | Y | Y |  |  |  |  |  |  |  |
| MediaTek | Y |  |  |  |  |  |  |  | We think informing SA2/CT1 about the eDRX agreements from this meeting should be the main goal. If companies think that it is necessary, point B (10485.76s eDRX cycle) can be included in the LS as well. |
| CATT | y | y | n | n | n | n | y (partly) | n | For G it can be updated based on A |
| vivo | Y | Y | Y | Y | N | Y | Y | N | We think all RAN2 progress on eDRX should be informed to SA2/CT1. Meanwhile, all the feasibility check for eDRX is needed. Anyway, there is no harm to inform them such information. For e), we assume it could be decided later based on the feedback from SA2/CT1. |
| DENSO | y |  | y |  |  |  |  |  | We ask SA2/CT1 for feedback for the proposal decided in RAN2 |
| ZTE | Y | Y |  |  |  |  |  |  | A/B): these two are the key issues for SA/CT confirmation/approval. C): On which node is responsible for eDRX configuration for RRC INACTIVE, RAN2 can make decision and inform SA2/CT1. D): whether DL data is buffered in CN or not should be decided by SA2/CT1. Per our understanding this is only one of potential methods to resolve the issue caused by large eDRX cycle. Another potential method is to increase the retransmission timer. After all, how to address this issue should be discussed and decided in SA2/CT1. E): whether this is needed depends on which node make decision for RRC INACTIVE eDRX configuration and which node buffers DL data. It is too early to ask this. |
| Huawei, HiSilicon | Y | Y | N | N | N | N | N | N | We agree with Oppo’s comments. |
| CMCC | Y | Y | Y |  |  |  |  |  | We think the most important thing is to check the feasibility of having a maximum eDRX cycle up set to 10485.76s. And it’s fine to inform RAN2’s progress, including the decision on which node configures RAN paging eDRX. |
| Sequans | ? | Y |  |  |  | ? | ? |  | Point B is the main reason to send the LS. If other relevant agreements are reached this meeting they can be considered. We think it is enough to provide the WID and TR references as a courtesy |
| LGE | Y | Y |  | ? | ? |  | Y |  | Regarding D & E: We prefer to discuss them in RAN2 first. If RAN2 think it is possible solutions, we are fine to include them in the LS. |
| Convida | Y | Y |  |  |  | Y | Y |  | - Inform SA2 about RAN2 preferences and agreements. Note B might not be needed depending on how the agreements in A are formulated. - For C, we only need to inform SA2 about a RAN2 decision. - D is not clear and requires more discussion. “when the UE is unreachable from CN perspective, i.e. when the UE is in eDRX in RRC_INACTIVE” should be “when the UE is unreachable from RAN perspective, i.e. when the UE is in eDRX in RRC_INACTIVE”. Also, it is worth noting that in the case of WB-E-UTRA and LTE-M connected to 5GC, “If eDRX cycle is applied in RRC-INACTIVE, the RAN buffers DL packets up to the duration of the eDRX cycle chosen by NG-RAN“ as specified in TS 23.501 section 5.31.7.2.1. For D&E, RAN2 can inform SA2 on related agreements once RAN2 reaches some conclusion. |

# Report: summary and proposals

This report summarizes the views of 23 companies (Nokia, Nokia Shanghai Bell, OPPO, Ericsson, Apple, Qualcomm, Intel, Samsung, Xiaomi, Sharp, NEC, Futurewei, MediaTek, CATT, vivo, DENSO, ZTE, Huawei, HiSilicon, CMCC, Sequans, LGE, Convida).

Aiming to help with the meeting discussion/progress, the proposals are categorized starting with:

- [To agree] when there is large support and hence proposed for easy agreement.

- [To discuss] when there is substantial level of support and agreement may be possible.

- [FFS] when there is low support or companies propose new solutions or options to possibly consider further e.g. if there is sufficient support (understanding that these topic have not been discussed by all companies when providing their views in the different discussion points).

The proposals also start with a number: for the format [x], ‘x’ represents the number of supportive companies (i.e. these solutions are marked as FFS as the proposed solutions were not discussed by all companies) and, for the format [x/y], ‘x’ represents the number of supportive companies, and (y-x) the number of companies with different view.

## Control Node of the eDRX for RRC_INACTIVE

The Discussion point 1) asked about which node (i.e. RAN or CN) should control the extended DRX in RRC_INACTIVE.

- “RAN controls eDRX for RRC_INACTIVE” is supported by 20 companies (Nokia, Nokia Shanghai Bell, OPPO, Ericsson, Qualcomm, Intel, Samsung, Xiaomi, Sharp, NEC, Futurewei, MediaTek, vivo, DENSO, Huawei, HiSilicon, CMCC, Sequans, LGE, Convida).

- “CN controls eDRX for RRC_INACTIVE” supported by 3 companies (Apple, CATT, ZTE).

Rapporteur: propose for agreement that RAN controls the configuration of eDRX for RRC_INACTIVE” considering the large majority support.

The following comments were raised by supporting companies:

- There seems to be a common view that from network perspective, keeping the principle that RAN configures the DRX and eDRX cycles for the UE in RRC_INACTIVE, is beneficial and provides flexibility in handling all the UEs in RRC_INACTIVE mode from which CN does not have too much visibility (other than the anchor point of the RAN-CN interface).

- Ericsson clarifies that the question should clarify that this refer to the configuration of eDRX. Rapporteur: proposal clarifies this.

- Nokia also indicated that RAN can then provide assistance information towards CN about DL data forwarding for UEs in RRC_INACTIVE. This seems to be supported by other companies e.g. Ericsson, Qualcomm, Futurewei, Huawei, HiSilicon, Sequans. Rapporteur: added in proposal for potential discussion.

- Samsung indicated that CN may need to provide any useful information to RAN, e.g. eDRX configuration for RRC_IDLE. Rapporteur: this is discussed implicitly as one of the potential points to raise in the LS to SA2/CT1.

The following comments were raised by not supporting companies:

- Apple explains that RAN config of INACTIVE is useful for shorter DRX cycles (like in legacy), but with RedCap this is not always the case. We think it might simplify the design to just allow CN to configure for both IDLE and INACTIVE.

- CATT and ZTE also indicate that CN has better insight on the UE traffic profile.

- [To agree] [20/23] RAN controls the configuration of eDRX for RRC_INACTIVE.

- [FFS] [7] If there is larger support, to further discuss in future meetings whether RAN can provide assistance information towards CN about DL data forwarding for UEs in RRC_INACTIVE configured with eDRX by RAN.

## Configuration of the eDRX for RRC_IDLE / RRC_INACTIVE

The Discussion point 2) asked whether configuration of the eDRX for RRC_IDLE / RRC_INACTIVE should be (a) same or (b) different:

- Option (a) is supported by 4 companies (Apple, CATT, DENSO, CMCC)

- Option (b) is supported by 21 companies (Nokia, Nokia Shanghai Bell, OPPO, Ericsson, Qualcomm, Intel, Samsung, Xiaomi, Sharp, NEC, Futurewei, MediaTek, vivo, DENSO, ZTE, Huawei, HiSilicon, CMCC, Sequans, LGE, Convida).

The following comments were raised by supporting companies of option (a):

- CATT indicates that this would be simpler option.

- OPPO and CMCC clarifies that some approaches for Option a) seem applicable approaches for Option b).

The following comments were raised by supporting companies of option (b):

- Nokia clarifies that DL Data buffering in the NW is different in IDLE and INACTIVE and it would be beneficial to have flexibility for the NW to configure different configuration including eDRX cycle for IDLE and INACTIVE.

- OPPO and Ericsson clarifies that this allows RAN to configure UEs in INACTIVE with shorter eDRX cycles which would reduce latency when needed.

- ZTE explains that PTW for RRC idle and RRC inactive should overlap as much as possible, i.e. eDRX cycle for RRC IDLE and INACTIVE can be different, but the PTWs are partial overlapped

Rapporteur: propose for agreement on option (a) of the Discussion point 2) due to the large support.

- [To agree] [21/23] The configurations of the eDRX for RRC_IDLE and RRC_INACTIVE can be different.

The Discussion point 3) asked further details on the preferred approach if option a) of the Discussion point 2) were supported.

- Approach a.1) is supported by 2 companies (DENSO, CMCC).

- Approach a.3) is supported by 2 companies (DENSO, CMCC).

- Approach a.5) is supported by 2 companies (CATT, CMCC).

- Approach a.6) I supported by 1 company (Apple).

Rapporteur: No proposal for discussion is added considering that option (a) of the Discussion point 2) is not supported by majority view as captured in previous  Proposal 3.

The Discussion point 4) aims to clarify further details if option b) of the Discussion point 2) were supported.

- Any coordination between eDRX configurations for RRC_IDLE / RRC_INACTIVE is left up to network implementation [1].

- This approach (b.1) is supported by 9 companies (Qualcomm, Intel, Samsung, NEC, MediaTek, vivo, DENSO, ZTE, Sequans).

- RAN guarantees that guarantees that the configuration provided for eDRX in RRC_INACTIVE is a sub-set of one provided for eDRX in RRC_IDLE [3]. E.g. eDRX cycle in RRC_INACTIVE may be shorter than the eDRX cycle in RRC_IDLE, and the PTW should be either common or overlapping in the case either window is shorter than the other [6].

- This approach (b.2) is supported by 11 companies (Intel, Samsung, Xiaomi, Sharp, NEC, MediaTek, vivo, Huawei, HiSilicon, Sequans, LGE).

- Different eDRX configuration, i.e. eDRX cycle and when applicable, PTW [1][2][3][6][9].

- This approach (b.3) is supported by 12 companies (Nokia, Nokia Shanghai Bell, Ericsson, Intel, Samsung, NEC, Futurewei, DENSO, Huawei, HiSilicon, Sequans, Convida).

- Different eDRX cycle, but a common PTW length and the PTW starting point [2].

- This approach (b.4) is supported by 4 companies (OPPO, Sharp, MediaTek, CMCC).

- For eDRX > 10.24sec, common PTW but with different eDRX cycle [7].

- This approach (b.5) is supported by 5 companies (Qualcomm, MediaTek, vivo, ZTE, LGE).

- Other approaches to consider.

- None

Rapporteur: 21 companies provided inputs to this discussion point. The proposal is to only continue discussion on the proposals with larger support i.e. approach (b.1), (b.2), (b.3). Moreover, supporting companies also explain that points (b.2) and (b.3) would be handled by network implementation (as explained in point (b.1))

- [To discuss] To discuss the following details in relation to Proposal 3:

  - [To discuss] [9/21] To consider that any coordination between eDRX configurations for RRC_IDLE / RRC_INACTIVE is left up to network implementation

  - [To discuss] [11/21] To consider that RAN guarantees that the configuration provided for eDRX in RRC_INACTIVE is a sub-set of one provided for eDRX in RRC_IDLE. E.g. eDRX cycle in RRC_INACTIVE may be shorter than the eDRX cycle in RRC_IDLE, and the PTW should be either common or overlapping in the case either window is shorter than the other.

  - [To discuss] [12/21] To consider that different eDRX configuration refers to different eDRX cycle and when applicable, different PTW.

## LS to SA2/CT1: list of issues and topics to include

The Discussion point 5) discuss potential point(s) to be included on the LS to SA2/CT1.

- Include RAN2 agreements from this meeting, if any, related to NR eDRX design (e.g. node to control INACTIVE eDRX, whether eDRX config. for IDLE and INACTIVE are same or different), and ask them for any feedback, if any [3][14][4][2].

- This point (A) is supported by 21 companies (Nokia, Nokia Shanghai Bell, OPPO, Ericsson, Apple, Intel, Samsung, Xiaomi, Sharp, NEC, Futurewei, MediaTek, CATT, vivo, DENSO, ZTE, Huawei, HiSilicon, CMCC, Sequans, LGE, Convida). Note: Qualcomm and Sequans responded as a maybe (?).

- Ask them about feasibility of having a maximum eDRX cycle up set to 10485.76s for both RRC_IDLE and RRC_INACTIVE (e.g. considering NAS re-transmission timer)[3][6][7][14][2].

- This point (B) is supported by 19 companies (OPPO, Ericsson, Apple, Qualcomm, Intel, Samsung, Xiaomi, Sharp, NEC, Futurewei, CATT, vivo, ZTE, Huawei, HiSilicon, CMCC, Sequans, LGE, Convida).

- Ask them about the responsible node to decide eDRX parameters for RRC_INACTIVE [5].

- This point (C) is supported by 5 companies (Apple, Xiaomi, vivo, DENSO, CMCC).

- Ask them about feasibility of buffering data in CN when the UE is unreachable from CN perspective, i.e. when the UE is in eDRX in RRC_INACTIVE. If so, RAN2 assumes that CN provides an indication to RAN in case the mobile terminating traffic is pending towards the UE [6].

- This point (D) is supported by 5 companies (Ericsson, Apple, Samsung, vivo). Note: LGE responded as a maybe (?).

- RAN2 assumes that RAN provides necessary information to CN so that CN can estimate when the UE is unreachable, e.g. by providing the RAN paging configuration [6].

- This point (E) is supported by 5 companies (Ericsson, Apple, Intel, Samsung, NEC). Note: LGE responded as a maybe (?).

- Include WI objective and ask them whether there is any concern [6][15].

- This point (F) is supported by 4 companies (Ericsson, Xiaomi, vivo, Convida). Note: Sequans responded as a maybe (?).

- Include the conclusions on eDRX in TR 38.875, v2.0.0 [6][16].

- This point (G) is supported by 9 companies (Ericsson, Apple, Qualcomm, Samsung, NEC, CATT (partly), vivo, LGE, Convida). Note: Sequans responded as a maybe (?).

- Other aspects

- None

Rapporteur: 23 companies provided inputs to this discussion point. The proposal is to only agree on those with a large majority (points (A) and (B)) and mark the remaining for discussion. Note that point (C) is not included in the list of FFSs assuming that related Proposal 1 is agreed.

- [To agree] LS to SA2/CT1 will include following:

  - [To agree] [21/23] Include RAN2 agreements from this meeting, if any, related to NR eDRX design (e.g. node to control INACTIVE eDRX, whether eDRX config. for IDLE and INACTIVE are same or different), and ask them for any feedback, if any.

  - [To agree] [19/23] Ask them about feasibility of having a maximum eDRX cycle up set to 10485.76s for both RRC_IDLE and RRC_INACTIVE (e.g. considering NAS re-transmission timer).

- [To discuss] To discuss the following details in relation to Proposal 5:

  - [To discuss] [9/23] Include the conclusions on eDRX in TR 38.875, v2.0.0.

  - [To discuss] [5/23] Ask them about feasibility of buffering data in CN when the UE is unreachable from CN perspective, i.e. when the UE is in eDRX in RRC_INACTIVE. If so, RAN2 assumes that CN provides an indication to RAN in case the mobile terminating traffic is pending towards the UE.

  - [To discuss] [5/23] RAN2 assumes that RAN provides necessary information to CN so that CN can estimate when the UE is unreachable, e.g. by providing the RAN paging configuration

  - [To discuss] [4/23] Include WI objective and ask them whether there is any concern.

# Conclusion

Aiming to help with the meeting discussion/progress, the proposals are categorized starting with:

- [To agree] when there is large support and hence proposed for easy agreement.

- [To discuss] when there is substantial level of support and agreement may be possible.

- [FFS] when there is low support or companies propose new solutions or options to possibly consider further e.g. if there is sufficient support (understanding that these topic have not been discussed by all companies when providing their views in the different discussion points).

The proposals also start with a number: for the format [x], ‘x’ represents the number of supportive companies (i.e. these solutions are marked as FFS as the proposed solutions were not discussed by all companies) and, for the format [x/y], ‘x’ represents the number of supportive companies, and (y-x) the number of companies with different view.

The proposals captured are the following:

Proposal 1.	[To agree] [20/23] RAN controls the configuration of eDRX for RRC_INACTIVE.

Proposal 2.	[FFS] [7] If there is larger support, to further discuss in future meetings whether RAN can provide assistance information towards CN about DL data forwarding for UEs in RRC_INACTIVE configured with eDRX by RAN.

Proposal 3.	[To agree] [21/23] The configurations of the eDRX for RRC_IDLE and RRC_INACTIVE can be different.

Proposal 4.	[To discuss] To discuss the following details in relation to Proposal 3:

Proposal 4.1.	[To discuss] [9/21] To consider that any coordination between eDRX configurations for RRC_IDLE / RRC_INACTIVE is left up to network implementation

Proposal 4.2.	[To discuss] [11/21] To consider that RAN guarantees that the configuration provided for eDRX in RRC_INACTIVE is a sub-set of one provided for eDRX in RRC_IDLE. E.g. eDRX cycle in RRC_INACTIVE may be shorter than the eDRX cycle in RRC_IDLE, and the PTW should be either common or overlapping in the case either window is shorter than the other.

Proposal 4.3.	[To discuss] [12/21] To consider that different eDRX configuration refers to different eDRX cycle and when applicable, different PTW.

Proposal 5.	[To agree] LS to SA2/CT1 will include following:

Proposal 5.1.	[To agree] [21/23] Include RAN2 agreements from this meeting, if any, related to NR eDRX design (e.g. node to control INACTIVE eDRX, whether eDRX config. for IDLE and INACTIVE are same or different), and ask them for any feedback, if any.

Proposal 5.2.	[To agree] [19/23] Ask them about feasibility of having a maximum eDRX cycle up set to 10485.76s for both RRC_IDLE and RRC_INACTIVE (e.g. considering NAS re-transmission timer).

Proposal 6.	[To discuss] To discuss the following details in relation to Proposal 5:

Proposal 6.1.	[To discuss] [9/23] Include the conclusions on eDRX in TR 38.875, v2.0.0.

Proposal 6.2.	[To discuss] [5/23] Ask them about feasibility of buffering data in CN when the UE is unreachable from CN perspective, i.e. when the UE is in eDRX in RRC_INACTIVE. If so, RAN2 assumes that CN provides an indication to RAN in case the mobile terminating traffic is pending towards the UE.

Proposal 6.3.	[To discuss] [5/23] RAN2 assumes that RAN provides necessary information to CN so that CN can estimate when the UE is unreachable, e.g. by providing the RAN paging configuration

Proposal 6.4.	[To discuss] [4/23] Include WI objective and ask them whether there is any concern.

The following order is suggested for the online discussion:

Proposals for potential agreement

Proposal 1.	[To agree] [20/23] RAN controls the configuration of eDRX for RRC_INACTIVE.

Proposal 3.	[To agree] [21/23] The configurations of the eDRX for RRC_IDLE and RRC_INACTIVE can be different.

Proposal 5.	[To agree] LS to SA2/CT1 will include following:

Proposal 5.1.	[To agree] [21/23] Include RAN2 agreements from this meeting, if any, related to NR eDRX design (e.g. node to control INACTIVE eDRX, whether eDRX config. for IDLE and INACTIVE are same or different), and ask them for any feedback, if any.

Proposal 5.2.	[To agree] [19/23] Ask them about feasibility of having a maximum eDRX cycle up set to 10485.76s for both RRC_IDLE and RRC_INACTIVE (e.g. considering NAS re-transmission timer).

Proposals for potential discussion online

Proposal 4.	[To discuss] To discuss the following details in relation to Proposal 3:

Proposal 4.1.	[To discuss] [9/21] To consider that any coordination between eDRX configurations for RRC_IDLE / RRC_INACTIVE is left up to network implementation

Proposal 4.2.	[To discuss] [11/21] To consider that RAN guarantees that the configuration provided for eDRX in RRC_INACTIVE is a sub-set of one provided for eDRX in RRC_IDLE. E.g. eDRX cycle in RRC_INACTIVE may be shorter than the eDRX cycle in RRC_IDLE, and the PTW should be either common or overlapping in the case either window is shorter than the other.

Proposal 4.3.	[To discuss] [12/21] To consider that different eDRX configuration refers to different eDRX cycle and when applicable, different PTW.

Proposal 6.	[To discuss] To discuss the following details in relation to Proposal 5:

Proposal 6.1.	[To discuss] [9/23] Include the conclusions on eDRX in TR 38.875, v2.0.0.

Proposal 6.2.	[To discuss] [5/23] Ask them about feasibility of buffering data in CN when the UE is unreachable from CN perspective, i.e. when the UE is in eDRX in RRC_INACTIVE. If so, RAN2 assumes that CN provides an indication to RAN in case the mobile terminating traffic is pending towards the UE.

Proposal 6.3.	[To discuss] [5/23] RAN2 assumes that RAN provides necessary information to CN so that CN can estimate when the UE is unreachable, e.g. by providing the RAN paging configuration

Proposal 6.4.	[To discuss] [4/23] Include WI objective and ask them whether there is any concern.

Proposals for potential discussion in future meetings

Proposal 2.	[FFS] [7] If there is larger support, to further discuss in future meetings whether RAN can provide assistance information towards CN about DL data forwarding for UEs in RRC_INACTIVE configured with eDRX by RAN.

# Annex: companies’ point of contact

| Company | Point of contact | Email address |
| --- | --- | --- |
| Intel Corporation | Marta Martinez Tarradell | marta.m.tarradell@intel.com |
| Nokia, Nokia Shanghai Bell | Jussi-Pekka Koskinen | Jussi-pekka.koskinen@nokia.com |
| OPPO | Haitao Li | lihaitao@oppo.com |
| Apple | Naveen Palle | naveen.palle@apple.com |
| Qualcomm | Linhai He | linhaihe@qti.qualcomm.com |
| Sharp | Lei Liu | lei.liu@cn.sharp-world.com |
| Futurewei | Yunsong Yang | yyang1@futurewei.com |
| MediaTek | Pradeep Jose | pradeep[dot]jose[at]mediatek[dot]com |
| CATT | Pierre Bertrand | pierrebertrand@catt.cn |
| ZTE | LiuJing | liu.jing30@zte.com.cn |
| Sequans | Noam Cayron | noam.cayron@sequans.com |
| Convida | Jerome Vogedes | Vogedes.jerome@convidawireless.com |

# Reference

- R2-2102681	Discussions on eDRX configuration	Qualcomm Incorporated.

- R2-2102736	Discussion on eDRX  for RedCap UEs	OPPO.

- R2-2102852	Extend paging DRX for RedCap devices	Intel Corporation	discussion

- R2-2102862	Discussion on e-DRX for Redcap Devices	Xiaomi Communications	discussion

- R2-2102962	Work on eDRX for RedCap UEs	DENSO CORPORATION.

- R2-2102965	Discussion of eDRX for RedCap	Ericsson	discussion	NR_redcap-Core

- R2-2103039	Discussion on eDRX for RedCap UE	ZTE Corporation, Sanechips.

- R2-2103112	Discussion On eDRX for NR RRC Inactive and Idle	CATT.

- R2-2103530	eDRX for REDCAP	Nokia, Nokia Shanghai Bell.

- R2-2103622	eDRX for RedCap UEs in RRC_IDLE/RRC_INACTIVE	LG Electronics UK.

- R2-2103707	Discussion on eDRX for RedCap	CMCC.

- R2-2103783	Further considerations for eDRX	MediaTek Inc..

- R2-2103887	RedCap UE power-saving with 2.56 DRX cycle	Apple.

- R2-2104059	eDRX for RedCap UE	Huawei, HiSilicon.

- RP-210918, Revised WID on support of reduced capability NR devices, Rel-17 WID, March 2021.

- TR 38.875, Study on support of reduced capability NR devices, Rel-17.