3GPP TSG-RAN WG2 #113bis-e	R2-2104500

Electronic meeting, 2021-04-12 – 2021-04-20

Agenda Item:	8.11.4

Source:	Ericsson

Title:	On demand PRS for energy savings and view on UE-initiated Request

Document for:	Discussion, Decision

# 1	Introduction

As per the WID agreement, one of the agendas that RAN2 should discuss and specify is in the below.

- Specify on-demand transmission and reception of DL PRS for DL and DL+UL positioning for UE-based and UE-assisted positioning solutions, including: [RAN2, RAN1, RAN3]

  - UE-initiated request of on-demand DL PRS transmission;

  - LMF (network)-initiated request of on-demand DL PRS transmission;

RAN2 performed the study and concluded that dynamic PRS configuration is beneficial. In this paper, we discuss how energy efficient DL-PRS transmission should be enabled.

# 2	Discussion

## 2.1 UE-Initiated Request

Before going into details of UE-Initiated request; RAN2 needs to take the below fundamentals into considerations

- DL-PRS Resource is owned by gNB. The transmitted DL-PRS is used by all UEs (shared) and not exclusive for one UE

- The configuration is provided via either RRC Broadcast or LPP dedicated signaling

- NW always strives to provide best configuration for all UEs.

During SI phase, there were 2 variants of UE-Initiated; one based upon on framework similar to “On demand SI request”, however this was concluded to be inefficient because of massive signalling involved.

Another variant was UE provides the recommendation on its preferred DL-PRS configuration.

It has been mentioned that UE can initiate number of requests such as Beam On/OFF, preferred DL-PRS configuration, occasion, periodicity, number of gNBs/TRPs etc.

The question that RAN2 need to address is:

a) What if UE1 request to turn off Beam A and UE2 request to turn off Beam B?

b) What if UE1 says I need 10 TRPs and UE2 says I need 20 TRPs?

c) What if UE1 says I need large BW and short periodicity and UE2 says I am contend with small BW and large periodicity; what is the incentive from NW perspective as why not to provide also best config to UE 2 since it is already needed to commit to UE 1

RAN2 discusses and addresses the questions before proceeding to UE-Initiated Request.

UE-Initiated request in our view was that UE provides feedback on the current DL-PRS transmission to the LMF so that the LMF aggregates the result from several UEs and retunes the transmitted DL-PRS if need be via broadcast and/or via dedicated signalling to be applicable for several UEs.

## 2.2	Energy Savings

Considering a TRP’s transmit power requirement to transmit X number of resource sets and Y number of resources per resource set is Z watts. However, if the TRP is only required to transmit half the resource set and half the resources per resource set; there can be potential energy savings by one fourth (Z/4 watts. It is essential that 3gpp eco-system provides a solution which promotes energy savings.

There can be considerable energy transmission saving when the number of resource sets and resources required for DL-PRS transmission is reduced. RAN2 should strive to provide solution which are energy efficient.

RAN2 to agree to provide solution which leads to reduction of energy consumption for DL-PRS transmission.

As discussed during study item; PRS might need to be transmitted in a beamformed fashion to compensate the higher path loss at higher carrier frequencies. The PRS transmission to all beam sweeping directions results in an unnecessary transmission of PRSs. Thus, a solution is required to identify a mechanism to optimize the PRS transmission.

By selecting the optimum number of beams for PRS transmission and by switching off the PRS transmission in a more opportunistic way, it should be possible to be energy efficient. For this to happen,

- the UE should provide the RSRP result of PRS being transmitted from different beams.

- LMF should compute the PRS utilization from different beams.

- LMF should provide a list of low utilized or un-utilized beams to gNB to be turned off.

It is evident that in order to provide energy friendly solutions, UE co-operation would be needed. Basically, to identify which TRPs/beams should be used for PRS transmission and further to determine the usefulness of PRS that is being transmitted, UE feedback is required.

It is critical to have UE feedback on identification of TRPs to be used for DL-PRS transmission and on the usefulness of transmitted PRS.

RAN2 to discuss what sort of feedback and how those feedbacks can be provided by UE identifying the needed TRPs/beams for DL-PRS transmission to ensure 3gpp ecosystem provides energy efficient solution.

Further signalling over NRPPa would be required, hence RAN2 should request RAN3 to design NRPPa signalling message from LMF to the gNB in order to send meaningful PRS information, such as e.g. aggregated PRS measurement report. Such report could include the PRS resources and the RSRP measurement information from the best/worst available TRPs/cells on each frequency layer. Once it has received it, the gNB would take this PRS report into account for efficiently activating/deactivating the beams for PRS transmission.

RAN2 to request RAN3 to provide solution/signalling for providing PRS beam utilization in NRPPa to reduce PRS overhead.

### 2.1.1	GDOP Report and LOS/NLOS detection report

[2], [3] provides the analysis and result showing how geometry plays important role in positioning performance. The Geometric dilution of Precision (GDOP) is an important attribute that can influence the accuracy of location in positioning methods which use multilateration. It describes error caused by the relative position of the base stations, cells or beams etc. If the base stations (beams/cells) location are too close to each other and not well spread around the UE, the reported RSTD value would suffer from poor GDOP resulting in location estimation with large error. However, if they have certain distance/angular separation between them it can result in good GDOP (multilateration) which can identify the UE location more precisely. In a simple form GDOP can be computed as a ratio of accuracy of position fix to the statistical accuracy of the ranging measurements [3].

The

geometric dilution of precision (GDOP) is deﬁned as the ratio

of the accuracy of a position ﬁx to the statistical accuracy of

Based upon GDOP result and analysis, LMF may learn whether certain TRPs are contributing to positioning performance or not. Hence, this result can help LMF to filter TRPs for PRS transmission.

GDOP results, DL-PRS RSRP and positioning ranging error/uncertainties are provided to LMF by UE.

Further, RAN2 should also explore any other possibilities where UE based UE can provide feedback on the measured PRS and obtain UE specific DL-PRS configuration.

RAN2 to discuss ways on how UE operating in UE based mode can provide feedback in order to obtain suitable DL-PRS configurations.

Further in the WID, it has been specified that one of the WI agenda is as below.

- Study and specify, if agreed, the enhancements of information reporting from UE and gNB for multipath/NLOS mitigation [RAN1, RAN2, RAN3]

As part of this, it may be possible to classify whether the received signal is in LOS/NLOS.

LOS/NLOS classification, if known, is used to identify suitable TRPs from a certain location.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	There can be considerable energy transmission saving when the number of resource sets and resources required for DL-PRS transmission is reduced. RAN2 should strive to provide solution which are energy efficient.

Observation 2	It is critical to have UE feedback on identification of TRPs to be used for DL-PRS transmission and on the usefulness of transmitted PRS.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 discusses and addresses the questions before proceeding to UE-Initiated Request.

a) What if UE1 request to turn off Beam A and UE2 request to turn off Beam B?

b) What if UE1 says I need 10 TRPs and UE2 says I need 20 TRPs?

c) What if UE1 says I need large BW and short periodicity and UE2 says I am contend with small BW and large periodicity; what is the incentive from NW perspective as why not to provide also best config to UE2 since it is already needed to commit to UE 1

Proposal 2	RAN2 to agree to provide solution which leads to reduction of energy consumption for DL-PRS transmission.

Proposal 3	RAN2 to discuss what sort of feedback and how those feedbacks can be provided by UE identifying the needed TRPs/beams for DL-PRS transmission to ensure 3gpp ecosystem provides energy efficient solution.

Proposal 4	RAN2 to request RAN3 to provide solution/signalling for providing PRS beam utilization in NRPPa to reduce PRS overhead.

Proposal 5	GDOP results, DL-PRS RSRP and positioning ranging error/uncertainties are provided to LMF by UE.

Proposal 6	RAN2 to discuss ways on how UE operating in UE based mode can provide feedback in order to obtain suitable DL-PRS configurations.

Proposal 7	LOS/NLOS classification, if known, is used to identify suitable TRPs from a certain location.

# 4	References

RP-210903 , “Revised WID on NR Positioning Enhancements“, March 16 - 26, 2021

Sven Fischer, “OTDOA Positioning in 3gpp LTE”, June 2014

Ian Sharp et al; “GDOP Analysis for Positioning System Design”, IEEE Vehicular Technology, September 2009