3GPP TSG-RAN WG2 Meeting #113bis electronic					R2-2104306

Online, April 12 – April 20, 2021

Agenda Item:	10.6

Source: 			Session chair (CMCC)

Title: 			Report from SON/MDT session

Document for:	Approval

Recording of voice or video at meetings is not used in 3GPP. This applies also to this e-Meeting. At this e-Meeting, no specific actions are taken to prevent the recording of web conferences. Companies that have concerns related to recordings, if any, may express those by email in the main meeting organizational thread [AT112e][000]

Organizational:

- LSs – contact companies should flag LSs that need presenting.  Otherwise we will directly note them

- Running CRs will be endorsed to be used as baseline and moved to email discussion.  Further agreements will be captured on that baseline CR.

- Only Email discussions and summary discussions will be treated during e-meetings (indicated clearly in the meeting notes)

- All organization emails and notes will be shared over the following email discussion throughout the two meeting weeks:

[AT113bis-e][800][SON/MDT] Organizational Hu

Scope:

    - Share plans for the meetings and list of ongoing email discussions for the sessions related to SON/MDT

    - Share meetings notes and agreements for review and endorsement

## 6.6	SON/MDT support for NR

(NR_SON_MDT-Core; leading WG: RAN3; REL-16; started: Jun 19; Completed June 20; WID: RP-191776).

Documents in this agenda item will be handled in a break out session

Tdoc Limitation: 7 tdocs. See also tdoc limitation for Agenda Item 6

### 6.6.1	General and stage-2 corrections

Including incoming LSs, TS 37.320 corrections

R2-2102632	Reply LS on MDT Stage 2 and Stage 3 alignment (R3-211140; contact: Ericsson)	RAN3	LS in	Rel-16	eMDT_UMTSLTE-Core	To:SA5, RAN2	Cc:-

=>	Noted without online presentation.

R2-2102641	Reply LS on limitation of Propagation of immediate MDT configuration in case of Xn inter-RAT HO (R3-211335; contact: ZTE)	RAN3	LS in	Rel-16	NR_SON_MDT	To:SA5, RAN2	Cc:-

=>	Noted without online presentation.

R2-2102671	Reply LS on propagation of user consent related information during Xn inter-PLMN handover (S3-211330; contact: Ericsson)	SA3	LS in	Rel-16	NR_SON_MDT-Core	To:RAN3	Cc:RAN2, SA5

=>	Noted without online presentation.

R2-2102672	Reply LS on the user consent for trace reporting (S3-211338; contact: Qualcomm)	SA3	LS in	Rel-16	NR_SON_MDT-Core	To:RAN2, SA5	Cc:RAN3

=>	Noted without online presentation.

R2-2104430	Summary for 6.6.1 General and stage-2 corrections	CMCC

R2-2103549	Clarification on RAN measurements collection period	Nokia, Nokia Shanghai Bell	CR	Rel-16	37.320	16.4.0	0105	-	F	NR_SON_MDT-Core

R2-2104037	Clarification on Average UE throughout measurement	Samsung	discussion	NR_SON_MDT-Core

R2-2104199	Correction to 37320 on MDT context handling	ZTE Corporation, Sanechips	CR	Rel-16	37.320	16.4.0	0106	-	C	NR_SON_MDT-Core

The flowing email discussion will be arranged during the meeting week:

[AT113bis-e][811][NR/R16 SON/MDT]  Stage-2 corrections (CMCC, Nokia)

-	Step1: discuss the changes in R2-2103549, R2-2104037 and R2-2104199 and collect the companies’ views on the changes.

-	Step2: Try to agree the changes under majority support @ 11:11 UTC Thursday 15/04/2021

-	Step3: capture the agreed changes related to stage-2 corrections into one merged 37.320 CR

Intended outcome: agreed CR (R2-2104433)

Deadline: 14:00 UTC Monday 19/04/2021

R2-2104444	Merged Corrections to TS 37.320	CMCC, Nokia	CR	Rel-16	37.320	16.4.0	0107	-	C	NR_SON_MDT-Core

=>	CR is agreed in principle.

R2-2103819	[Draft] Reply LS on MDT Stage 2 and Stage 3 alignment	Ericsson	LS out	Rel-16	NR_SON_MDT-Core	To:RAN3	Cc:SA5

### 6.6.2	TS 38.314 corrections

R2-2103821	On corrections to packet loss rate measurements	Ericsson	CR	Rel-16	38.314	16.3.0	0014	-	F	NR_SON_MDT-Core

### 6.6.3	RRC corrections

Including outcome of email discussion [Post113-e][850][NR16 SON/MDT]  Timestamp of event triggered MDT (Ericsson)

R2-2103820	Report of email discussion [Post113-e][NR/R16 SON/MDT] Timestamp of event triggered MDT	Ericsson	discussion

=>	RAN2 confirms that the UE behavior for the event L1 based logging of measurements in logged MDT is as per the current field description of the event L1.

=>	RAN2 confirms that the UE behavior for the outOfCoverage event based logging of measurements in logged MDT is as per the current field description of the outOfCoverage event.

R2-2102909	Correction on periodic logging in any cell selection state	Samsung Electronics Co., Ltd	CR	Rel-16	38.331	16.4.1	2488	-	F	NR_SON_MDT-Core

=>	The change is agreed and will be merged to the big CR.

R2-2102911	Correction on RLF report content determination for EUTRA frequency measurements	Samsung Electronics Co., Ltd	CR	Rel-16	38.331	16.4.1	2489	-	F	NR_SON_MDT-Core

=>	The change is agreed and will be merged to the big CR.

R2-2102912	Correction on RLF report for re-connection	Samsung Electronics Co., Ltd	CR	Rel-16	36.331	16.4.0	4610	-	F	NR_SON_MDT-Core

=>	The change is agreed and will be merged to the big CR.

R2-2103101	Correction on Inter-RAT MRO in 38.331	CATT	CR	Rel-16	38.331	16.4.1	2500	-	F	NR_SON_MDT-Core

=>	The change is agreed and will be merged to the big CR.

R2-2103812	on ReconnectionCellId and timeUntilReconnection field	Ericsson	CR	Rel-16	36.331	16.4.0	4623	-	F	NR_SON_MDT-Core

=>	The change is agreed and will be merged to the big CR. The wording can be furthered enhanced through 810.

R2-2103815	On the lack of PLMN identity check in case of anyCellSelected state related logging	Ericsson	CR	Rel-16	38.331	16.4.1	2536	-	F	NR_SON_MDT-Core

=>	Waiting progress from LTE session

R2-2103817	On TimeUntilReconnection and ReconnectionCellID logging as part of RLF report	Ericsson	CR	Rel-16	38.331	16.4.1	2537	-	F	NR_SON_MDT-Core

=>	The intention is agreed and correct the wording through email discussin for the merged big CR.

R2-2103818	On releasing WLAN-BT-Sensor configurations upon returning from inactive	Ericsson	CR	Rel-16	38.331	16.4.1	2538	-	F	NR_SON_MDT-Core

=>	The change is agreed and will be merged to the big CR.

R2-2103822	ReconnectCellID in multi PLMN scenarios	Ericsson	discussion

=>	The change is not pursued in R16

R2-2103875	Discussion on RLF reporting	Apple, Ericsson, MediaTek Inc.	discussion	Rel-16	NR_SON_MDT-Core

=>	The changes in text proposal are agreed and will be merged into big CR.

R2-2103876	Clarification on RA reporting	Apple	draftCR	Rel-16	38.331	16.4.1	F	NR_SON_MDT-Core

=>	Postponed to next meeting.

R2-2104002	Discussion on the location reporting in inter-RAT measurement for immediate MDT	Huawei, HiSilicon	discussion	Rel-16	NR_SON_MDT-Core

=>	The changes are agreed and will be merged to the big CR.

R2-2104198	CR to 36331 on RLF report and logged MDT report	ZTE Corporation, Sanechips	CR	Rel-16	36.331	16.4.0	4636	-	F	NR_SON_MDT-Core

=>	The third and second changes are agreed and will be merged to the big CR.

R2-2103766	Clarification on LocationInfo reporting for SON	NTT DOCOMO, INC.	CR	Rel-16	36.331	16.4.0	4621	-	F	NR_ENDC_SON_MDT_enh-Core

=>	change to “NOTE x:	Any subsequent measurement report includes RLF report and SCGFailureInformationNR.”

=>	will be merged into big CR. Further clarification on UE behavior can be discussed through email discussion 810.

R2-2103073	Handling of user contest for location reporting in SONMDT 	QUALCOMM Incorporated 	discussion

R2-2104003	Discussion on the user consent for trace reporting	Huawei, HiSilicon	discussion	Rel-16	NR_SON_MDT-Core

[AT113bis-e][810][NR/R16 SON/MDT]  Stage-3 corrections (Ericsson, Huawei)

-	capture the agreements related to stage-3 corrections into one merged 38.331 and 36.331 CRs

Intended outcome: agreed CRs (R2-2104435 for 38.331, R2-2104436 for 36.331)

Deadline: Thursday 15/04/2021

R2-2104435	SON-MDT Changes agreed in RAN2#113-bis meeting	Ericsson, Huawei	CR	Rel-16	38.331	16.4.1	????	-	F	NR_SON_MDT-Core

=>	Add the CR number and correct the document number.

=>	With these changes the CR is agreed in principle in R2-2104599

R2-2104599	SON-MDT Changes agreed in RAN2#113-bis meeting	Ericsson, Huawei	CR	Rel-16	38.331	16.4.1	2578	-	F	NR_SON_MDT-Core

=>	In principle agreed

R2-2104436	SON-MDT Changes agreed in RAN2#113-bis meeting	Ericsson, Huawei	CR	Rel-16	36.331	16.4.0	????	-	F	NR_ENDC_SON_MDT_enh-Core

=>	Add the CR number and correct the document number.

=>	With these changes the CR is agreed in principle.

R2-2104600	SON-MDT Changes agreed in RAN2#113-bis meeting	Ericsson, Huawei	CR	Rel-16	36.331	16.4.0	4650	-	F	NR_ENDC_SON_MDT_enh-Core

=>	In principle agreed

## 8.13	SON/MDT

(NR_ENDC_SON_MDT_enh-Core; leading WG: RAN3; REL-17; WID: RP-201281)

Time budget: 1 TU

Tdoc Limitation: 6 tdocs

Email max expectation: 6 threads

### 8.13.1	Organizational

R2-2102639	LS on information needed for MRO in SCG Failure Report (R3-211332; contact: Samsung)	RAN3	LS in	Rel-17	NR_ENDC_SON_MDT_enh	To:RAN2	Cc:-

=>	Noted without online presentation.

R2-2102629	Reply LS on on energy efficiency (R3-207014; contact: Ericsson)	RAN3	LS in	Rel-17	NR_ENDC_SON_MDT_enh	To:SA5	Cc:RAN2, SA

=>	Noted without online presentation.

R2-2102640	LS on the details of logging forms reported by the gNB-CU-CP, gNB-CU-UP and gNB-DU under measurement pollution conditions (R3-211334; contact: Ericsson)	RAN3	LS in	Rel-17	NR_ENDC_SON_MDT_enh	To:SA5, RAN2	Cc:-

=>	Noted without online presentation.

### 8.13.2	SON

#### 8.13.2.1	Handover related SON aspects

Including conditional handover and DAPS

Including outcome of email discussion [Post113-e][851][NR17 SON/MDT]  HO related SON changes (Ericsson)

R2-2103945	[Post113-e][851][NR17 SON/MDT] HO related SON changes (Ericsson)	Ericsson	discussion	NR_ENDC_SON_MDT_enh-Core

=>	RAN2 to focus on the following CHO scenarios at least:

a.	Scenario 1 (too late HO): 1a, 1b, 1c, 1d

b.	Scenario 2 (too early HO): 2a, 2b

c.	Scenario 3 (HO to wrong cell): 3a, 3b, 3c, 3e, 3f

FFS the need to merge certain scenarios, e.g. 1b/1c, 2a/2b

Agreements:

1	Include in the RLF-report for CHO the following:

a.	Configured CHO execution condition(s) (A3 and/or A5 event configuration, TTT values)

b.	Fulfilled CHO execution condition(s), i.e. whether A3 and/or A5 event was fullfilled, for the cell(s) in which CHO execution was triggered.

c.	Latest radio measurement results of the candidate target cells

Inclusion of a) and c) are subject to the RAN3 reply to the RAN2 LS R2-2102149.

Try to reuse existing mechanism as much as possible.

2	Include in the RLF report for CHO the following information:

a.	Indication of whether a measured neighbour cell included in the existing measResultNeighCells was a CHO candidate cell or not.

b.	List of candidate cells IDs.

Inclusion of a) and b) are subject to the RAN3 reply to the RAN2 LS R2-2102149

3	The following information in the RLF report for CHO are needed:

b.	CHOCellId, to indicate the selected CHO cell after the first connection failure and before the reestablishment

c.	CellID to indicate the cell in which the UE attempted the second reestablishment after failure of the first reestablishment following an HOF/RLF.

How to provide these information is FFS.

=>	RAN2 to focus on the following DAPS scenarios:

a.	Scenario 1 (too late DAPS): 1a, 1b

b.	Scenario 2 (too early DAPS): 2a, 2b/2c

c.	Scenario 3 (DAPS to wrong cell): 3a, 3b/3c

FFS whether to merge scenarios 2b/2c and 3b/3c.

Agreements:

1	Include in the RLF report for DAPS HO, the following measurements (reuse the legacy mechanism and IEs):

a.	Measurements of neighbour cells when HOF or RLF occurs

2	RAN2 to agree the intention of the following timers:

a.	Time elapsed since DAPS HO execution until RLF occurs in source cell before fallback

b.	Time elapsed since DAPS HO execution until RLF occurs in source cell after fallback

c.	The elapsed time between the execution of DAPS and RLF in target cell

FFS if for the above timers the existing timers can be reused.

3	Include in the RLF report for DAPS HO the following information:

a.	RLF-cause of the RLF occurred in the source cell while performing a DAPS HO

b.	Explicit indicator for DAPS handover failure

4	At least the following triggering conditions are applied for generating an HO Success Report in the case that the HO succeeds:

a.	The UE logs the HO success report if, while doing HO, T310 value exceeds a threshold

b.	The UE logs the HO success report if, while doing HO, T312 value exceeds a threshold

c.	The UE logs the HO success report if, while doing HO, T304 exceeds a threshold

d.	In case of DAPS, if the UE gets an RLF in the source while doing DAPS.

[AT113bis-e][801][NR/R17 SON/MDT]  Successful HO Report (Ericsson)

-	Step 1: Collect the companies’ view on the proposals in 2.3 in R2-2103945. Deadline is 16:00 UTC 15/04/2021.

-	Step 2: Based on majority view provide the agreeable proposals

-	Step 3: Collect the companies’ view on the new proposals and goto Step 2 until 04:00 UTC Friday 16/04/2021.

-	Step 4: Providing final proposals for CB session. Note that in CB session there will not be technical discussion again. We try to directly agree the final proposals.

Intended outcome: Easily agreeable proposals (R2-2104438)

Deadline: Friday 16/04/2021

R2-2104572	[AT113bis-e][801][NR/R17 SON/MDT] Successful HO Report	Ericsson

Agreements:

1	RAN2 to focus on the following scenarios for HO Success Report:

a.	Scenario 1 (ordinary HO): 1a, 1b

b.	Scenario 2 (CHO): 2a, 2b

c.	Scenario 3 (DAPS): 3a

2	RAN2 for further discuss whether the following scenarios should be considered under the RLF report or under the HO success report:

a.	Scenario 2c

b.	Scenario 3b

3	The following radio related measurements are as part of the successful HO report:

a.	Latest radio measurement results of the candidate target cells in the case of conditional HO. FFS best cell(s) should be included in.

b.	Flag to indicate RLF issues in source cell during DAPS HO

4	The following time-related measurements are as part of the successful HO report:

a.	Time elapsed between the CHO execution towards the target cell and the corresponding latest CHO configuration received for the selected target cell

5	Location information is included as part of the successful HO report.

Proposal 4	RAN2 to further discuss the need of the following parameters as part of the successful HO report:

a.	Latest radio link quality of neighbour cells before HO command was received for all HO types.

b.	Configured CHO execution condition(s), e.g. A3 and/or A5 event configuration, of the candidate target cells. The inclusion of this parameter depends on the RAN3 reply to the RAN2 LS R2-2102149.

c.	The radio quality of source cell when ConditionalReconfiguration is received before conditional handover execution condition is satisfied

d.	Latest radio link quality of source cell before HO command was received in the case of DAPS.

Proposal 6	RAN2 to further discuss the need of the following time-related measurements as part of the successful HO report:

a.	Elapsed time for T310 timer for normal HO

b.	Elapsed time for T310 timer for Conditional HO

R2-2104502	Summary of AI 8.13.2.1 Handover related SON aspects	Ericsson

R2-2103065	HO related SON changes 	QUALCOMM Incorportated 	discussion	Rel-17

R2-2103098	Discussion on CHO and DAPS Aspect	CATT	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

R2-2103142	Further consideration on handover related SON	OPPO	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

R2-2103157	Discussion on CHO configuration optimization aspects	China Telecommunication	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

R2-2103164	Discussion on signalling and content of DAPS HO report	vivo	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

R2-2103385	SON Enhancements for CHO	Lenovo, Motorola Mobility	discussion	Rel-17

R2-2103386	SON Enhancement for DAPS Handover	Lenovo, Motorola Mobility	discussion	Rel-17

R2-2103550	Further clarifications on MRO	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

R2-2103709	Further consideration on SON Enhancement for CHO	CMCC	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

R2-2103710	SON Enhancement for DAPS	CMCC	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

R2-2103731	Discussion on handover related SON aspects	Huawei, HiSilicon	discussion	Rel-17

R2-2103933	Introducing additional CHO related failure/ success info, including multiple event	Samsung Telecommunications	discussion	NR_ENDC_SON_MDT_enh-Core

R2-2103944	Handover-related SON aspects	Ericsson	discussion	NR_ENDC_SON_MDT_enh-Core

R2-2104045	SON Enhancements for DAPS HO	Samsung	discussion	NR_ENDC_SON_MDT_enh-Core

R2-2104070	Discussion on RLF report for DAPS	SHARP Corporation	discussion	NR_ENDC_SON_MDT_enh-Core

R2-2104192	Remaining issues on RLF report enhancements	ZTE Corporation, Sanechips	discussion	Rel-17

#### 8.13.2.2	2-step RA related SON aspects

Including outcome of email discussion [Post113-e][852][NR17 SON/MDT]  2 step RA and other SON changes (CATT)

R2-2103093	Report of [Post113-e][852][NR17 SON/MDT] 2 step RA and other SON changes (CATT)	CATT	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core	Late

[AT113bis-e][802][NR/R17 SON/MDT]  2 step RA and other SON enhancements (CATT)

-	Step 1: Collect the companies’ view on the proposals in R2-2103093. Deadline is 16:00 UTC 15/04/2021.

-	Step 2: Based on majority view provide the agreeable proposals

-	Step 3: Collect the companies’ view on the new proposals and goto Step 2 until 04:00 UTC Friday 16/04/2021.

-	Step 4: Providing final proposals for CB session. Note that in CB session there will not be technical discussion again. We try to directly agree the final proposals.

-	Step 5: drafting the LS to RAN3 based on the potential agreements from the final proposals…

Intended outcome: Easily agreeable proposals (R2-2104439)and LS draft (R2-2104440)

Deadline: Friday 16/04/2021

R2-2104536	Report of [AT113b-e][802][NR/R17 SON/MDT]  2 step RA and other SON enhancements (CATT)	CATT

2-step RA report enhancements

Agreements:

1	The RA report includes an explicit indication per RA attempt that enables the network to know that the fallback from 2-step RA to 4-step RA was performed by the UE.

2	RAN2 already agreed “UE includes the measured RSRP of DL pathloss reference obtained just ‎before performing RACH ‎procedure in 2step RA report. FFS how to reduce the report ‎overhead.‎”

=> RAN2 to discuss and ‎reply LS to R2-2008731, focusing on agreements and no further observations. ‎

[AT113bis-e][888][NR/R17 SON/MDT]  Draft reply LS to reply R2-2008731 (CATT)

-Inform RAN3 the agreements from RAN2#113bis related to 2-step RA report enhancements

Outcome: approved LS

Deadline: 15:00 UTC 20/04/2021

SgNB RACH report

FFS:	Proposal 11: UE reports the SN RACH report to the MN, and then MN sends the SN RACH report to the SN.

Agreements:

Mobility history information enhancements

1	If PSCell MHI is introduced, at least include PSCell ID (may include CGI or frequency+PCI) and the time UE stayed in each PSCell into PSCell MHI.

‎

Other RACH Optimization

Proposal 12: The following RACH optimization can be further considered:

a): UE also includes the PCell in the RA report in case the RA occurred in an SCell.

UL/DL coverage imbalance

Proposal 13A: FFS how to identify and solve the problem about UL/DL coverage imbalance.

Enhancement for SN Change failure

Proposal 15A: FFS on enhancements for SN change failure, and request from R3 LS R2-2102639 is baseline for further discussions.

R2-2104625	Reply LS on RACH report for 2-step RACH CATT

=>	LS is approved

R2-2104292	Summary of AI 8.13.2.2	CATT

R2-2103094	[Draft] Reply LS on RACH report for 2-step RACH	CATT	LS out	Rel-17	NR_ENDC_SON_MDT_enh-Core	To:RAN3

R2-2103165	Discussion on signalling model of 2-step RACH report	vivo	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

R2-2103421	Discussion on 2-step RACH reporting in SON	OPPO	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

R2-2103551	Remaining Issues and New Aspects in 2-step NR UE Report	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

R2-2103711	SON Enhancement for 2-step RA	CMCC	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

R2-2103732	Discussion on 2 step RA related SON aspects	Huawei, HiSilicon	discussion	Rel-17

R2-2103942	2-Step RA information for SON purposes	Ericsson	discussion	NR_ENDC_SON_MDT_enh-Core

R2-2104055	SON Enhancements for 2SRA	Samsung	discussion	NR_ENDC_SON_MDT_enh-Core

R2-2104057	Discussion on RA information for 2-step RA	SHARP Corporation	discussion	NR_ENDC_SON_MDT_enh-Core

R2-2104193	Remaining issues on RA related enhancements	ZTE Corporation, Sanechips	discussion	Rel-17

#### 8.13.2.3	Other WID related SON features

Including RAN3 input features, successful handover report, MRO for SN change failure, RACH optimization enhancements, UL-DL coverage mismatch, …

-	All the documents in 8.13.2.3 will not be treated in this meeting.

-	Encourage companies to provide joint contributions for the new topics (except NR-U related enhancements, successful handover report, MRO for SN change failure, RACH optimization enhancements, UL-DL coverage mismatch) to show common interest.

R2-2103066	NR-U Related Enhancements  	QUALCOMM Incorporated	discussion	Rel-17

R2-2103095	Solution for the UE RACH Report for SN	CATT	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

R2-2103096	Further Consideration on PSCell MHI	CATT	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

R2-2103099	On Successful HO Report	CATT	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

R2-2103148	Consideration on successful handover report and UE history information in EN-DC	OPPO	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

R2-2103298	Discussion on signalling aspects of successful handover report	NEC	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

R2-2103387	MRO for Inter-RAT handover	Lenovo, Motorola Mobility	discussion	Rel-17

R2-2103388	MRO for fast MCG link recovery	Lenovo, Motorola Mobility	discussion	Rel-17

R2-2103552	Discussion on other SON aspects	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

R2-2103553	MPE impact on MRO	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

R2-2103712	Discussion on Successful Handover Report	CMCC	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

R2-2103713	Further consideration on UL-DL coverage mismatch	CMCC	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

R2-2103733	Discussion on other SON aspects	Huawei, HiSilicon	discussion	Rel-17

R2-2103755	Discussion on enhancement of RLF report	NTT DOCOMO, INC.	discussion	Rel-17	Late

R2-2103943	Other WID related SON features	Ericsson	discussion	NR_ENDC_SON_MDT_enh-Core

R2-2104056	SON Enhancements for Successful HO Report	Samsung	discussion	NR_ENDC_SON_MDT_enh-Core

R2-2104058	Other SON Enhancements	Samsung	discussion	NR_ENDC_SON_MDT_enh-Core

R2-2104071	Successful HO report in DAPS	SHARP Corporation	discussion

R2-2104194	Further considerations on successful HO report	ZTE Corporation, Sanechips	discussion	Rel-17

R2-2104195	Consideration on RAN3 concerned issues	ZTE Corporation, Sanechips	discussion	Rel-17

### 8.13.3	MDT

#### 8.13.3.1	Immediate MDT enhancements

including M5/M6/M7 in all bearer type scenarios, immediate MDT for MR-DC

Including outcome of email discussion [Post113-e][853][NR17 SON/MDT]  IMM MDT (Huawei)

R2-2104006	Report of [Post113-e][853][NR17 SON/MDT]  IMM MDT	Huawei	report	Rel-17	NR_ENDC_SON_MDT_enh-Core

[AT113bis-e][803][NR/R17 SON/MDT]  ]  IMM MDT (Huawei)

-	Step 1: Collect the companies’ view on the proposals in R2-2104006. Deadline is 16:00 UTC 15/04/2021.

-	Step 2: Based on majority view provide the agreeable proposals

-	Step 3: Collect the companies’ view on the new proposals and goto Step 2 until 04:00 UTC Friday 16/04/2021.

-	Step 4: Providing final proposals for CB session. Note that in CB session there will not be technical discussion again. We try to directly agree the final proposals.

Intended outcome: Easily agreeable proposals (R2-2104441)and LS draft(R2-2104442)

Deadline: Friday 16/04/2021

R2-2104441	Report of [AT113b-e][803][NR/R17 SON/MDT]  IMM MDT	Huawei

Agreements:

1	For MN terminated SCG bearer and SN terminated MCG bearer, the terminated node, e.g., MN in case of MN terminated SCG bearer,configures the configuration to UE.

=>	RAN2 understanding is that for the accuracy of the result, the M6 result can be indicated with data marker (duplication indicator).

=>	All the immediate MDT configurations and reporting in EN-DC scenario (i.e. section 5.4.1.3 Immediate MDT for MR-DC in TS 37.320) are also applicable for (NG)EN-DC, NE-DC and NR-DC.

R2-2104295	Summary of 8.13.3.1 Rel-17 Imm MDT	Huawei

R2-2103064	 On the accuracy of M5 and M7 measurements in split-bearer	QUALCOMM Incorporated 	discussion	Rel-17

R2-2103100	Further Consideration on Immediate MDT Enhancements	CATT	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

R2-2103166	Support of immediate MDT in MR-DC	vivo	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

R2-2103810	On Immediate MDT Enhancements	Ericsson	discussion

R2-2103985	D1 in Immediate MDT	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

R2-2104007	Discussion on immediate MDT enhancements	Huawei, HiSilicon	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

R2-2104180	Discussion on immediate MDT	OPPO	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

#### 8.13.3.2	Logged MDT enhancements

Including outcome of email discussion [Post113-e][854][NR17 SON/MDT]  Logged MDT (CMCC)

R2-2103708	Report of [Post113-e][854][NR17 SON MDT]  Logged MDT	CMCC	report	Rel-17	NR_ENDC_SON_MDT_enh-Core

[AT113bis-e][804][NR/R17 SON/MDT]  ]  Logged MDT (CMCC)

-	Step 1: Collect the companies’ view on the proposals in R2-2103708. Deadline is 16:00 UTC 15/04/2021.

-	Step 2: Based on majority view provide the agreeable proposals

-	Step 3: Collect the companies’ view on the new proposals and goto Step 2 until 04:00 UTC Friday 16/04/2021.

-	Step 4: Providing final proposals for CB session. Note that in CB session there will not be technical discussion again. We try to directly agree the final proposals.

Intended outcome: Easily agreeable proposals (R2-2104434)

Deadline: Friday 16/04/2021

R2-2104434 Report of [AT113b-e][804][NR/R17 SON/MDT] Logged MDT (CMCC)	CMCC

Agreements:

1	UE reports the SIBs that UE actually intends to request.

2	Both Msg1-based and Msg3-based SI request related information are supported.

3	Option 3 (R2-2104434) is used for logged MDT in EN-DC, i.e., do not introduce SN configuration for logged MDT (neither for camping nor for non-camping/ EMR specific frequencies).

4	UE provides assistance by which network can avoid overwriting of an MDT configuration.

5	Introduce the logged MDT type (i.e. the management based MDT or the signalling based MDT) in the logged MDT configuration.

Proposal 4: It is FFS whether to extend current RA-report to include the on demand SI related information.

Proposal 6: It is FFS whether there is a need to avoid logged MDT configuration in the following cases from network perspective:

1)	Logged MDT is configured, but no results are available e.g. so far nothing stored, or all previously stored results retrieved

2)	Logged MDT configuration is released, but UE still has un-retrieved results that would be discarded upon accepting a new configuration

R2-2104431	Summary for 8.13.3.2 Logged MDT enhancements	CMCC

R2-2103063	Logged Measurement Enhancements	QUALCOMM Incorporated 	discussion	Rel-17

R2-2103097	MDT enhancements for On-demand SI	CATT	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

R2-2103143	Enhancements for logged MDT	OPPO	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

R2-2103501	Discussion on recording SI related information	OPPO	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

R2-2103554	Logged MDT in EN-DC and other enhancements	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

R2-2103697	MDT for slice unavailability	CMCC	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

R2-2103811	On logged MDT related enhancements	Ericsson	discussion

R2-2103930	R17 Logged MDT issues (overwrite, IRAT/ MR-DC, logging non camping freqs, IDC and OSI)	Samsung Telecommunications	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

R2-2104008	Discussion on logged MDT enhancements	Huawei, HiSilicon	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

R2-2104196	Remaining issues on logging of on-demand SI request information	ZTE Corporation, Sanechips	discussion	Rel-17

R2-2104197	On support MDT in NPN	ZTE Corporation, Sanechips	discussion	Rel-17

### 8.13.4	L2 Measurements

R2-2103156	Discussion on introduction of the UE DL PDCP packet average delay measurement	China Telecommunication	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

R2-2103824	On layer-2 measurements	Ericsson	discussion

R2-2104009	Discussion on L2M	Huawei, HiSilicon	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

******************************************************************************************

Summary of the report

Approved LS:

R2-2104625	Reply LS on RACH report for 2-step RACH CATT

=>	LS is approved

In principle agreed CRs:

R2-2104444	Merged Corrections to TS 37.320	CMCC, Nokia	CR	Rel-16	37.320	16.4.0	0107	-	C	NR_SON_MDT-Core

=>	CR is agreed in principle.

R2-2104599	SON-MDT Changes agreed in RAN2#113-bis meeting	Ericsson, Huawei	CR	Rel-16	38.331	16.4.1	2578	-	F	NR_SON_MDT-Core

=>	In principle agreed

R2-2104600	SON-MDT Changes agreed in RAN2#113-bis meeting	Ericsson, Huawei	CR	Rel-16	36.331	16.4.0	4650	-	F	NR_ENDC_SON_MDT_enh-Core

=>	In principle agreed