# 3GPP TSG-RAN WG2 Meeting #113bis-e	   R2-2204550

# Online, April 12th - April 20th, 2021

Title:	LS on the Intra-band and Inter-band (NG)EN-DC/NE-DC Capabilities

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	RAN2

To:	RAN4/RAN1

Cc:

Contact Person:

Name:	Wenting Li

E-mail Address:	li.wenting@zte.com.cn

# 1. Overall Description

RAN2 discussed the Intra-band and Inter-band (NG)EN-DC/NE-DC capabilities listed below:

| R1: 6-24 | Applying the same UL timing between NR and LTE | ul-TimingAlignmentEUTRA-NR Indicates whether to apply the same UL timing between NR and LTE for dynamic power sharing capable UE operating in a synchronous intra-band contiguous (NG)EN-DC. If this field is absent, UE shall be capable of handling a timing difference up to applicable MTTD requirements when operating in a synchronous intra-band contiguous (NG)EN-DC network, as specified in TS 38.133 [5]. If this capability is included in an inter-band (NG)EN-DC BC with an intra-band (NG)EN-DC BC part, this capability is used to indicate the restriction to the intra-band (NG)EN-DC BC part. |
| --- | --- | --- |
| R1: 6-23 | Incapability motivated by impacts of PA phase discontinuity with overlapping transmissions with non-aligned starting or ending times or hop boundaries across carriers for intra-band EN-DC, intra-band CA, and FDM based ULSUP | pa-PhaseDiscontinuityImpacts Indicates incapability motivated by impacts of PA phase discontinuity with overlapping transmissions with non-aligned starting or ending times or hop boundaries across carriers for intra-band (NG)EN-DC/NE-DC, intra-band CA and FDM based ULSUP. |
| R4: 2-16 | PA architectures for intra-band EN-DC | dualPA-Architecture For an intra-band band combination, this field indicates the support of dual PAs. If absent in an intra-band band combination, the UE supports single PA for all the ULs in the intra-band band combination. For other band combinations, this field is not applicable. |
| R4:2-4 | Simultaneous reception and transmission for inter-band EN-DC (TDD-TDD or TDD-FDD) | simultaneousRxTxInterBandENDC Indicates whether the UE supports simultaneous transmission and reception in TDD-TDD and TDD-FDD inter-band (NG)EN-DC/NE-DC. It is mandatory for certain TDD-FDD and TDD-TDD band combinations defined in TS 38.101-3 [4]. |
| R4:2-6 | Asynchronous FDD-FDD intra-band EN-DC DC | asyncIntraBandENDC Indicates whether the UE supports asynchronous FDD-FDD intra-band (NG)EN-DC with MRTD and MTTD as specified in clause 7.5 and 7.6 of TS 38.133 [5]. If asynchronous FDD-FDD intra-band (NG)EN-DC is not supported, the UE supports only synchronous FDD-FDD intra-band (NG)EN-DC. |

During RAN2 discussion, the below 5 (NG)EN-DC/NE-DC BC types were identified for the purpose of analyzing the applicability of the UE capability parameters (RAN2 may later decide to use another wording in TS38.306, once feedback is received from RAN1/RAN4) : (the bolder part denotes UL)

- Type 1: Intra-band (NG)EN-DC/NE-DC combination without additional inter-band NR and LTE CA component, e.g. DC 41A_n41A

- Type 2: Intra-band (NG)EN-DC/NE-DC combination supporting both UL and DL intra-band (NG)EN-DC/NE-DC parts with additional inter-band NR/LTE CA component, e.g. DC_25A_41A_n41A

- Type 3: Intra-band (NG)EN-DC/NE-DC combination without supporting UL in both the bands of the intra-band (NG)EN-DC/NE-DC UL part, e.g. DC_25A_41A_n41A

- Type 4: Inter-band (NG)EN-DC/NE-DC combination without Intra-band component, in short we call it as Inter-band (NG)EN-DC/NE-DC combination.

- Type 5: Inter-band (NG)EN-DC combination configurations where the frequency range of the E-UTRA band is a subset of the frequency range of the NR band, e.g., DC_B42_n77 and DC_B42_n78.

RAN2 sincerely requests RAN4/1 to provide answers of following questions for RAN2 future work:

Question 1: For which (NG)EN-DC/NE-DC BC types the above capabilities (ul-TimingAlignmentEUTRA-NR/ pa-PhaseDiscontinuityImpacts /dualPA-Architecture/ /simultaneousRxTxInterBandENDC /asyncIntraBandENDC ) are applicable?

Question 2: If the capability ul-TimingAlignmentEUTRA-NR/ pa-PhaseDiscontinuityImpacts /ul-dualPA-Architecture/ asyncIntraBandENDC are applicable to the (NG)EN-DC/NE-DC BC Type 1/2/3, whether they are used to indicate the restriction to the intra-band (NG)EN-DC/NE-DC BC part?

# 2. Actions:

ACTION: 	RAN2 sincerely requests RAN4/1 to provide answers of above questions for RAN2 future work.

3. Date of Next TSG-RAN2 Meetings:

3GPP  RAN2#114-e 	                19 May – 27 May 2021			Electronic Meeting

3GPP  RAN2#115-e 	                16 August – 27 August 2021		Electronic Meeting