3GPP TSG-RAN WG2 #113bis electronic	R2-210xxxx

e-Meeting, 12 April – 20 April 2021

Agenda Item:	5.4.3

Source:	ZTE, Sanechips

Title:	Email discussion summary of [AT113bis-e][011][NR15] UE caps III (ZTE)

Document for:	Discussion, Decision

1	Introduction

This contribution summarizes the following discussion:

[AT113bis-e][011][NR15] UE caps III (ZTE)

Scope: Treat R2-2104185, R2-2104186, R2-2104187, R2-2104188, R2-2102618, R2-2103025, R2-2103026, R2-2102610, R2-2103759, R2-2103760,

Phase 1, determine agreeable parts, Phase 2, for agreeable parts Work on CRs.

Intended outcome: Report and Agreed-in-principle CRs.

Deadline: Schedule A (Phase 1 deadline- Wednesday April 14 1000 UTC)

Contact from companies

| Company | Email |
| --- | --- |
| ZTE | li.wenting@zte.com.cn |
| Huawei, HiSilicon | kuangyiru@huawei.com |
| Intel | seau.s.lim@intel.com |
| MediaTek | Chun-fan.tsai@mediatek.com |
| Qualcomm Incorporated | mkitazoe@qti.qualcomm.com |
| OPPO | qianxi.lu@oppo.com |
| Samsung | seungri.jin@samsung.com |
| Apple | naveen.palle@apple.com |
| Lian | lian.araujo@ericsson.com |
| CATT | erlin.zeng@catt.cn |
| Nokia | amaanat.ali@nokia.com |
| vivo | Yangxiaodong5g@vivo.com |
| LG | Sunghoon.jung@lge.com |

- Discussion

2.1	Part 1: Intended to determine agreeable parts

Part 1 discussion is focusing on reaching conclusion whether the proposals/CRs can be agreed in principle, and Part 2 discussion would then focus on detailed changes for those agreeable contributions.

    - Intra-band and Inter-band EN-DC Capability

R2-2104185	Clarification on the Intra-band and Inter-band EN-DC Capabilities	ZTE Corporation, Sanechips	discussion	Rel-15	NG_RAN_PRN-Core	R2-2101562

R2-2104186	CR on the Intra-band and Inter-band EN-DC Capabilities-R15	ZTE Corporation, Sanechips	CR	Rel-15	38.306	15.13.0	0517	1	F	NR_newRAT-Core	R2-2101563

R2-2104187	CR on the Intra-band and Inter-band EN-DC Capabilities-R16	ZTE Corporation, Sanechips	CR	Rel-16	38.306	16.4.0	0518	1	A	NR_newRAT-Core	R2-2101564

R2-2104188	Draft LS on the Intra-band and Inter-band EN-DC Capabilities	ZTE Corporation, Sanechips	LS out	Rel-15	NR_newRAT-Core	R2-2101565	To:RAN4

These 4 papers are on the Intra-band and Inter-band EN-DC Capabilities. The intention was to clarify the related (NG)EN-DC/NE-DC BC types for these capabilities. Before going to the detail of these capabilities, the proponent hope to give a clear clarification as below on the intra-band (NG)EN-DC/NE-DC combination (with or without additional inter-band NR/LTE CA component) and inter-band (NG)EN-DC/NE-DC combination.

In the last meeting, companies have achieved some consensus on the terminologies for the intra/inter-band (NG)EN-DC/NE-DC combination types in [Post113-e][009][NR15] EN-DC BCS (Nokia) [1]. However, it was only reflected in the field description of the supportedBandwidthCombinationSetIntraENDC. E.g.

- Type 1: Intra-band (NG)EN-DC/NE-DC combination without additional inter-band NR and LTE CA component, e.g. DC 41A_n41A

- Type 2: Intra-band (NG)EN-DC/NE-DC combination supporting both UL and DL intra-band (NG)EN-DC/NE-DC parts with additional inter-band NR/LTE CA component, e.g. DC_25A_41A_n41A

- Type 3: Intra-band (NG)EN-DC/NE-DC combination without supporting UL in both the bands of the intra-band (NG)EN-DC/NE-DC UL part, e.g. DC_25A_41A_n41A

- Type 4: Inter-band (NG)EN-DC/NE-DC combination without Intra-band component, in short we call it as Inter-band (NG)EN-DC/NE-DC combination.

Obviously, these terminologies shall be used consistently among all of the related intra/inter-band (NG)EN-DC/NE-DC combination capabilities. Before extending these terminologies to the field description of other capabilities, it’s better to give a clear definition (as the proposal 1 below) on the intra-band (NG)EN-DC/NE-DC combination (with or without additional inter-band NR/LTE CA component) and inter-band (NG)EN-DC/NE-DC combination either in the chairman note or in the spec, which would be helpful for the readers who didn’t attend the post email discussion of [Post113-e][009][NR15] EN-DC BCS (Nokia).

Q1: Do companies generally agree with the proposal 1 in [2]?

Proposal 1: Ran2 confirm that the intra-band (NG)EN-DC/NE-DC combination (with or without additional inter-band NR/LTE CA component) in 38306 means the (NG)EN-DC/NE-DC band combinations that have the same band component at NR and Eutra side (irrespective of SPcell or Scell), for other cases, it would be defined as inter-band (NG)EN-DC/NE-DC combination.

| Company | Agree | Include in the spec or chairman note? | Comments |
| --- | --- | --- | --- |
| ZTE | Agree(proponent) | Spec or chairman note | Normally when we say intra-band EN-DC, the pcell and PScell were considered. However, according to the current spec, for the intra-band (NG)EN-DC/NE-DC band combinations with the inter-band component, it could be scell of the MCG and the PScell sharing the same band ( e.g. type 3 BC as above DC_25A_41A_n41A), it could also be the pcell and scell of the SCG sharing the same band . Thus we need this clarification for the intra-band (NG)EN-DC/NE-DC combination (with or without additional inter-band NR/LTE CA component), which would be helpful for the readers who didn’t attend the post email discussion of [Post113-e][009][NR15] EN-DC BCS (Nokia). |
| Huawei, HiSilicon | Agree | Chairman note |  |
| Intel | Agree | No | We are not sure of the ambiguity. |
| MediaTek | Agree | Chairman note if needed |  |
| Qualcomm Incorporated | Agree | Chairman notes |  |
| Nokia | Agree | What is broken? | Not sure we need anything to capture in specification nor in chair notes as this is basic understanding. |
| OPPO | Agree |  |  |
| Samsung | Not sure |  | We somewhat agree with the understanding from ZTE but it may require further checking whether this term is intentionally made. |
| Apple | Agree | Same view as Nokia, but we are ok to add chair note for further clarification. |  |
| Ericsson | Agree with the intention | ok | We agree with the intention but the formulation leaves the same ambiguity: Do we call it „intra-band“ if it contains LTE and NR carriers only in the same band or at least in the same band. And is DL sufficient or does that have to be also UL? Would the following be clearer? Intra-band (NG)EN-DC or NE-DC combination: A band combination supporting at least one EUTRA downlink serving cell and one NR downlink serving cell in the same band. Rapporteur: Thanks to the comments and agree with it, would be include it in the proposal. If this is considered correct and sufficient, we could just use the definition (right side of the colon) in 38.306 and avoid using the ambiguous term (left side of the colon). |
| CATT | Agree | Seems ok to clarify in Chair notes. |  |
| vivo | Agree | Same view as Nokia, but we are ok to add chair note for further clarification. |  |
| LG | Agree | Also fine with having it somewhere in spec. |  |

Summary:

- 13 companies provided inputs, 11 companies agree with the proposal, and 9 companies agree to include it in the chairman note (or spec). One company also point that DL sufficient. Based on these comment, rapporteur suggest to include the following clarification into the chairman note.

- Proposal 1: Ran2 confirm that the intra-band (NG)EN-DC/NE-DC combination in 38306 means the (NG)EN-DC/NE-DC band combinations that supporting at least one EUTRA downlink serving cell and one NR downlink serving cell in the same band (irrespective of SPcell or Scell). For other cases, it would be defined as inter-band (NG)EN-DC/NE-DC combination. (Include this clarification in the chairman note).

In the following questions, the related (NG)EN-DC/NE-DC BC types for the Intra-band and Inter-band EN-DC Capabilities would be discussed. For discussion convenience, the below 5 BC types were defined.

- Type 1: Intra-band (NG)EN-DC/NE-DC combination without additional inter-band NR and LTE CA component, e.g. DC 41A_n41A

- Type 2: Intra-band (NG)EN-DC/NE-DC combination supporting both UL and DL intra-band (NG)EN-DC/NE-DC parts with additional inter-band NR/LTE CA component, e.g. DC_25A_41A_n41A

- Type 3: Intra-band (NG)EN-DC/NE-DC combination without supporting UL in both the bands of the intra-band (NG)EN-DC/NE-DC UL part, e.g. DC_25A_41A_n41A

- Type 4: Inter-band (NG)EN-DC/NE-DC combination without Intra-band component, in short we call it as Inter-band (NG)EN-DC/NE-DC combination.

- Type 5: Inter-band (NG)EN-DC combination configurations where the frequency range of the E-UTRA band is a subset of the frequency range of the NR band, e.g., DC_B42_n77 and DC_B42_n78.

- ul-TimingAlignmentEUTRA-NR/dualPA-Architecture/pa-PhaseDiscontinuityImpact

| ul-TimingAlignmentEUTRA-NR Indicates whether to apply the same UL timing between NR and LTE for dynamic power sharing capable UE operating in a synchronous intra-band contiguous (NG)EN-DC. If this field is absent, UE shall be capable of handling a timing difference up to applicable MTTD requirements when operating in a synchronous intra-band contiguous (NG)EN-DC network, as specified in TS 38.133 [5]. If this capability is included in an inter-band (NG)EN-DC BC with an intra-band (NG)EN-DC BC part, this capability is used to indicate the restriction to the intra-band (NG)EN-DC BC part. | BC | No | N/A | N/A |
| --- | --- | --- | --- | --- |
| dualPA-Architecture For an intra-band band combination, this field indicates the support of dual PAs. If absent in an intra-band band combination, the UE supports single PA for all the ULs in the intra-band band combination. For other band combinations, this field is not applicable. | BC | No | N/A | N/A |
| pa-PhaseDiscontinuityImpacts Indicates incapability motivated by impacts of PA phase discontinuity with overlapping transmissions with non-aligned starting or ending times or hop boundaries across carriers for intra-band (NG)EN-DC/NE-DC, intra-band CA and FDM based ULSUP. | FS | No | N/A | N/A |

Q2: Do companies agree with the proposal 2 as below in [2]?

Proposal 2: The ul-TimingAlignmentEUTRA-NR/ ul-dualPA-Architecture/ pa-PhaseDiscontinuityImpacts is for the Type 1/2 BC, and not for the Type 3/4 BC.

| Company | Agree (Yes or No) | Comments |
| --- | --- | --- |
| ZTE | Agree(proponent) |  |
| Huawei, HiSilicon |  | Seems ok but it should be confirmed by RAN4 first. |
| Intel | Yes |  |
| MediaTek |  | This does not looks like a simple discussion. We would like request more time to check with a post meeting email discussion. The LS to RAN4 is of course needed and we could also discuss the LS content. |
| Qualcomm Incorporated | Yes |  |
| Nokia |  | Agree with MTK, first of all we think the notes proposed in the capabilities just even confuse more. Would it be better to define band combination types and put them in the annex and let the capability column refer to given BC type and relevance? Of course this also means LS to RAN4 is needed and we need some clear understanding of how RAN4 sees this. |
| OPPO | Yes with comment | there is anyway a left-issue on the definition of “contiguous”, so we need to R4 to solve that aspects at least. |
| Samsung | Yes |  |
| Apple |  | Similar view as Nokia/MediaTek. We need to get input from RAN4 and also RAN1 (as some of these are RAN1 capabilities). |
| Ericsson |  | Agree with Huawei and MediaTek. |
| CATT |  | Agree with Huawei and MediaTek. |
| vivo |  | Prefer to wait for RAN4 discussion |
| LG | Yes, but | This is not RAN2 issue alone, input from RAN4/1 is needed. |

Q2.1: Do companies generally agree with the related intention/modification on these 3 capabilities in the CRs [3][4]?

| Company | Agree Intention (Yes or No) | Agree Modifications (Yes or No) | Comments |
| --- | --- | --- | --- |
| ZTE | Agree(proponent) | Agree(proponent) |  |
| Huawei, HiSilicon |  |  | Prefer to first wait for RAN4 confirmation. |
| Intel | No | No | We do not see any ambiguity. |
| MediaTek |  |  | Prefer to wait RAN4 |
| Qualcomm Incorporated | Yes | No | We suggest we first check with RAN4 on the handling of type 5, and then see if any clarification is needed once RAN4 feedback is received. |
| Nokia |  |  | Agree with MTK |
| OPPO |  |  | we are fine with the proposal on type 1-4, and maybe further check on type-5 is needed anyway, as commented by QC. |
| Samsung |  |  | We are not sure how much needs to be clarified. Wait for RAN4 seems fine to use |
| Apple |  |  | Need RAN4/1 input. |
| Ericsson |  |  | Prefer to wait RAN4. |
| CATT |  |  | We should wait for R4 input. |
| vivo |  | Prefer to wait for RAN4 discussion | vivo |
| LG | Yes | No | Input from RAN4(/1) is needed to make changes in spec |

Q3: Do companies agree with the proposal 3 as below in [2]?

Proposal 3: Confirm with Ran 4 whether the ul-TimingAlignmentEUTRA-NR ul-dualPA-Architecture/ pa-PhaseDiscontinuityImpacts shall be adopted for the Type 5 BC.

| Company | Agree | Comments |
| --- | --- | --- |
| ZTE | Agree(proponent) |  |
| Huawei, HiSilicon |  | Seems ok but it should be confirmed by RAN4 first. |
| Intel |  | We think this needs to be checked with RAN4 first. |
| Qualcomm Incorporated | Yes | We suggest including the 5 “types“ in the LS to RAN4. |
| Nokia |  | Agree with HW and Intel |
| OPPO | Agree |  |
| Samsung |  | Agree with HW and Intel |
| Apple |  | Same view as HW/Intel. RAN1 should also be involved. |
| Ericsson |  | We should in general discuss the content of the LS, in any case those capabilities should of course be mentioned. |
| CATT |  | Agree with HW and Intel. |
| vivo |  | Prefer to wait for RAN4 discussion |
| LG | Agree |  |

- asyncIntraBandENDC

| asyncIntraBandENDC Indicates whether the UE supports asynchronous FDD-FDD intra-band (NG)EN-DC with MRTD and MTTD as specified in clause 7.5 and 7.6 of TS 38.133 [5]. If asynchronous FDD-FDD intra-band (NG)EN-DC is not supported, the UE supports only synchronous FDD-FDD intra-band (NG)EN-DC. | BC | No | FDD only | FR1 only |
| --- | --- | --- | --- | --- |

Q4: Do companies agree that the asyncIntraBandENDC is only for Type 1/2 BC?

| Company | Agree | Comments |
| --- | --- | --- |
| ZTE | Agree(proponent) |  |
| Huawei, HiSilicon |  | We understand Type 3 can also be applied for asyncIntraBandENDC, but it should be confirmed by RAN4 first. |
| Intel |  | We are wondering why it does not include Type 3 which is also for intra-Band ENDC |
| MediaTek |  | We would like request more time to check with a post meeting email discussion. |
| Qualcomm Incorporated |  | MRTD clearly is for DL, so type 3 is applicable. Can check with RAN4. |
| Nokia |  | Agree with MTK |
| OPPO |  | Can check with RAN4. |
| Samsung |  | Can check with RAN4 |
| Apple |  | Can check with RAN4 |
| Ericsson |  | We also prefer to confirm with RAN4. |
| CATT |  | Can check with RAN4 |
| vivo |  | Prefer to wait for RAN4 discussion |
| LG |  | We think type3 is applicable. Check with RAN4 is needed. |

Q4.1: Do companies generally agree with the related modification on this capability in the CRs [3][4]?

| Company | Agree (Yes or No) | Comments |
| --- | --- | --- |
| ZTE | Agree(proponent) |  |
| Huawei, HiSilicon |  | Prefer to first wait for RAN4 confirmation. |
| Intel | No | We do not see any ambiguity. |
| MediaTek |  | Prefer to wait |
| Qualcomm Incorporated | No | MRTD clearly is for DL, so type 3 is applicable. Can check with RAN4. |
| Nokia |  | Agree with MTK |
| OPPO | See comment | We assume the NOTE2 which is used for the other IEs is also applicable in this change. |
| Samsung |  | Can check with RAN4 |
| Apple | No | We need RAN4/1 input before making any changes. |
| Ericsson |  | Also prefer to wait. |
| CATT |  | Also prefer to wait. |
| vivo |  | Prefer to wait for RAN4 discussion |
| LG | No | We think type3 is applicable. No change until we check with RAN4 seems better. |

- simultaneousRxTxInterBandENDC

| simultaneousRxTxInterBandENDC Indicates whether the UE supports simultaneous transmission and reception in TDD-TDD and TDD-FDD inter-band (NG)EN-DC/NE-DC. It is mandatory for certain TDD-FDD and TDD-TDD band combinations defined in TS 38.101-3 [4]. | BC | CY | N/A | N/A |
| --- | --- | --- | --- | --- |

Q5: Do companies agree that the simultaneousRxTxInterBandENDC is for Type 2/3/4 (not for type 1)?

| Company | Agree | Comments |
| --- | --- | --- |
| ZTE | Agree(proponent) |  |
| Huawei, HiSilicon |  | Seems ok but it should be confirmed by RAN4 first. |
| Intel | Agree |  |
| Qualcomm Incorporated | Yes |  |
| Nokia |  | Agree with HW |
| OPPO | Agree |  |
| Samsung |  | Seems ok but it should be confirmed by RAN4 first. |
| Apple | We tend to agree but | Better to get confirmation from RAN4 before making changes. |
| Ericsson |  | We think it should be confirmed with RAN4. |
| CATT |  | agree with comments above. |
| vivo |  | Agree with Huawei |
| LG |  | Same view as HW |

Q5.1: Do companies generally agree with the related intention/ modification on this capability in the CRs [3][4]?

| Company | Agree Intention (Yes or No) | Agree Modifications (Yes or No) | Comments |
| --- | --- | --- | --- |
| ZTE | Agree(proponent) | Agree(proponent) |  |
| Huawei, HiSilicon |  |  | Prefer to first wait for RAN4 confirmation. |
| Intel | No | No | We do not see any ambiguity. |
| Qualcomm Incorporated | Yes | No | We suggest we first check with RAN4 on the handling of type 5, and then see if any clarification is needed once RAN4 feedback is received. |
| Nokia |  |  | Check with RAN4 |
| OPPO |  |  | Agree to check with R4 |
| Samsung |  |  | Check with RAN4 |
| Apple |  |  | Check with RAN4/1 |
| Ericsson |  |  | We prefer to wait for further RAN4 input. |
| CATT |  |  | agree with comments above. |
| vivo |  |  | Agree with Huawei |
| LG | Yes | No | Prefer no change until we check with RAN4 |

Q6: Do companies agree to send a LS to RAN4 to confirm whether the simultaneousRxTxInterBandENDC is needed also for the type 5 BC?

| Company | Agree | Comments |
| --- | --- | --- |
| ZTE | Agree(proponent) |  |
| Huawei, HiSilicon | Agree | We prefer to send LS to RAN4 to let them confirm the correct BC type(s) for all the capability parameters listed, then RAN2 to decide what clarification should be captured. |
| Intel | Agree to check with RAN4 | We think this needs to be checked with RAN4 first as like for Q3 |
| Qualcomm Incorporated | Yes | We suggest including the 5 “types“ in the LS to RAN4. |
| Nokia | Agree | We need to check with RAN4 all these capabilities and applicability to given BC type. |
| OPPO | Yes |  |
| Samsung | Yes |  |
| Apple | Yes |  |
| Ericsson |  | Agree with Huawei. |
| CATT |  | Agree with Huawei. |
| vivo |  | Agree with Huawei |
| LG | Yes | Same view as HW |

Summary:

- 13 companies provided inputs, almost all of the companies prefer to send an LS to RAN4 confirm these capabilities for all of the BC types, two companies point out that RAN1 shall also be involved, one company point out that in the LS the 5 BC types shall also be included.

Proposal 2: Send an LS to Ran 4 to confirm for which BC types the ul-TimingAlignmentEUTRA-NR ul-dualPA-Architecture/ pa-PhaseDiscontinuityImpacts/asyncIntraBandENDC/ simultaneousRxTxInterBandENDC shall be adopted respectively. Ran1 can also be involved for part of these capabilities. In the LS, also include the below 5 BC types:

- Type 1: Intra-band (NG)EN-DC/NE-DC combination without additional inter-band NR and LTE CA component, e.g. DC 41A_n41A

- Type 2: Intra-band (NG)EN-DC/NE-DC combination supporting both UL and DL intra-band (NG)EN-DC/NE-DC parts with additional inter-band NR/LTE CA component, e.g. DC_25A_41A_n41A

- Type 3: Intra-band (NG)EN-DC/NE-DC combination without supporting UL in both the bands of the intra-band (NG)EN-DC/NE-DC UL part, e.g. DC_25A_41A_n41A

- Type 4: Inter-band (NG)EN-DC/NE-DC combination without Intra-band component, in short we call it as Inter-band (NG)EN-DC/NE-DC combination.

- Type 5: Inter-band (NG)EN-DC combination configurations where the frequency range of the E-UTRA band is a subset of the frequency range of the NR band, e.g., DC_B42_n77 and DC_B42_n78.

    - Cross-Carrier Operation

R2-2102618	LS on Interpretation of UE Features in Case of Cross-Carrier Operation (R1-2102085; contact: ZTE)	RAN1	LS in	Rel-15	NR_newRAT-Core	To:RAN2

Moved from 5.1

R2-2103025	CR on UE capability in case of Cross-Carrier operation	ZTE Corporation, Sanechips, Ericsson	CR	Rel-15	38.306	15.13.0	0544	-	F	NR_newRAT-Core

R2-2103026	CR on UE capability in case of Cross-Carrier operation	ZTE Corporation, Sanechips, Ericsson	CR	Rel-16	38.306	16.4.0	0545	-	A	NR_newRAT-Core

Q7: Do companies generally agree with these 2 CRs?

| Company | Agree (Yes or No) | Comments |
| --- | --- | --- |
| ZTE | Agree(proponent) |  |
| Huawei, HiSilicon | Agree but | For NOTE 2, we are not sure why it states “Only applicable for cross carrier scheduling with the same SCS…” RAN1 does not mention that it is only applicable for such case, and RAN1 is discussing how to understand pdcch-MonitoringAnyOccasionsWithSpanGap in case of cross carrier scheduling with the different SCS. |
| Intel | Yes, but | On the note: If the reported value is different between the band of the scheduled/triggered/indicated cell and the band of the scheduling/triggering/indicating cell, the value reported for the scheduling/triggering/indicating cell is applied. We think this can be simplified to just ‘the value reported for the scheduling/triggering/indicating cell is applied’ since this is always the case regardless of if the reported value is the same or different according to the LS. |
| MediaTek | Agree |  |
| Qualcomm Incorporated | Agree |  |
| Nokia | Agree | Take the comments of other companies into account |
| Samsung | Agree |  |
| Apple | Agree | Same view as Nokia |
| Ericsson | Agree | To HW’s comments, we think “only applicable” avoids confusion since there is no conclusion for different SCS as said. But if companies prefer we would be fine from our side to remove the “only”. To Intel’s comments, despite more long wording we would prefer to keep the sentence aligned with the already existing ones in such section, hence it would be preferable to keep it as it is. |
| CATT | Agree |  |
| vivo | agree |  |
| LG | Agree |  |

Summary:

- 12 companies provided inputs, all companies agree with the CR, and 3 companies give more comments. It tends to pursue these 2 CRs, and update the wording (if needed) based on the comments

Proposal 3: To agree CR R2-2103025/R2-2103026 with potential updates in Phase 2 based on comments.

2.1.3 Simultaneous CSI-RS resources

R2-2102610	Reply LS on the use of simultaneous CSI-RS resources and ports (R1-2101962; contact: Ericsson)	RAN1	LS in	Rel-15	NR_newRAT-Core	To:RAN2

Moved from 5.1

R2-2103759	Correction to the use of simultaneous CSI-RS resources	Ericsson, Nokia	CR	Rel-15	38.306	15.13.0	0552	-	F	NR_newRAT-Core

R2-2103760	Correction to the use of simultaneous CSI-RS resources	Ericsson, Nokia	CR	Rel-16	38.306	16.4.0	0553	-	A	NR_newRAT-Core

Q8: Do companies generally agree with these 2 CRs?

| Company | Agree (Yes or No) | Comments |
| --- | --- | --- |
| ZTE | Agree | We agree with these 2 CRs, which align with RAN1’s LS |
| Huawei, HiSilicon | Agree |  |
| Intel | Yes |  |
| MediaTek | Agree |  |
| Qualcomm Incorporated | Agree |  |
| Nokia | Agree | Proponent |
| OPPO | Yes |  |
| Samsung | Agree | Should be clarified based on RAN1 LS. |
| Apple | Ok. |  |
| Ericsson | Agree (proponent) |  |
| CATT | Agree |  |
| vivo | agree |  |
| LG | Yes | In line with RAN1 reply LS. |

Summary:

- 13 companies provided inputs, all companies agree with the CR.

Proposal 4: To agree CR R2-2103759/R2-2103760.

2.2	Part 2: Intended to progress discussion on agreeable parts

- To be updated after discussion on part 1 -

3	Conclusion

- Phase 1

- Proposal 1: Ran2 confirm that the intra-band (NG)EN-DC/NE-DC combination in 38306 means the (NG)EN-DC/NE-DC band combinations that supporting at least one EUTRA downlink serving cell and one NR downlink serving cell in the same band (irrespective of SPcell or Scell). For other cases, it would be defined as inter-band (NG)EN-DC/NE-DC combination. (Include this clarification in the chairman note).

- Proposal 2: Send an LS to Ran 4/1 to confirm for which BC types the ul-TimingAlignmentEUTRA-NR/ ul-dualPA-Architecture/ pa-PhaseDiscontinuityImpacts/asyncIntraBandENDC/ simultaneousRxTxInterBandENDC shall be adopted respectively. In the LS, also include the below 5 BC types:

- Type 1: Intra-band (NG)EN-DC/NE-DC combination without additional inter-band NR and LTE CA component, e.g. DC 41A_n41A

- Type 2: Intra-band (NG)EN-DC/NE-DC combination supporting both UL and DL intra-band (NG)EN-DC/NE-DC parts with additional inter-band NR/LTE CA component, e.g. DC_25A_41A_n41A

- Type 3: Intra-band (NG)EN-DC/NE-DC combination without supporting UL in both the bands of the intra-band (NG)EN-DC/NE-DC UL part, e.g. DC_25A_41A_n41A

- Type 4: Inter-band (NG)EN-DC/NE-DC combination without Intra-band component, in short we call it as Inter-band (NG)EN-DC/NE-DC combination.

- Type 5: Inter-band (NG)EN-DC combination configurations where the frequency range of the E-UTRA band is a subset of the frequency range of the NR band, e.g., DC_B42_n77 and DC_B42_n78.

Proposal 3: To agree CR R2-2103025/R2-2103026 with potential updates in Phase 2 based on comments.

Proposal 4: To agree CR R2-2103759/R2-2103760.

4	References

- R2-2102215	Summary of [Post113-e][009][NR15] EN-DC BCS (Nokia)	Nokia, Nokia Shanghai Bell

- R2-2104185	Clarification on the Intra-band and Inter-band EN-DC Capabilities	ZTE Corporation, Sanechips	discussion	Rel-15	NG_RAN_PRN-Core	R2-2101562

- R2-2104186	CR on the Intra-band and Inter-band EN-DC Capabilities-R15	ZTE Corporation, Sanechips	CR	Rel-15	38.306	15.13.0	0517	1	F	NR_newRAT-Core	R2-2101563

- R2-2104187	CR on the Intra-band and Inter-band EN-DC Capabilities-R16	ZTE Corporation, Sanechips	CR	Rel-16	38.306	16.4.0	0518	1	A	NR_newRAT-Core	R2-2101564

- R2-2104188	Draft LS on the Intra-band and Inter-band EN-DC Capabilities	ZTE Corporation, Sanechips	LS out	Rel-15	NR_newRAT-Core	R2-2101565	To:RAN4

- R2-2102618	LS on Interpretation of UE Features in Case of Cross-Carrier Operation (R1-2102085; contact: ZTE)	RAN1	LS in	Rel-15	NR_newRAT-Core	To:RAN2 Moved from 5.1

- R2-2103025	CR on UE capability in case of Cross-Carrier operation	ZTE Corporation, Sanechips, Ericsson	CR	Rel-15	38.306	15.13.0	0544	-	F	NR_newRAT-Core

- R2-2103026	CR on UE capability in case of Cross-Carrier operation	ZTE Corporation, Sanechips, Ericsson	CR	Rel-16	38.306	16.4.0	0545	-	A	NR_newRAT-Core

- R2-2102610	Reply LS on the use of simultaneous CSI-RS resources and ports (R1-2101962; contact: Ericsson)	RAN1	LS in	Rel-15	NR_newRAT-Core	To:RAN2 Moved from 5.1

- R2-2103759	Correction to the use of simultaneous CSI-RS resources	Ericsson, Nokia	CR	Rel-15	38.306	15.13.0	0552	-	F	NR_newRAT-Core

- R2-2103760	Correction to the use of simultaneous CSI-RS resources	Ericsson, Nokia	CR	Rel-16	38.306	16.4.0	0553	-	A	NR_newRAT-Core