3GPP TSG-RAN WG2 #113bis-e	R2-210xxxx

eMeeting, 12th – 20th April, 2021

Title:	[Draft] Reply LS on single-uplink operation in more than one band pair of a band combination

Response to:	R4-2103144 / R2-2102644

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	MediaTek [to be RAN2]

To:	RAN4

Cc:

Contact Person:

Name:	Chun-Fan (Felix), Tsai

E-mail Address:	Chun-Fan.Tsai (at) mediaTek.com

1. Overall Description

RAN2 would like to thank RAN4 for the LS R4-2103144 on single-uplink operation in more than one band pair of a band combination. RAN2 has discussed the question raised by RAN4 and concluded as following.

RAN2 confirms that singleUL-Transmission can not indicate dual UL in one UL band pair and single UL in another UL band pair in one band combination. However, with the ASN.1 signalling from Rel-15, UE is able to indicate dual UL transmission capability in one UL band pair and single UL transmission capability in another UL band pair in different band combination entries.

- For example, suppose a UE supports (hypothetical) band combination DC_1A_2A_n66A_n78A where single UL is allowed for DC_2A_n66A and DC_2A_n78A. If UE indicates singleUL-Transmission for DC_1A_2A_n66A_n78A but only need it with DC_2A_n66A (i.e. single UL is not needed for DC_2A_n78A), UE can indicate separate band combination DC_2A_n78A where singleUL-Transmission is not included to indicate that single UL is not required for DC_2A_n78A. That is:

  - BC entry 1 - DC_1A_2A_n66A_n78A (with UL DC_2A_n66A) – Indicates SUO is required

  - BC entry 2 - DC_1A_2A_n66A_n78A (with UL DC_2A_n78A) – Not include SUO

Therefore, RAN2 can solve this issue from Rel-15 for two UL CC case and there is still limitation on current signaling if the UE supports more than 2 UL bands simultaneously with some UL band pair requests SUO. However, RAN2 has no plan to implement additional solutions.

In addition, RAN2 would like to point out that UE capability signalling is considered per BC when deciding RRC configuration. Network is not required to derive UE configuration for a BC based on multiple band combination capabilities.

2. Actions:

ACTION: 	RAN2 respectfully asks RAN4 to take above RAN2 conclusions into consideration.

3. Date of Next RAN2 Meetings:

RAN2#114e, eMeeting, 19-27 May. 2021

RAN2#115e, eMeeting, 16-27 Aug. 2021