3GPP TSG-RAN WG2 Meeting #113bis electronic	R2-210xxxx

Online, April 12th –20th, 2021

Agenda Item:	6.1.4.1.1

Source:	Fujitsu

Title:	Report of [Offline-019][NR16] Connection Control

Document for:	Discussion, Agreement

1	Introduction

This contribution is related to the following email discussion.

[AT113bis-e][019][NR16] Connection Control (Fujitsu)

Scope: Treat R2-2103209, R2-2103210, R2-2104247, R2-2104240, R2-2103280, R2-2103449, R2-2102854, R2-2104167, R2-2103937

Phase 1, determine agreeable parts, Phase 2, for agreeable parts Work on CRs.

Intended outcome: Report and Agreed-in-principle CRs

Deadline: Schedule A

2	Contact Information

| Company | Contact: Name (E-mail) |
| --- | --- |

3	Discussion

3.1	Restriction on DCI format 0_2/1_2 for unlicensed band

R2-2103209	CR on the configuration restriction on DCI format 0_2/1_2 for unlicensed band (Option 1)	OPPO, Samsung, Xiaomi, ZTE, Apple, Intel	CR	Rel-16	38.331	16.4.1	2502	-	F	NR_IIOT-Core, NR_unlic-Core

R2-2103210	CR on the UE capability restriction on DCI format 0_2/1_2 for unlicensed band (Option 2)	OPPO, Samsung, Xiaomi	CR	Rel-16	38.306	16.4.0	0548	-	F	NR_IIOT-Core, NR_unlic-Core

Summary of Changes from the CRs:

- Option 1: In Section 6.3.2 of TS38.331, clarify DCI format 1_2/0_2 is not allowed for unlicensed band.

- Option 2: In Section 4.2.7.10 of TS38.306, clarify dci-Format1-2And0-2-r16 is not applied for unlicensed band.

Rapporteur opinion: The change is needed. Option 1 is slightly preferred.

Do you agree with the intention of these CRs? If yes, which option do you prefer?

| Company | Response | Comments |
| --- | --- | --- |

Summary: To be added later

3.2	Correction on releasing referenceTimePreferenceReporting and sl-AssistanceConfigNR

R2-2104247 Correction on releasing referenceTimePreferenceReporting and sl-AssistanceConfigNR     Google Inc.  CR  Rel-16 38.331   16.4.1    2562      -      F     5G_V2X_NRSL-Core, NR_IIOT-Core

Summary of Changes from the CR:

If the selected cell is not a CHO candicated cell, the UE releases referenceTimePreferenceReporting and sl-AssistanceConfigNR.

Rapporteur opinion: The change seems acceptable.

Do you agree with the changes proposed in this CR?

| Company | Response | Comments |
| --- | --- | --- |

Summary: To be added later

3.3	Correction on description of subCarrierSpacing in BWP

R2-2104240	Correction on description of subCarrierSpacing in BWP	Fujitsu, Samsung	CR	Rel-16	38.331	16.4.1	2561	-	F	NR_unlic-Core

Summary of Changes from the CR:

The description of subCarrierSpacing in BWP is changed into: Subcarrier spacing to be used in this BWP for all channels and reference signals unless explicitly configured elsewhere. Corresponds to subcarrier spacing according to TS 38.211 [16], table 4.2-1. The value kHz15 corresponds to µ=0, value kHz30 corresponds to µ=1, and so on. Only the values 15 kHz, 30 kHz, or 60 kHz (FR1), and 60 kHz or 120 kHz (FR2) are applicable. For the initial DL BWP this field has the same value as the field subCarrierSpacingCommon in MIB of the same serving cell for operation in licensed spectrum, and has the value corresponding to the subcarrier spacing of the SSB associated to the initial DL BWP for operation with shared spectrum channel access.

Rapporteur opinion: The change is essential and needed.

Do you agree with the changes proposed in this CR?

| Company | Response | Comments |
| --- | --- | --- |

Summary: To be added later

3.4	Correction on description of ssb-PositionsInBurst in ServingCellConfigCommonSIB

R2-2103280	Correction on description of ssb-PositionsInBurst in ServingCellConfigCommonSIB	Fujitsu	CR	Rel-16	38.331	16.4.1	2505	- F	NR_unlic-Core

Summary of Changes from the CR:

Remove ‘only mediumBitmap is used’ in description of ssb-PositionsInBurst in ServingCellConfigCommonSIB

Rapporteur opinion: The change is editorial and acceptable. Additionally, for easy reading, maybe the description could be re-constructed as follows:

ssb-PositionsInBurst Time domain positions of the transmitted SS-blocks in an SS-burst as defined in TS 38.213 [13], clause 4.1. For operation with shared spectrum channel access, only inOneGroup is used and the UE interprets this field same as mediumBitmap in ServingCellConfigCommon. The UE assumes that a bit at position k > NSSBQCL is 0, where NSSBQCL is obtained from MIB as specified in TS 38.213 [13], clause 4.1.

Do you agree with the intention of this CR? If yes, do you agree with the above changes suggested by the Rapporteur?

| Company | Response | Comments |
| --- | --- | --- |

Summary: To be added later

3.5	Correction on freqMonitorLocations

R2-2103449	Correction on freqMonitorLocations	ASUSTeK	CR	Rel-16	38.331	16.4.1	2508	-	F	NR_unlic-Core

Summary of Changes from the CR:

Change least significant bit in field description of freqMonitorLocations to most significant bit.

Rapporteur opinion: The change is for alignment with TS 38.213. It is acceptable.

Do you agree with the changes proposed in this CR?

| Company | Response | Comments |
| --- | --- | --- |

Summary: To be added later

3.6	Correction on repetition for L1-SINR

R2-2102854	Correction on repetition for L1-SINR	vivo	draftCR	Rel-16	38.331	16.4.1	F	NR_eMIMO-Core

Summary of Changes from the CR:

In the field description of repetition in NZP-CSI-RS-ResourceSet, update the field description that the repetition could be also configured for CSI-RS resource sets with report of L1 SINR.

Rapporteur opinion: The change seems acceptable.

Do you agree with the changes proposed in this CR?

| Company | Response | Comments |
| --- | --- | --- |

Summary: To be added later

3.7	Miscellaenous corrections on BH RLC channel management for IAB-MT

R2-2104167	Miscellaenous corrections on BH RLC channel management for IAB-MT	Huawei, HiSilicon	CR	Rel-16	38.331	16.4.1	2557	-	F	NR_IAB-Core

Summary of Changes from the CR:

- Based on the received CellGroupConfig IE which contains the spCellConfig with reconfigurationWithSync, IAB-MT will resume BH RLC channels, if suspended.

- Upon the initiation of the RRC re-establishment, suspend BH RLC channels and BAP entity at IAB-MT.

- Editorial changes for IABOtherInformation-IEs field descriptions. For iab-IPv4-AddressReport: This field is used to report the IPv4 address per specific usage assigned by OAM for IAB-DU; for iab-IPv6-AddressReport: This field is used to report the IPv6 address per specific usage assigned by OAM for IAB-DU.

- Editorial change for all-Traffic-IAB-IP-Address in IAB-IP-AddressAndTraffic-IEs field descriptions: This field is used to report to IAB-donor-CU the IP address(es) or IPv6 address prefix for all traffic.

- Extend the LogicalChaneelIdentity IE description for BH RLC channel: The IE LogicalChannelIdentity is used to identify one logical channel (LogicalChannelConfig) and the corresponding RLC bearer (RLC-BearerConfig) or BH RLC channel (BH-RLC-ChannelConfig).

Rapporteur opinion: The changes all look acceptable.

Do you agree with the changes proposed in this CR?

| Company | Response | Comments |
| --- | --- | --- |

Summary: To be added later

3.8	Clarification to BAP address field description in the BAP-RoutingID IE

R2-2103937	Clarification to BAP address field description in the BAP-RoutingID IE	Ericsson	CR	Rel-16	38.331	16.4.1	2542	-	F	NR_IAB-Core

Summary of Changes from the CR:

- In the field description of the BAP-Address IE included in the UL BAP-RoutingID IE, remove “destination IAB-node” from “The ID of a destination IAB-node or IAB-donor-DU used in the BAP header”.

Rapporteur opinion: The change is unnecessary. Current text is a generic description of how BAP routing ID is defined.

Do you agree with the changes proposed in this CR?

| Company | Response | Comments |
| --- | --- | --- |

Summary: To be added later

4	Conclusion

To be added later