3GPP TSG-RAN WG2 Meeting #113bis-e		R2-21xxxxx

Electronic, April 12 – April 20 2021

Agenda item:	6.1.4.1.2

Source:	Apple

Title:	Summary of [AT113bis-e][020][NR16] RRM and Measurements (Apple)

Document for:	Discussion

# 1	Introduction

This is the summary of the following email offline discussion.

[AT113bis-e][020][NR16] RRM and Measurements (Apple)

Scope: Treat R2-2102650, R2-2103030, R2-2103169, R2-2103879, R2-2103281, R2-2104173,

Phase 1, determine agreeable parts, Phase 2, for agreeable parts Work on CRs.

Intended outcome: Report and Agreed-in-principle CRs

Deadline: Schedule A

Schedule A from chairman is also copied here for reference:

Schedule A (a schedule for main session for many offline dicussion):

A first round with Deadline for comments Wednesday April 14 1000 UTC to settle scope what is agreeable etc (phase 1)

A pre-final round with Deadline for any functional and/or scope comments Monday April 19 1800 UTC. At this point all non-agreeable parts shall be removed/excluded. (phase 2)

A final round (last 24h) for checking and smaller simplification / removal comments only including agreeable parts, with Deadline EOM (at this point all outcome documents need to be available in inbox with tdoc numbers).

Additional check-points etc if needed are defined by the Rapporteur. Offline discussion rapporteur must notify chairman / session chair if on-line comeback discussion is needed, if discussion doesn’t converge etc.

# 2   Contact info

| Company Name | Contact Person | Email Address |
| --- | --- | --- |
| Apple | Yuqin Chen | yuqin_chen@apple.com |
| Huawei, HiSilicon | Xiao XIAO | xiaoxiao6@huawei.com |
| Lenovo | Hyung-Nam Choi | hchoi5@lenovo.com |
| MediaTek | Felix Tsai | Chun-fan.tsai@mediatek.com |
| Nokia | Jarkko Koskela | Jarkko.t.koskela@nokia.com |
| Qualcomm | Mouaffac | mambriss@qti.qualcomm.com |
| Intel | Sudeep Palat | Sudeep.k.palat@intel.com |
| vivo | Boubacar | kimba@vivo.com |

# 3   Topic 1: E-UTRA CGI reporting with autonomous gap

R2-2102650	LS on CGI reading with autonomous gaps (R4-2103610; contact: ZTE)	RAN4	LS in	Rel-16	NR_RRM_enh-Core	To:RAN2	Cc:-

R2-2103030	Correction on T321 for autonomous gap based E-UTRAN CGI reporting	ZTE Corporation, Sanechips	CR	Rel-16	38.331	16.4.1	2494	-	F	NR_RRM_enh-Core

RAN4 sent the following agreement to RAN2 in LSin R2-2102650 [1] on E-UTRA CGI reporting with autonomous gap.

Copied from RAN4 LSin R2-2102650[1]: RAN4 had agreed the value of T321 timer for autonomous gap based CGI reading in FR1 and FR2, and an LS [2] was sent to RAN2 in RAN4#96e meeting. RAN4 has further discussed the T321 timer value for CGI reading in LTE and conclusion has been made. The value of T321 timer for autonomous gap based CGI reading in LTE is 0.2 seconds (200 ms).

Based on the above info, R2-2103030 [2] proposes the following change to TS38.331:

2> if the reportType is set to reportCGI in the reportConfig associated with this measId: 3> if the measObject associated with this measId concerns E-UTRA: 4> if the useAutonomousGaps is included in the reportConfig associated with this measId: 5> start timer T321 with the timer value set to 200 ms for this measId;

Question 1: Do companies agree with the change proposed in R2-2103030 [2]?

| Company Name | Yes/No? | Comments |
| --- | --- | --- |
| Apple | Yes |  |
| Lenovo | Yes |  |
| MediaTek | Yes |  |
| Nokia | Yes |  |
| Ericsson | Yes |  |
| Qcom | Yes |  |
| Intel | Yes |  |
| vivo | Yes |  |

# 4	  Topic 2: CGI reporting in NPN

R2-2103169	Clarification on NPN related CGI report	Huawei, CMCC, China Telecom, China Unicom, HiSilicon	CR	Rel-16	38.331	16.4.1	2501	-	F	NG_RAN_PRN-Core

Summary of change: In 5.5.5.1, refine the descriptions of the measurement reporting procedure to clarify that the UE reports the plmn-IdentityInfoList only when cellReservedForOtherUse is not set to “true” for CGI reporting.

2> else: 3> if the cell indicated by cellForWhichToReportCGI is an NR cell: 4> if cellReservedForOtherUse is not set to true for the concerned cell and plmn-IdentityInfoList of the cgi-Info for the concerned cell has been obtained: 5> include the plmn-IdentityInfoList including plmn-IdentityList, trackingAreaCode (if available), ranac (if available), cellIdentity and cellReservedForOperatorUse for each entry of the plmn-IdentityInfoList; 5> include frequencyBandList if available;

Question 2: Do companies agree with the change in R2-2103169?

| Company Name | Yes/No? | Comments |
| --- | --- | --- |
| Apple | Yes |  |
| Huawei, HiSilicon | Yes | Proponent. Also, the need of the change has been confirmed by the co-sourcing operators, in order to avoid potentially ambiguous information reported to the network by the UE. |
| Lenovo | No | Normally, the UE does not need to take care of the cellReservedForOtherUse field for cgi reporting. But the proposed change would then affect R16 non-NPN-capable UEs to consider this flag for cgi-reporting. |
| MediaTek | No | The proposed change is NBC to R15 UE, which is not acceptable to us. We also don’t understand the reason for change. Whether a cell is a NPN depends not only depends on the cellReservedForOtherUse but also the npn-IdentityInfoList IE. |
| Nokia | No | This is not backward compatible - in NPN-only cells the operator should configure PLMN ID that does not cause problems. The proposed solution is NBC: a Rel-15 UE will report in a different way as a Rel-16 UE. And also there can be cells that are reserved due to other reasons with this flag. |
| Ericsson | Yes |  |
| Qcom | Yes |  |
| Intel | Yes | Another option to solve this issue would have been better to report this field as in the CGI report but it seems too late to make that sort of change. |
| vivo | No | This NBC proposal, we prefer not agree with such change. |

# 5	  Topic 3: NR-U RRM measurement

R2-2103879	Discussion on NR-U RRM measurement	Apple, xiaomi, LG Electronics	discussion	Rel-16	NR_unlic-Core

R2-2103281	Discussion on configuration of SSBs to be measured for NR-U	Fujitsu	discussion	Rel-16	NR_unlic-Core

## 5.1    Sub-topic 1: Change to SSB-ToMeasure field description

R2-2103879 [4] mentions the following issue and has proposal 1.

Issue 1: Lack restriction in SSB-ToMeasure However, we observed that for SSB-ToMeasure in MeasObjectNR, the spec does not have such restriction that only mediumBitmap is used for operation with shared spectrum. This may lead to unnecessary complexity in spec and also UE implementation. For example, if NW configures SSB-ToMeasure with longBitmap, we would need a special handling, e.g., truncation. In order to simplify the spec, we propose to introduce the same restriction to SSB-ToMeasure that only mediumBitmap is used for operation with shared spectrum.

In R2-2103879 [4]:

Proposal 1: Suggest to have the same restriction to SSB-ToMeasure that only mediumBitmap is used for operation with shared spectrum.

Text proposal for SSB-ToMeasure: mediumBitmap Bitmap when maximum number of SS/PBCH blocks per half frame equals to 8 as defined in TS 38.213 [13], clause 4.1. For operation with shared spectrum channel access, only mediumBitmap is used. iIf the k-th bit is set to 1, the UE assumes that one or more SS/PBCH blocks within the discovery burst transmission window with candidate SS/PBCH block indexes corresponding to SS/PBCH block index equal to k – 1 may be transmitted; if the kt-th bit is set to 0, the UE assumes that the corresponding SS/PBCH block(s) are not transmitted. If ssb-PositionQCL is configured, the k-th bit is set to 0, where k > ssb-PositionQCL and the number of actually transmitted SS/PBCH blocks is not larger than the number of 1's in the bitmap.

Question 3: Do companies agree with the proposal 1 in R2-2103879 shown above?

| Company Name | Yes/No? | Comments |
| --- | --- | --- |
| Apple (proponent) | Yes |  |
| MediaTek | Yes |  |
| Nokia | No | Not needed. We have already limitation in SIB1 and network will not configure conflicting configuration in measObject. |
| Qcom | Yes | This should be already the case, given NRU is only defined in FR1 TDD bands in R16, but it does not hurt to make it more clear |
| Intel | Yes |  |
| vivo | Yes |  |

R2-2103281 [5] mentions the following issue and correspondingly has Proposal 1.

Firstly, since this field is used for indicating SSB pattern for measurement, the first sentence for case of NR-U should elaborate from measurement perspective, instead of SSB transmission perspective. For example, ‘SMTC measurement duration’ should be used rather than ‘discovery burst transmission window’, and ‘may be transmitted’ could be changed to ‘are to be measured’, as shown in the following example given in a RAN1 LS [2] which was discussed in RAN2#109bis meeting.

In R2-2103281 [5]:

Proposal 1: For configuration of SSBs to be measured for NR-U, RAN2 is kindly asked to clarify how to interpret mediumBitmap in SSB-ToMeasure from measurement perspective.

    - The following example can be considered.

SSB-ToMeasure field descriptions

mediumBitmap Bitmap when maximum number of SS/PBCH blocks per half frame equals to 8 as defined in TS 38.213 [13], clause 4.1. For operation with shared spectrum channel access, if the k-th bit is set to 1, the UE assumes that one or more SS/PBCH blocks within the SMTC measurement duration with candidate SS/PBCH block indexes corresponding to SS/PBCH block index equal to k – 1 are to be measured; if the k-th bit is set to 0, the UE assumes that the corresponding SS/PBCH block(s) are not to be measured. If ssb-PositionQCL is configured, the k-th bit is set to 0, where k > ssb-PositionQCL and the number of actually transmitted SS/PBCH blocks is not larger than the number of 1's in the bitmap.

Question 4: Do companies agree with the proposal 1 in R2-2103281 as shown above?

| Company Name | Yes/No? | Comments |
| --- | --- | --- |
| Apple | Yes | After checking with our RAN1 and RAN4 colleagues, we think NR-U should have no impact on SMTC measurement. |
| MediaTek | See comments | Change from “discovery burst transmission” to “SMTC measurement duration” maybe okay. No strong view. It seems that the term “discovery burst transmission” is common usage in NR-U. So, no change is also fine. Change from “may be transmitted” to “to be measured” has different meaning. The original one is more correct indicate that SSB may NOT transmitted in NR-U. (e.g. LBT failure) |
| Nokia | No | Change for the usage of ssb-ToMeasure should be coming from RAN1 (or RAN4) |
| Qcom | Yes |  |
| Intel | See comments | These are not essential change and also do not see any issue with the current text |
| vivo | No strong view | But we think it is not essential |

## 5.2   Sub-topic 2: Replace ssb-PositionQCL with ssb-PositionQCL-Common in SSB-ToMeasure

In both contributions [4] and [5], the same proposal was brought up that SSB-ToMeasure should associates with ssb-PositionQCL-Common-r16,

In R2-2103879:

Proposal 2: Confirm in RAN2 that ssb-ToMeasure associates with ssb-PositionQCL-Common-r16.

In R2-2103281:

Proposal 2: For configuration of SSBs to be measured for NR-U, RAN2 is kindly asked to change ssb-PositionQCL to ssb-PositionQCL-Common in description of mediumBitmap in SSB-ToMeasure.

Question 5: Do companies agree with the following text proposal to replace ssb-PositionQCL with ssb-PositionQCL-Common in SSB-ToMeasure?

SSB-ToMeasure field descriptions

mediumBitmap <Unrelevant text omitted> If ssb-PositionQCL-Common is configured, the k-th bit is set to 0, where k > ssb-PositionQCL-Common and the number of actually transmitted SS/PBCH blocks is not larger than the number of 1's in the bitmap.

| Company Name | Yes/No? | Comments |
| --- | --- | --- |
| Apple (proponent) | Yes |  |
| MediaTek | Yes | Change from “ssb-PositionQCL” (per target cell configuration) to “ssb-PositionQCL-Common” (per target frequency configuration) seems fine as SSB-ToMeasure is per frequencies configuration. |
| Nokia | Yes |  |
| Qcom | Yes |  |
| Intel | Yes |  |
| vivo | Yes |  |

## 5.3   Sub-topic 3: ssb-ToMeasure with a longer bitmap than cell specific Nqcl value

R2-2103879 [4] discusses the case where the cell specific Nqcl value (ssb-PositionQCL-CellsToAddModList-r16) is smaller than ssb-ToMeasure (Case 1 in below table) and correspondingly has Proposal 3 copied below.

|  | Common Nqcl | Cell specific Nqcl | ssb-ToMeasure | UE behavior |
| --- | --- | --- | --- | --- |
| Case 1 | 8 | 4 (cell A) | 10001000 | Not clear to RRM on cells with cell specific Nqcl (common problematic case) |
| Case 2 | 4 | 8 (cell A) | 10001000 | Not clear to RRM on cells with common Nqcl (rare case, no need to concern) |

In R2-2103879 [4]:

Proposal 3: If ssb-ToMeasure indicates a longer bitmap (10001000) while a smaller Nqcl (I.e.=4) is configured for ssb-PositionQCL-CellsToAddModList-r16, only the first Nqcl bits in ssb-ToMeasure are applicable.

Question 6: Do companies agree with Proposal 3 in R2-2103879 [4]?

| Company Name | Yes/No? | Comments |
| --- | --- | --- |
| Apple (proponent) | Yes |  |
| Nokia | No | This should be discussed in RAN1. |
| Qcom | Yes |  |

## 5.4   Sub-topic 4: Different values on ssb-PositionQCL-Common-r16/ ssb-PositionQCL-CellsToAddModList-r16 from MN/SN

R2-2103879 [4] brings the following issue if MN and SN configures different values on ssb-PositionQCL-Common-r16/ ssb-PositionQCL-CellsToAddModList-r16 according to TS 38.133.

Main issue we can see is UE may perform measurement on wrong candidate SSB index positions. For example, say MN configures Nqcl to 4 and SN configures Nqcl to 8, while one target cell is actually using SSB QCL value 8. For RRM with MN configured MO, suppose the ssb-ToMeasure points to SSB index #3, since UE calculates the SSB index according to QCL 4 (wrong one) thus UE may refer to the candidate SSB index 7 in Fig. 1, which is not the right candidate SSB index position (which should be candidate SSB index 3). Fig. 1 - Time shifted SSB transmission subject to LBT

To solve the issue, two alternatives are proposed in R2-2103879 [4].

- Alt 1: MN and SN always configure the same value on Nqcl on the same carrier and/or cell.

- Alt 2: If Alt 1 cannot be guaranteed, UE simply follows the configuration of NW in performing RRM.

Question 7: Which alternative do companies prefer in order to solve the issue that UE may perform measurement on wrong candidate SSB index positions if MN and SN configure different values on ssb-PositionQCL-Common-r16/ ssb-PositionQCL-CellsToAddModList-r16 according to TS 38.133?

- Alt 1: MN and SN always configure the same value on Nqcl on the same carrier and/or cell.

- Alt 2: If Alt 1 cannot be guaranteed, UE simply follows the configuration of NW in performing RRM.

| Company Name | Alt 1/Alt 2? | Comments |
| --- | --- | --- |
| Apple | Alt 1 | Alt 1 is much simpler and can guarantee the correctness of measurement results. |
| MediaTek | Alt 1 |  |
| Nokia | None | No need to specify limitation. UE just follows what is configured via Uu. Most likely NW will follow alt.1 behaviour. |
| Qcom | Alt1 or Alt2 | Although the issue used to justify it may not happen in our opinion, since we believe the “candidate” SSB position should be unique irrespective of QCL-factor config, but it is good to have it clarified. |
| Intel | Alt 1 |  |
| vivo | Alt 1 |  |

## 5.5   Sub-topic 5: Inter-RAT NR-U RRM in LTE spec

R2-2103879 [4] brings up that In LTE spec, for inter-RAT NR-U RRM related configuration in SIB24 and MeasObjectNR, ssb-PositionQCL-Common-r16 is optional present.

In R2-2103879 [4]:

Proposal 5: Suggest to make ssb-PositionQCL-Common-r16 in SIB24/MeasObjectNR conditional mandatory for shared spectrum in LTE spec, to align with NR spec.

Question 8: Do companies agree with Proposal 5 in R2-2103879 [4]?

| Company Name | Yes/No? | Comments |
| --- | --- | --- |
| Apple (proponent) | Yes |  |
| MediaTek | Yes |  |
| Nokia | Maybe | No strong need as anyway measurements wont work if this is not configured. But ASN.1 should not be changed as UE behaviour does not change. |
| vivo | No strong view |  |

Question 9: If the answer to Question 8 is Yes, do companies agree with the text proposal in change 1 and 2 in Annex 2 in R2-2103879 [4]?

The TP is also copied in Annex-1 in this paper for reference.

| Company Name | Yes/No? | Comments |
| --- | --- | --- |
| Apple (proponent) | Yes |  |
| MediaTek | Yes |  |
| Nokia | No | See the comments on specific proposals |
| Qcom | Yes |  |

Question 10: Do company agree with the following proposal in R2-2103879 [4]?

Proposal 6: Suggest to modify the field description to SSB-ToMeasure as below in TS36.331.

Text proposal to SSB-ToMeasure in TS36.331 mediumBitmap Bitmap when maximum number of SS/PBCH blocks per half frame equals to 8 as defined in TS 38.213 [88], clause 4.1. For operation with shared spectrum channel access, only mediumBitmap is used. If the k-th bit is set to 1, the UE assumes that one or more SS/PBCH blocks within the discovery burst transmission window with candidate SS/PBCH block indexes corresponding to SS/PBCH block index equal to k – 1 may be transmitted; if the kt-th bit is set to 0, the UE assumes that the corresponding SS/PBCH block(s) are not transmitted. If ssb-PositionQCL is configured, the k-th bit is set to 0, where k > ssb-PositionQCL and the number of actually transmitted SS/PBCH blocks is not larger than the number of 1's in the bitmap.

| Company Name | Yes/No? | Comments |
| --- | --- | --- |
| Apple (proponent) | Yes |  |
| MediaTek | Yes |  |
| Nokia | Maybe | See comments on specific proposals |
| Qcom | Yes |  |

## 5.6   Sub-topic 6: rmtc-SubframeOffset

In R2-2103879 [4], it brings up that when rmtc-SubframeOffset is not configured, it’s not clear if this random value generation is done per RMTC period, or per every ReportInterval, or upon every RRCReconfiguration.

It’s Rapporteur’s understanding that this issue also involves RAN1. However, it’s also beneficial if RAN2 can have some discussions first and then request RAN1’s confirmation since RAN2 is responsible for TS 38.331.

Question 11: Which option do companies think should be applied for the random value generation when rmtc-SubframeOffset is not configured?

Option 1: Per RMTC period

Option 2: Per every ReportInterval

Option 3: Per every RRCReconfiguration message

| Company Name | Option | Comments |
| --- | --- | --- |
| Apple | Option 1 | As we explained in the paper, we think the original motivation of the randomized value is to achieve a higher possibility to find out the overloaded carrier. If one fixed value (say 0) per Option 2 and 3 is used, the RSSI measurement result gets biased and then UE may miss the detection on the overloaded carrier. |
| MediaTek |  | We think it should be discussed in RAN1 first and we prefer leave this to UE implementation. |
| Nokia | None | This is RAN1 topic and discussion should be initiated there |
| Intel |  | Wait for RAN1 |
| vivo |  | Agree with MediaTek |

Question 12: Should the potential agreement in Question 11 also apply to LAA?

| Company Name | Yes/No? | Comments |
| --- | --- | --- |
| Apple | Yes | It’s better to also have the same description in LAA. |
| Nokia | No | See Q11 |

Question 13: Should an LS sent to RAN1 to inform them about RAN2 agreement?

| Company Name | Yes/No? | Comments |
| --- | --- | --- |
| Apple | Yes | We understand some companies may feel this should be decided by RAN1 but we encourage people to align with their RAN1 colleagues internally to save time. Once RAN2 gets something agreed, it’s better to inform RAN1. |
| Nokia | No | See Q11 |

# 6	  Topic 4: Missing smtc3 for IAB

R2-2104173	Missing smtc3 for smtc restriction with ssbFrequency	Samsung R&D Institute UK	CR	Rel-16	38.331	16.4.1	2558	-	F	NR_IAB-Core

Summary of change: Add “and that an smtc3list included in any measurement object with the same ssbFrequency has the same value” in the general section of measurement configuration.

### 5.5.2 Measurement configuration #### 5.5.2.1 General The network applies the procedure as follows: - to ensure that, whenever the UE has a measConfig associated with a CG, it includes a measObject for the SpCell and for each NR SCell of the CG to be measured; - to configure at most one measurement identity across all CGs using a reporting configuration with the reportType set to reportCGI; - to configure at most one measurement identity per CG using a reporting configuration with the ul-DelayValueConfig; - to ensure that, in the measConfig associated with a CG: - for all SSB based measurements there is at most one measurement object with the same ssbFrequency; - an smtc1 included in any measurement object with the same ssbFrequency has the same value and that an smtc2 included in any measurement object with the same ssbFrequency has the same value and that an smtc3list included in any measurement object with the same ssbFrequency has the same value; - to ensure that all measurement objects configured in this specification and in TS 36.331 [10] with the same ssbFrequency have the same ssbSubcarrierSpacing; - to ensure that, if a measurement object associated with the MCG has the same ssbFrequency as a measurement object associated with the SCG: - for that ssbFrequency, the measurement window according to the smtc1 configured by the MCG includes the measurement window according to the smtc1 configured by the SCG, or vice-versa, with an accuracy of the maximum receive timing difference specified in TS 38.133 [14]. - if both measurement objects are used for RSSI measurements, bits in measurementSlots in both objects corresponding to the same slot are set to the same value. Also, the endSymbol is the same in both objects.

Question 14: Do companies agree with the change in R2-2104173 [6]?

| Company Name | Yes/No? | Comments |
| --- | --- | --- |
| Apple | Yes |  |
| Huawei, HiSilicon | No strong view | This can be considered as straightforward editorial change, which can be merged to the miscellaneous CR. Without any change, we can always trust CU implementation. |
| Nokia | No | Without any change we can just trust proper implementation |
| Qualcomm | See comment | Same as Huawei and Nokia. |
| Intel | Yes |  |
| vivo | No strong view |  |

# 7	  Reference

[1] R2-2102650 LS on CGI reading with autonomous gaps (R4-2103610; contact: ZTE)	RAN4	LS in

[2] R2-2103030 Correction on T321 for autonomous gap based E-UTRAN CGI reporting	ZTE Corporation, Sanechips	CR	Rel-16	38.331

[3] R2-2103169 Clarification on NPN related CGI report	Huawei, CMCC, China Telecom, China Unicom, HiSilicon	CR	Rel-16	38.331

[4] R2-2103879 Discussion on NR-U RRM measurement	Apple, xiaomi, LG Electronics	discussion	Rel-16	NR_unlic-Core

[5] R2-2103281 Discussion on configuration of SSBs to be measured for NR-U	Fujitsu	discussion	Rel-16	NR_unlic-Core

[6] R2-2104173 Missing smtc3 for smtc restriction with ssbFrequency	Samsung R&D Institute UK	CR	Rel-16	38.331

# 8	  Annex 1 - Text Proposal to TS 36.331 (Change 1 and 2) in R2-2103879 [4]

### ---------------------------------------------------------------------------<start of 1st change>---------------------------------------------------------------------------

### 6.3.1	System information blocks

<Text omitted>

#### –	SystemInformationBlockType24

The IE SystemInformationBlockType24 contains information relevant for inter-RAT cell re-selection (i.e. information about NR frequencies and NR neighbouring cells relevant for cell re-selection), which can also be used for NR idle/inactive measurements. The IE includes cell re-selection parameters common for a frequency.

SystemInformationBlockType24 information element

-- ASN1START

SystemInformationBlockType24-r15 ::=	SEQUENCE {

carrierFreqListNR-r15				CarrierFreqListNR-r15				OPTIONAL,		-- Need OR

t-ReselectionNR-r15					T-Reselection,

t-ReselectionNR-SF-r15				SpeedStateScaleFactors				OPTIONAL,	-- Need OR

lateNonCriticalExtension			OCTET STRING					OPTIONAL,

...,

[[	carrierFreqListNR-v1610			CarrierFreqListNR-v1610		OPTIONAL    -- Need OR

]]

}

CarrierFreqListNR-r15 ::=		SEQUENCE (SIZE (1..maxFreq)) OF CarrierFreqNR-r15

CarrierFreqListNR-v1610 ::=		SEQUENCE (SIZE (1..maxFreq)) OF CarrierFreqNR-v1610

CarrierFreqNR-r15 ::=				SEQUENCE {

carrierFreq-r15						ARFCN-ValueNR-r15,

multiBandInfoList-r15				MultiFrequencyBandListNR-r15		OPTIONAL,	-- Need OR

multiBandInfoListSUL-r15			MultiFrequencyBandListNR-r15		OPTIONAL,	-- Need OR

measTimingConfig-r15				MTC-SSB-NR-r15						OPTIONAL,	-- Need OR

subcarrierSpacingSSB-r15			ENUMERATED {kHz15, kHz30, kHz120, kHz240},

ss-RSSI-Measurement-r15				SS-RSSI-Measurement-r15		OPTIONAL,		-- Cond RSRQ2

cellReselectionPriority-r15			CellReselectionPriority		OPTIONAL,		-- Need OP

cellReselectionSubPriority-r15		CellReselectionSubPriority-r13	OPTIONAL,	-- Need OR

threshX-High-r15					ReselectionThreshold,

threshX-Low-r15						ReselectionThreshold,

threshX-Q-r15						SEQUENCE {

threshX-HighQ-r15				ReselectionThresholdQ-r9,

threshX-LowQ-r15				ReselectionThresholdQ-r9

}																OPTIONAL,	-- Cond RSRQ

q-RxLevMin-r15						INTEGER (-70..-22),

q-RxLevMinSUL-r15					INTEGER (-70..-22)				OPTIONAL,		-- Need OR

p-MaxNR-r15							P-MaxNR-r15,

ns-PmaxListNR-r15					NS-PmaxListNR-r15					OPTIONAL,	-- Need OR

q-QualMin-r15						INTEGER (-43..-12)				OPTIONAL,		-- Need OP

deriveSSB-IndexFromCell-r15			BOOLEAN,

maxRS-IndexCellQual-r15				MaxRS-IndexCellQualNR-r15		OPTIONAL,		-- Need OR

threshRS-Index-r15					ThresholdListNR-r15				OPTIONAL,		-- Need OR

...,

[[	multiBandNsPmaxListNR-v1550		MultiBandNsPmaxListNR-1-v1550	OPTIONAL,	-- Need OR

multiBandNsPmaxListNR-SUL-v1550	MultiBandNsPmaxListNR-v1550		OPTIONAL,	-- Need OR

ssb-ToMeasure-r15				SSB-ToMeasure-r15				OPTIONAL		-- Need OR

]]

}

CarrierFreqNR-v1610 ::=		SEQUENCE {

smtc2-LP-r16						MTC-SSB2-LP-NR-r16				OPTIONAL,	  -- Need OR

ssb-PositionQCL-CommonNR-r16	SSB-PositionQCL-RelationNR-r16	OPTIONAL,	-- Cond SharedSpectrum2

whiteCellListNR-r16				WhiteCellListNR-r16					OPTIONAL,	-- Cond SharedSpectrum

highSpeedCarrierNR-r16			ENUMERATED {true}				OPTIONAL	  -- Need OR

}

MultiBandNsPmaxListNR-1-v1550	::=	SEQUENCE (SIZE (1.. maxMultiBandsNR-1-r15)) OF NS-PmaxListNR-r15

MultiBandNsPmaxListNR-v1550	::=	SEQUENCE (SIZE (1.. maxMultiBandsNR-r15)) OF NS-PmaxListNR-r15

WhiteCellListNR-r16 ::=			SEQUENCE (SIZE (1..maxCellWhiteNR-r16)) OF PhysCellIdNR-r15

-- ASN1STOP

SystemInformationBlockType24 field descriptions

carrierFreqListNR List of carrier frequencies of NR carriers. These frequencies correspond to GSCN values as specified in TS 38.101 [85]. If the carrierFreqListNR-v1610 is present, it contains the same number of entries, listed in the same order as in the carrierFreqListNR (without suffix).

cellReselectionPriority The field concerns the absolute priority of the concerned carrier frequency as used by the cell reselection procedure. Corresponds with parameter "priority" in TS 36.304 [4].

deriveSSB-IndexFromCell The field indicates whether the UE may use, to derive the SSB index of a cell on the indicated SSB frequency and subcarrier spacing, the timing of any detected cell with the same SSB frequency and subcarrier spacing. If this field is set to TRUE, the UE assumes SFN and frame boundary alignment across cells on the same NR carrier frequency as specified in TS 36.133 [16].

highSpeedCarrierNR If the field is present, the UE shall apply the enhanced inter-RAT NR measurement requirements to support high speed up to 500 km/h as specified in TS 36.133 [16] to the NR carrier.

maxRS-IndexCellQual Number of SS blocks to average for cell measurement derivation. Corresponds to the parameter nrofSS-BlocksToAverage in TS 38.304 [92].

measTimingConfig Used to configure measurement timing configurations, i.e., timing occasions at which the UE measures SSBs. If the field is absent, the UE assumes that SSB periodicity is 5ms in this frequency.

multiBandInfoList Indicates the list of frequency bands for which the NR cell reselection parameters apply. The UE shall select the first listed band which it supports in the multiBandInfoList field to represent the NR neighbour carrier frequency. The network always includes this field.

multiBandInfoListSUL Indicates the list of frequency bands for which the NR cell reselection parameters apply. The UE shall select the first listed band which it supports in the multiBandInfoListSUL field to represent the NR neighbour carrier frequency.

multiBandNsPmaxListNR Indicates the NS-PmaxListNR configuration for the NR frequency band(s) listed in multiBandInfoList. The first entry corresponds to the second listed band in multiBandInfoList, and second entry corresponds to the third listed band in multiBandInfoList, and so on.

multiBandNsPmaxListNR-SUL Indicates the NS-PmaxListNR configuration for the NR SUL frequency band(s) listed in multiBandInfoListSUL. The first entry corresponds to the first listed band in multiBandInfoListSUL, and second entry corresponds to the second listed band in multiBandInfoListSUL, and so on.

Ns-PmaxListNR Indicates a list of additionalPmax and additionalSpectrumEmission, corresponds to the first listed band in the multiBandInfoList.

p-MaxNR Indicates the maximum power for NR (see TS 38.104 [91]) the UE can use in NR SCG.

q-QualMin Parameter "Qqualmin" in TS 36.304 [4], applicable for NR neighbour cells. If the field is not present, the UE applies the (default) value of negative infinity for Qqualmin. The actual value Qqualmin = field value [dB].

q-RxLevMin Parameter "Qrxlevmin" in TS 38.304 [92], applicable for NR neighbour cells. The actual value Qrxlevmin = field value * 2 [dBm].

q-RxLevMinSUL Parameter "Qrxlevmin" in TS 38.304 [92], applicable for NR neighbouring cells. The actual value Qrxlevmin = field value * 2 [dBm].

Smtc2-LP Measurement timing configuration for inter-RAT neighbour cells in NR with a Long Periodicity (LP) indicated by periodicity in smtc2-LP. The timing offset and duration are equal to the offset and duration indicated in measTimingConfig in CarrierFreqNR. The periodicity in smtc2-LP can only be set to a value strictly larger than the periodicity in measTimingConfig in CarrierFreqNR (e.g. if measTimingConfig indicates sf20 the Long Periodicity can only be set to sf40, sf80 or sf160, if measTimingConfig indicates sf160, smtc2-LP cannot be configured). The pci-List, if present, includes the physical cell identities of the inter-RAT neighbour cells with Long Periodicity. If smtc2-LP is absent, the UE assumes that there are no inter-RAT neighbour cells with a Long Periodicity.

Ssb-PositionQCL-CommonNR Indicates the QCL relationship between SS/PBCH blocks for NR neighbor cells on the indicated frequency as specified in TS 38.213 [88], clause 4.1.

ssb-ToMeasure The set of SS blocks to be measured within the SMTC measurement duration (see TS 38.215 [89]). When the field is absent the UE measures on all SS-blocks.

ss-RSSI-Measurements Indicates the SSB-based RSSI measurement configuration. If the field is absent, the UE behaviour is defined in TS 38.215 [89], clause 5.1.3.

threshRS-Index List of thresholds for consolidation of L1 measurements per RS index. Corresponds to the parameter absThreshSS-BlocksConsolidation in TS 38.304 [92].

threshX-High Parameter "ThreshX, HighP" in TS 36.304 [4].

threshX-HighQ Parameter "ThreshX, HighQ" in TS 36.304 [4].

threshX-Low Parameter "ThreshX, LowP" in TS 36.304 [4].

threshX-LowQ Parameter "ThreshX, LowQ" in TS 36.304 [4].

t-ReselectionNR Parameter "TreselectionNR" in TS 36.304 [4].

t-ReselectionNR-SF Parameter "Speed dependent ScalingFactor for TreselectionNR" in TS 36.304 [4]. If the field is not present, the UE behaviour is specified in TS 36.304 [4].

whiteCellListNR List of whitelisted neighbouring NR cells.

| Conditional presence | Explanation |
| --- | --- |
| RSRQ | The field is mandatory present if the threshServingLowQ is present in systemInformationBlockType3; otherwise it is not present. |
| RSRQ2 | The field is optional Need OP if the threshServingLowQ is present in systemInformationBlockType3; otherwise it is not present. |
| SharedSpectrum | The field is optional Need OP if NR operates with shared spectrum channel access; otherwise, it is not present. |
| SharedSpectrum2 | The field is mandatory present if NR operates with shared spectrum channel access; otherwise, it is not present. |

### ---------------------------------------------------------------------------<start of 2nd change>---------------------------------------------------------------------------

### 6.3.5	Measurement information elements

<Text omitted>

#### –	MeasObjectNR

The IE MeasObjectNR specifies information applicable for inter-RAT NR neighbouring cells.

MeasObjectNR information element

-- ASN1START

MeasObjectNR-r15 ::=				SEQUENCE {

carrierFreq-r15						ARFCN-ValueNR-r15,

rs-ConfigSSB-r15					RS-ConfigSSB-NR-r15,

threshRS-Index-r15					ThresholdListNR-r15				OPTIONAL,		-- Need OR

maxRS-IndexCellQual-r15				MaxRS-IndexCellQualNR-r15		OPTIONAL,		-- Need OR

offsetFreq-r15						Q-OffsetRangeInterRAT			DEFAULT 0,

blackCellsToRemoveList-r15			CellIndexList					OPTIONAL,		-- Need ON

blackCellsToAddModList-r15			CellsToAddModListNR-r15			OPTIONAL,		-- Need ON

quantityConfigSet-r15				INTEGER (1.. maxQuantSetsNR-r15),

cellsForWhichToReportSFTD-r15		SEQUENCE (SIZE (1..maxCellSFTD)) OF PhysCellIdNR-r15	OPTIONAL,	-- Need OR

...,

[[	cellForWhichToReportCGI-r15			PhysCellIdNR-r15				OPTIONAL,	-- Need ON

deriveSSB-IndexFromCell-r15			BOOLEAN							OPTIONAL,	-- Need ON

ss-RSSI-Measurement-r15				SS-RSSI-Measurement-r15			OPTIONAL,	-- Need ON

bandNR-r15					CHOICE {

release						NULL,

setup						FreqBandIndicatorNR-r15

}																OPTIONAL	-- Need ON

]],

[[

rmtc-ConfigNR-r16						SetupRelease {RMTC-ConfigNR-r16}			OPTIONAL		-- Cond SharedSpectrum

]]

}

RS-ConfigSSB-NR-r15 ::=			SEQUENCE {

measTimingConfig-r15			MTC-SSB-NR-r15,

subcarrierSpacingSSB-r15	ENUMERATED {kHz15, kHz30, kHz120, kHz240},

...,

[[	ssb-ToMeasure-r15			CHOICE {

release						NULL,

setup						SSB-ToMeasure-r15

}											OPTIONAL	-- Need ON

]],

[[

ssb-PositionQCL-CommonNR-r16	SSB-PositionQCL-RelationNR-r16	OPTIONAL,	-- Cond SharedSpectrum2

ssb-PositionQCL-CellsToAddModListNR-r16	SSB-PositionQCL-CellsToAddModListNR-r16	OPTIONAL,	-- Cond SharedSpectrum

ssb-PositionQCL-CellsToRemoveListNR-r16	SEQUENCE (SIZE (1..maxCellMeas)) OF PhysCellIdNR-r15	OPTIONAL	-- Cond SharedSpectrum

]]

}

CellsToAddModListNR-r15 ::=			SEQUENCE (SIZE (1..maxCellMeas)) OF CellsToAddModNR-r15

CellsToAddModNR-r15 ::=			SEQUENCE {

cellIndex-r15					INTEGER (1..maxCellMeas),

physCellId-r15					PhysCellIdNR-r15

}

SSB-PositionQCL-CellsToAddModListNR-r16 ::=	SEQUENCE (SIZE (1..maxCellMeas)) OF SSB-PositionQCL-CellsToAddNR-r16

SSB-PositionQCL-CellsToAddNR-r16 ::=	SEQUENCE {

physCellId-r16							PhysCellIdNR-r15,

ssb-PositionQCL-r16						SSB-PositionQCL-RelationNR-r16

}

RMTC-ConfigNR-r16 ::=	SEQUENCE {

rmtc-PeriodicityNR-r16				ENUMERATED {ms40, ms80, ms160, ms320, ms640},

rmtc-SubframeOffsetNR-r16			INTEGER(0..639)			OPTIONAL,	-- Need ON

measDurationNR-r16					ENUMERATED {sym1, sym14or12, sym28or24, sym42or36, sym70or60},

rmtc-FrequencyNR-r16				ARFCN-ValueNR-r15,

refSCS-CP-NR-r16					ENUMERATED {kHz15, kHz30, kHz60-NCP, kHz60-ECP},

...

}

-- ASN1STOP

MeasObjectNR field descriptions

bandNR Indicates the frequency band of the NR carrier frequency configured in this MeasObjectNR. This field is always set to setup when the network configures measurements with this MeasObjectNR.

carrierFreq Identifies the SSB frequency to be measured. E-UTRAN does not configure more than one measurement object for the same SSB frequency.

deriveSSB-IndexFromCell The field indicates whether the UE may use, to derive the SSB index of a cell on the indicated SSB frequency and subcarrier spacing, the timing of the NR serving cell with the same SSB frequency and subcarrier spacing if configured. Otherwise, the field indicates whether the UE may use the timing of any detected cell with the same SSB frequency and subcarrier spacing.

measDurationNR Number of consecutive symbols for which the Physical Layer reports samples of RSSI (see TS 38.215 [89]). Value sym1 corresponds to one symbol, sym14or12 corresponds to 14 symbols of the reference numerology for NCP and 12 symbols for ECP, and so on.

quantityConfigSet Indicates the n-th element of quantityConfigNRList provided in MeasConfig.

refSCS-CP-NR Indicates a reference subcarrier spacing and cyclic prefix to be used for RSSI measurements (see TS 38.215 [89]).

rmtc-FrequencyNR Indicates the center frequency of the measured bandwidth (see TS 38.215 [89]).

rmtc-PeriodicityNR Indicates the RSSI measurement timing configuration (RMTC) periodicity (see TS 38.215 [89]). Value ms40 corresponds to 40 ms periodicity, ms80 corresponds to 80 ms periodicity, and so on.

rmtc-SubframeOffsetNR Indicates the RSSI measurement timing configuration (RMTC) subframe offset (see TS 38.215 [89)). If not configured, the UE chooses a random value as rmtc-SubframeOffsetNR for measDurationNR which shall be selected to be between 0 and the configured rmtc-PeriodicityNR with equal probability.

rs-ConfigSSB Indicates the SSB configuration for measuring the set of SS blocks within the SMTC measurement duration.

ssb-PositionQCL-NR Indicates the QCL relationship between SS/PBCH blocks for a specific neighbor cell as specified in TS 38.213 [88], clause 4.1. If provided, the cell specific value overwrites the common value signalled by ssb-PositionQCL-CommonNR in MeasObjectNR for the indicated cell.

ssb-PositionQCL-CommonNR Indicates the QCL relationship between SS/PBCH blocks for NR neighbor cells as specified in TS 38.213 [88], clause 4.1.

threshRS-Index List of thresholds for consolidation of L1 measurements per RS index.

| Conditional presence | Explanation |
| --- | --- |
| SharedSpectrum | The field is optional Need ON if NR operates with shared spectrum channel access; otherwise, it is not present. |
| SharedSpectrum2 | The field is mandatory present if NR operates with shared spectrum channel access; otherwise, it is not present. |