3GPP TSG-RAN WG2 #113bis-e	Tdoc R2-210433

Electronic meeting, 2021-04-12 – 2021-04-20

Agenda Item:	6.4.2

Source:	Ericsson

Title:	[AT113bis-e][213][MOB] RRCReconfiguration with DAPS source release (Ericsson)

Document for:	Discussion, Decision

1	Introduction

This document summarizes the following email discussion:

[AT113bis-e][213][MOB] RRCReconfiguration with DAPS source release (Ericsson)

Scope:

    - Discuss how/whether to capture the agreements on what is allowed to be configured when daps-SourceRelease is sent to UE according to online agreements.

Intended outcome:

    - Discussion summary in R2-2104330 (by email rapporteur).

    - Agreeable CRs (if any)

Deadline for providing comments, for rapporteur inputs, conclusions and CR finalization:

    - Initial deadline (for company feedback):  1st week Thu, UTC 0900

    - Initial deadline (for rapporteur summary):  1st week Fri, UTC 0900

    - Deadline for CR finalization: 2nd week Tue, UTC 1000

The email discussion is related to R2-2102820 discussed during the online session on Monday 2021-04-12.

2	Discussion

The following was agreed for RRCReconfiguration with DAPS source release in the online session on Monday:

R2-2102820	Reconfiguration during DAPS HO	Ericsson	discussion	Rel-16	R2-2100488

Observation 1	The restriction to have UDC or EHC configured during a DAPS handover is missing in the Stage-2 specifications.

Observation 2	It is not clear from the LTE specifications when the target node can configure the UE with SCG, SCells, uplinkDataCompression, ethernetHeaderCompression and/or conditional handover at a DAPS handover.

Observation 3	It is not clear from the NR specifications when the target node can configure the UE with SCG, SCells, multi-TRP configuration, SUL, sidelink, ethernetHeaderCompression and/or conditional handover at a DAPS handover.

Observation 4	Since the daps-SourceRelease indication is handled in the beginning of the procedure in 5.3.5.3 (in both 36.331 and 38.331), it is possible to include configuration of features not supported together with DAPS HO in the same RRC Reconfiguration message.

Observation 5	The explicit source cell indication (daps-SourceRelease) in the RRC Reconfiguration message is included to allow the network to reconfigure the UE before completion of the DAPS HO.

Observation 6	If it would be required for the network to include the explicit daps-SourceRelease in the first RRC Reconfiguration message after successful DAPS HO, it would instead be an implicit release. The daps-SourceRelease indication would then only trigger a failure when not set correctly, which is not the intention.

Observation 7	Conditional reconfigurations are included within an RRC Reconfiguration message that is built by the serving node. They can thus be included in the same message that contains the daps-SourceRelease set by the serving node.

Proposal 1	Clarify in the specifications that the first possible addition of SCG or SCells and configuration of multi-TRP, UDC, EHC, SUL, sidelink or conditionalReconfiguration (CHO) in the target cell at a DAPS HO is in the RRC Reconfiguration message that includes daps-SourceRelease.

Proposal 2	Correct field descriptions for parameters that can be configured in the RRC Reconfiguration message with daps-SourceRelease but where it now says that they cannot be configured if there is a DAPS bearer configured.

Proposal 3	The Text Proposals in section 3 should be introduced in the specifications.

Discussion

-	MTK supports P1. Huawei also agrees but thinks this is already clear in our specifications as it's a normal reconfiguration. Nokia agrees with Huawei.

-	Intel agrees with P1 but thinks it would be good to clarify this in specifications to avoid ambiguities. QC agrees since order of UE implementations can differ.

- Agreement

- 1	RAN2 confirms that the first possible addition of SCG or SCells and configuration of multi-TRP, UDC, EHC, SUL, sidelink or conditionalReconfiguration (CHO) in the target cell at a DAPS HO is in the RRC Reconfiguration message that includes daps-SourceRelease.

Discuss in offline [213] how and whether to capture these in the specifications (Ericsson)

Thus, the general understanding in RAN2 is that the non-DAPS compatible features can be configured in the same RRC reconfiguration message that contains the daps-SourceRelease indication. The non-DAPS compatible features considered here are the following:

- Carrier Aggregation (CA)

- Dual Connectivity (DC)

- Multi-TRP (NR only)

- UDC (LTE only)

- EHC

- SUL (NR only)

- Sidelink

- conditionalReconfiguration (CHO)

To capture the above agreement the following options can be considered:

- Capture with a note in the chairman’s notes

- Capture in stage-2 only (i.e. 38.300 and 36.300)

- Capture in stage-3 only (i.e. 38.331 and 36.331)

- Capture in stage-2 + stage-3

Q1: Which of the options do you prefer?

| Company | Which option? | Comments |
| --- | --- | --- |
| Ericsson | d (b might also be ok) | It is not clear from the specifications that the non-DAPS compatible features can be configured in the same RRC reconfiguration message that contains the daps-SourceRelease indication. For some related parameters in 36.331 and 38.331 it is, in the field descriptions, stated that they cannot be included/configured when there is a DAPS bearer configured. This could then be interpreted as that the parameter cannot be set in the message that includes the daps-SourceRelease, which removes any DAPS bearer. Therefore this needs to be clarified somewhere. Option a is not a good option since if DAPS gets implemented in a few years time the implementers should not have to look a t the chairman‘s notes. |
| MediaTek | d | Stage-2 modifications are needed since we have related descriptions about non-DAPS compatible features “during DAPS handover”, and the proposed changes explain that the earliest message to configure non-DAPS compatible features is the one with daps-SourceRelease. Stage-3 modifications do help confirm that it is not a problem to have non-DAPS compatible features configured in the message with daps-SourceRelease, although reasonable UE implementation aligns with the proposed change even without the CR. |

If it is decided to capture the agreement in stage-2 (i.e. option b or d in Q1) the text proposals in R2-2102820 may be used as a starting point.

(Note that restriction to configure EHC during DAPS handover is currently missing in 38.300 and is therefore also added in the text proposal for 38.300).

Q2: Any comments on the stage-2 text proposals in R2-2102820?

| Company | Comments |
| --- | --- |

If it is decided to capture the agreement in stage-3 (i.e. option c or d in Q1) the text proposals in R2-2102820 may be used as a starting point.

Q3: Any comments on the stage-2 text proposals in R2-2102820?

| Company | Comments |
| --- | --- |

Conclusion

TBA

References

Tdoc Number, Title, Source, Meeting, Date

Spec number, Title, Source, Version, Date